SA WG2 Meeting SA2#161	S2-2402413

# 26 February – 1 March, 2024, Athens, Greece 	 revision of S2-2400111r07

Source: 	OPPO

Title: 	KI #1, New Sol: Energy Brokerage Function for energy related information processing

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes a new solution for KI#1

# Introduction

A new solution is proposed in this contribution for inclusion in TR 23.700-66. This solution outlines the specifics of using Energy Brokerage Function for energy related information acquisition and processing.

# 2	Proposal

It is proposed to capture the following solution in TR 23.700-66.

Start of Changes

## 6.X	Solution #X: Energy Brokerage Function for energy related information acquisition and processing

### 6.X.1	Key Issue mapping

This is a candidate solution for KI#1.

In KI#1, to support network energy related information exposure, the following aspects are to be studied:

-	How the network energy related information is exposed.

-	How and what network energy related information from the Network entities (i.e. RAN nodes, 5GC NFs) can be obtained in order to support network energy related information exposure.

### 6.X.2	Functional Description

Based on the requirements and use cases in TS 22.261 [X], it is proposed to specify an Energy Brokerage Function (EBF) in 5GC to act as an intermediary between the network and authorized third parties to acquire energy related information, process the information to generate energy consumption and energy efficiency KPI, and expose energy consumption and efficiency-related data to authorized third parties.

The functionalities of EBF shall include:

- Collects relevant counters from OAM and uses them to generate energy efficiency related KPIs.

- Calculate energy efficiency, carbon emissions, ratio of renewable energy, etc. based on requirements.

- Enables periodic reporting to authorized third parties based on their agreement (e.g., monthly, or yearly).

- Notifies when the energy consumption reaches the energy credit limit.

- With user consent, exposes energy efficiency information based on a subscriber's data volume and other metrics.

- Exposes energy related information to NFs and AFs, e.g., as data source to expose energy related information to e.g., NWDAF for them to generate energy related analytics.

Editor’s note: whether there is a need for a new 5GC NF or whether proposed functionality can be provided by existing NFs will be evaluated and concluded on during the evaluation and conclusion phase.

### 6.X.3	Procedures

### 6.X.3.1	Interactions between EBF and OAM

The EBF could collect relevant counters from OAM and use them to generate energy efficiency related KPIs. The interactions between EBF and OAM for data collection are illustrated in Figure 6.X.3.1-1. The type of performance measurements data to be collected depends on the use cases. This figure is an example of the OAM performance data file report management service that is defined in TS 28.532 [11]. The actual OAM services and reporting mechanisms that EBF may use are specified in TS 28.532, TS 28.550 [10] or TS 28.545 [12].

The flow below assumes that EBF is configured on how to subscribe to the relevant OAM services. OAM shall setup the required mechanisms to guarantee the continuous data collection requested by EBF.

Figure 6.X.3.1-1: Data collection from OAM

- Subscribe (Input): EBF subscribes to the notification(s) related to the services provided by the management service producer.

- Subscribe (Output): management service producer responses to EBF if the subscription is success or not.

- Data processing: management service producer prepares the data.

- Notification: management service producer notifies the data is ready.

In terms of network slice, based on the Network Slice information, the EBF could identify the Network Slice managed object relating to the S-NSSAI and NSI ID and consumes the management services to collect the management data of the corresponding Network Slice managed object (including the NRF serving the network slice, the NFs associated to the network slice, the NG RAN or 5GC performance measurements defined in TS 28.552, or the 5G end to end KPIs defined in TS 28.554) provided by OAM.

### 6.X.3.2	EBF Service Operation

Figure 6.X.3.2-1 illustrates two procedures. In one procedure, EBF service consumer (e.g. including NFs) could subscribe/unsubscribe at EBF to be notified on EE/EC information. In the other procedure, the EE information exposure to AFs may be performed via NEF by using EE information subscription to EBF.

Figure 6.X.3.2-1: EE information subscribe/unsubscribe at EBF

- EBF gets performance counters and energy consumption from OAM as described in clause 6.X.3.1, and calculates EE KPIs as described in S2-2402822.

- In steps 1a-1b, the EBF service consumer subscribes to or cancels subscription to EE information by invoking the Nebf_EEInfoSubscription_Subcribe/Nebf_EEInfoSubscription_Unsubscribe service operation.

If EBF service consumer is subscribed to EE information, the EBF notifies the EBF service consumer with the EE information by invoking Nebf_EEInfoSubscription_Notify service operation.

- In Steps 2a-2e, the AF subscribes to or cancels subscription to EE information via NEF by invoking the Nnef_EEInfoExposure_Subscribe/Nnef_EEInfoExposure_Unsubscribe service operation.

If the EE info subscription is authorized by the NEF, based on the request from the AF, the NEF subscribes to or cancels subscription to EE information by invoking the Nebf_EEInfoSubscription_Subscribe/Nebf_EEInfoSubscription_Unsubscribe service operation.

If the NEF has subscribed to EE information, the EBF notifies the NEF with the EE information by invoking Nebf_EEInfoSubscription_Notify service operation.

If the NEF receives the notification from EBF, the NEF notifies the AF with the EE information by invoking Nnef_EEInfoExposure_Notify service operation.

Figure 6.X.3.2-2 illustrates EBF Service Consumer may request and get from EBF EE information, using Nebf_EEInfo service operation. The EE information could be requested by EBF service consumer (eg. including NFs).

Figure 6.X.3.2-2: EE Information request by EBF service consumer

- The EBF service consumer requests EE information by invoking Nebf_EEInfo_Request service operation for specific EE KPIs defined in S2-2402822.

- The EBF responds with EE information to the EBF service consumer.

Figure 6.X.3.2-3 illustrates the EE information exposure to AF may be performed via NEF by using EEInfo request to EBF.

Figure 6.X.3.2-3: EE Information exposure via NEF

- The AF requests to receive EE info via NEF by invoking the Nnef_EEInfoExposure_Fetch service operation.

- Based on the request from the AF, the NEF requests EE info by invoking the Nebf_EEInfo_Request service operation.

- The EBF responds with EE information to the NEF.

- The NEF responds with EE information to the AF.

### 6.x.4	Impacts on existing services, entities and interfaces

EBF:

Support functions as described in clause 6.X.2

Support interactions with OAM as described in clause 6.X.3.1

Support service operations as described in clause 6.X.3.2

NEF:

Support EEInfo subscription and fetch service operations as described in clause 6.X.3.2

Editor’s Note: The details of the information provided to NFs and AFs are FFS.

Second Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 22.882: "Study on Energy Efficiency as service criteria".

[6]	3GPP TS 28.554 "5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 28.310 "Management and orchestration; Energy efficiency of 5G".

[8]	3GPP TS 22.261 "Service requirements for the 5G system".

[9]	3GPP TS 28.552 "Management and orchestration; 5G performance measurements".

[10]	3GPP TS 28.550 "Management and orchestration; Performance assurance".

[11]	3GPP TS 28.532 "Management and orchestration; Generic management services".

[12]	3GPP TS 28.545 "Management and orchestration; Fault Supervision (FS)".

End of Changes