3GPP TSG-SA/WG2 Meeting #161	S2-2402751

# Athens, GREECE, February 26 – March 01, 2024

Source:	CATT

Title:	KI#2, New solution – Network-based EAS and local UPF (re)selection

Document for:	Agreement

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for KI#2: Enhancement of EAS and local UPF (re)selection.

- Discussion

As described in the clause 5.2 of TR 23.700-49, the following aspects will be studied for KI#2:

- How to enable 5GC and/or a third party trusted and/or non-trusted Application Function to select the most suitable local UPF and EAS considering end to end delay that includes delay between a candidate N6 interface of the 5GS and a candidate EAS, and/or potentially EAS load.

- Whether and how to define N6 delay information (e.g., RTT).

- Whether considering the EAS load is needed, and if it is needed how the information representing the EAS load would be defined.

- For local UPF and EAS (re)selection, whether and how to take into account potentially EAS load and N6 Delay between the local PSA and EAS, including how to obtain and update the above information.

This paper is to discuss and resolve the above issues.

2	Proposal

It is proposed to agree the following texts in clause 6 of TR 23.700-49.

******* start of 1st change ********

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | Key Issue #2 |  |  |
| #x |  | x |  |  |

6.X	Solution #X: Network-based EAS and local UPF (re)selection

### 6.X.1	Key Issue mapping

The following solution corresponds to the key issue key issue #2 on enhancement of EAS and local UPF (re)selection.

### 6.X.2	Description

The AF may request EAS discovery based on N6 delay and/or EAS load. The SMF determines the N6 delay between each candidate PSA UPF and EAS via indicating PSA UPF to report the N6 delay measurements or receives delay analysis information from NWDAF, or determines the EAS load based on the information received from NWDAF. The SMF selects a target EAS and a local PSA UPF based on the determined N6 delay and/or EAS load, and indicates the selected EAS IP address to EASDF via DNS message handling rule.

6.x.3	Procedures

Figure 6.x.3-1: Network-based EAS and local UPF (re)selection

1.	[Optional] The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [x] is used to request EAS discovery based on N6 delay and/or EAS load. The AF request may include Indication for N6 delay based EAS discovery, Indication for EAS load based EAS discovery. In step 5 of figure 4.3.6.2-1 of TS 23.502 [x], the PCF creates PCC rule with Indication for N6 delay based EAS discovery, Indication for EAS load based EAS discovery.

2.	The UE triggers PDU session establishment procedure and sends DNS query to EASDF, and EASDF retrieves the DNS response from DNS server and notifies the DNS message reporting to SMF as shown in step 1-15 of figure 6.2.3.2.2-1.

3.	Based on the local policy or PCC rule received from PCF in step 2, the SMF determines the N6 delay or EAS load should be taken into consideration for EAS and local UPF selection.

Based on EAS information received from the EASDF in Neasdf_DNSContext_Notify of step 2 and other UPF selection criteria, the SMF determines one or more candidate PSA UPF(s). The SMF also receives one or more candidate EAS(s) as indicated in EAS information.

The SMF determines the N6 delay between each candidate PSA UPF and EAS via following step 3a and/or 3b, and SMF may receive EAS load via step 3b.

3a.	[Optional] The SMF sends Session Reporting Rule to each candidate PSA UPF, which indicates the PSA UPF to report the N6 delay (between PSA UPF and EAS) measurements.

The UPF measures the UL and/or DL N6 delay (e.g. via RTT measurement) and sends N4 Session Report to SMF to report the N6 delay measurements. The SMF responds with an N4 session report ACK message.

3b.	[Optional] The SMF sends Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe to request NWDAF to provide delay (e.g. UL/DL Average Packet Delay) and/or EAS load.

The SMF receives the delay and/or EAS load from NWDAF via Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify. The SMF determines the UL/DL N6 delay by subtracting the UL/DL PDB (delay between UE and PSA UPF) from UL/DL Average Packet Delay (delay between UE and EAS).

4.	The SMF selects an EAS and a local PSA UPF based on the N6 delay and/or EAS load determined in step 3a and/or 3b.

5. The SMF inserts the Local PSA UPF.

6a. The SMF sends the updated DNS message handling rule to EASDF via Neasdf_DNSContext_Update Request. The DNS message handling rule includes the selected EAS IP address.

6b.	The EASDF responds with Neasdf_DNSContext_Update Response.

7.	The EASDF sends the DNS Response with selected EAS IP address to the UE as indicated in the DNS message handling rule.

### 6.X.4	Impacts on services, entities and interfaces

SMF:

1.	Determines the N6 delay or EAS load should be taken into consideration for EAS and local UPF selection based on local policy or PCC rule.

2.	Determines one or more candidate PSA UPF(s) and EAS(s), and configures PSA UPF to measure the N6 delay between each candidate PSA UPF and EAS, and/or retrieves delay information or EAS load from NWDAF.

3.	Selects a target EAS and a local PSA UPF based on the determined N6 delay and/or EAS load, and indicates the selected EAS IP address to EASDF via DNS message handling rule.

UPF:

1.	Reports the N6 delay (between PSA UPF and EAS) measurements.

NWDAF:

1.	Provisions the EAS load information.

AF:

1.	Requests the Indication for N6 delay based EAS discovery and Indication for EAS load based EAS discovery.

******* End of 1nd change ********