SA WG2 Meeting #S2-162	S2-2404148

15 - 19 April, 2024, Changsha, China	(Revision of S2-2402161)

Source:	China Telecom

Title:	KI#3, Sol#13: Update to remove EN

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract: This contribution proposes to update Sol#13 to remove Editor’s notes.

1.	Discussion

In SA2#160-Ad Hoc-e, the following Editor's notes was captured in the description of Sol#13 (see TR 23.700-70 clause 6.13) and this document addresses it.

Editor's note:	It is FFS whether a special handling is needed for PDUs carrying an EDB indication.

In clause 5.37.8.3 of TS23.501, a NOTE about out of order reception issue is as following:

NOTE: There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.

In case of out of order reception, if some packets from the Data Burst are received by NG-RAN after the PDU with EDB indication, the real last received PDU with no EDB indication is actually more important than the PDU that carries an EDB indication.

Therefore, for the drop precedence, we should not consider the consequence of dropping a PDU that carries an EDB indication is greater than dropping the real last received PDU with no EDB indication, nor is greater than other PDUs with no EDB indication. In other word, it is unnecessary to perform separate DSCP marking for PDUs that carries an EDB indication.

In Rel-18, SA2 also has studied the case that the packet of the end PDU of PDU Set/Data Burst is lost, and concluded that no further standardization is needed. Therefore, it is unnecessary to assign a dedicated DSCP value for PDUs carrying End of Data Burst indication in advance to vary drop probability in the transport network.

With these clarifications, it is proposed to remove the Editor’s note in Sol#13 and add a NOTE to indicate there is no need to perform separate DSCP marking for PDUs carrying an End of Data Burst indication.

2.	Proposal

It is proposed to update TR 23.700-70 as follows.

# * * * * First change * * * *

## 6.13	Solution #13: Multiple DSCP markings per QoS Flow

### 6.13.1	Key Issue mapping

This solution addresses Key Issues #3 (Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network).

### 6.13.2	Description

Illustrated in Figure 6.13.2-1 is a solution for multiple DSCP markings per QoS Flow.

Figure 6.13.2-1: Multiple transport level marking per QoS Flow

The following are the salient features of this solution:

-	The N4 rule includes a Transport Level Marking List that contains a list of PDU Set Importance values, each of which is associated with a DSCP marking. The SMF may take into account the DNN, S-NSSAI, or other locally configured information when determining the Transport Level Marking List.

Editor's note:	It is FFS whether the N4 rule is FAR, QER or both.

-  	In an alternative solution, UPF derives the DSCP for DL packets (N3/N9 interface) based on PDU Set Importance value(s) for a given PDU Set and/or for the whole QoS Flow. The SMF can indicate to the UPF (via N4 rule) that DSCP marking should be performed based on PDU Set Importance based on UPF's own determination. UPF assigns different DSCP marking values per PDU Set within the same AF class of the DSCP value signalled by the SMF for the QoS flow, based on UPF's priority determination.

NOTE 1: It is unnecessary to assign a dedicated DSCP marking value for PDUs carrying an End of Data Burst (EDB) indication.

NOTE 2:	It is recommended that DSCP markings only be used to vary the drop precedence between PDUs. If the Class Selector Codepoint of the DSCP markings varies within a QoS Flow, the packets of the QoS Flow can be reordered by the transport network.

-	After determining the PDU Set-level information of a downlink PDU arriving on N6, the UPF encapsulates the downlink PDU with a GTP-U header as indicated in the N4 rule and includes the PDU Set-level information (if available) in the GTP-U header. The UPF then forwards the GTP-U packet on the N3/N9 reference point and includes the DSCP marking in the transport level header that corresponds to the derived PDU Set Importance value, as indicated in the Transport Level Marking List in the N4 rule.

### 6.13.3	Procedures

There are no changes to existing procedures; only a new parameter (Transport Level Marking List) in the N4 rule.

### 6.13.4	Impacts on services, entities and interfaces

SMF impact:

-	Ability to send a Transport Level Marking List in the N4 rule associated with a QoS Flow carrying PDU Sets. In the alternative solution, SMF instructs UPF via N4 rule to assign per PDU Set DSCP values for downlink traffic, either based on UPF's determination or from a specific Assured Forwarding traffic class (e.g. AF4x) of the DSCP value which is sent by SMF.

UPF impact:

-	Ability to use the derived PDU Set Importance value of an N6 downlink packet for determining the transport level marking for the corresponding GTP-U packet on N3/N9 as indicated in the Transport Level Marking List in the N4 rule. In the alternative solution, UPF derives the DSCP marking as per the indication from SMF.

# * * * * End of changes * * * *