3GPP TSG-WG SA2 Meeting #162	S2-2404676

# Changsha, China, April 15th – 19th, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0439 | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on U2U Relay selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection can be used with some clarifications to support selection of a relay while the End UEs are using direct PC5 communication, as per the RAN2 LS in R2-2401916. Supporting a relay selection mechanism that enables moving a connection from a direct PC5 connection to a U2U relay connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to support the 5G ProSe UE-to-UE Relay reselection when the end UEs have direct PC5 connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA2 specification and RAN2 specification is not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.x.1 (new), 6.7.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.7.x	5G ProSe UE-to-UE Relay selection

#### 6.7.x.1	General

When 5G ProSe End UEs are connected via a direct PC5 connection, a 5G ProSe End UEs may trigger the 5G ProSe UE-to-UE Relay selection based on conditions (e.g. the measured signal strength of PC5 unicast link) as specified in TS 38.300 [12].

For 5G ProSe UE-to-UE Relay selection, a 5G ProSe UE-to-UE Relay may be discovered by either the discovery procedures defined in clause 6.3.2.4 or by the negotiated 5G ProSe UE-to-UE Relay selection procedure defined in clause 6.7.x.2.

The negotiated UE-to-UE Relay selection defined in clause 6.7.x.2 applies to both Layer-2 UE-to-UE Relay and Layer-3 UE-to-UE Relay. One 5G ProSe End UE initiates the UE-to-UE Relay selection procedure, the 5G ProSe End UEs can negotiate a 5G ProSe UE-to-UE Relay to use using the existing direct PC5 connection and establish the communication via the selected 5G ProSe UE-to-UE Relay prior to releasing the direct PC5 connection.

#### 6.7.x.2	Negotiated 5G ProSe UE-to-UE Relay selection

The procedure defined in clause 6.7.4.2 can be applied to negotiated UE-to-UE Relay selection for both Layer-2 UE-to-UE Relay and Layer-3 UE-to-UE Relay, with the following adaptations:

-	In step 1, a PC5 unicast link is established between the 5G ProSe End UEs, based on the procedure defined in clause 6.4.3.1.

-	In step 2, the initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform UE-to-UE Relay selection.

-	In step 3 and step 5, the message is transferred directly between the 5G ProSe End UEs.

-	In step 3 and step 4, Relay re-selection is used to indicate Relay selection.

-	In step 6, the procedure defined in clause 6.7.2 is used for Layer-2 UE-to-UE Relay case, and the procedure defined in clause 6.7.1.1 is used for Layer-3 UE-to-UE Relay case.

# * * * * End of changes * * * *