SA WG2 Meeting #162	S2-2404737

# 15 – 19 April 2024, Changsha, China

Source: 	NTT DOCOMO

Title: 	KI#2: Update Sol#1: Provisioning of CAG info to the network that 5G Femto serves

Document for: 	Approval

Agenda Item: 	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This contribution proposes a resolution for EN.

1	Discussion

The following Editor's note needs to be resolved.

Editor's note:	Whether the flag in the above is needed FFS.

This flag has been introduced to limit the number of messages for AMF to access 5G CAS. However, the effect appears dubious since operators most probably set this flag to be true for most subscribers. In EPS CSS, the concept of Autonomous CSG roaming, which is based on Service Level Agreement between VPLMN and HPLMN and which does not show up in protocols, is used. It is considered sufficient to use the same concept for 5G Femto as well.

In addition, an editorial clean-up is proposed; the direction of the arrow of step 7 in Figure 6.1.2.1-1 is corrected.

2	Proposal

It is proposed to update TR 23.700-45 according to the following text.

********** First Change **********

## 6.1	Solution #1: Provisioning of CAG info to the network that 5G Femto serves

### 6.1.1	Description

This solution mainly addresses two aspects that KI#2 states:

i)	"The provisioning/updating of CAG info to the network that 5G Femto serves [..] will be considered".

ii)	"consider the scenario of allowing the access to a CAG cell in the [..] visited network".

This solution introduces a new NF called 5G-CAS. 5G-CAS is a UDM-like repository in the Serving PLMN to store CAG info. An authorized administrator inputs CAG info via NEF of the Serving PLMN to 5G-CAS. Either AMF retrieves the CAG info from 5G-CAS or 5G-CAS notifies AMF of the CAG info update. AMF then sends it to gNB, when applicable, and to UE.

CAG info in 5G CAS is deployed per UE and provisioned by the authorized administrator per UE. AMF retrieves the CAG info and is notified of the CAG info update per UE.

NOTE 1:	5G-CAS is conceptually the same as EPS CSS.

NOTE 2:	Control to allow only an authorized administrator to access NEF relies on the CAPIF functionality. That is not part of this solution.

NOTE 3:	Provisioning of CAG info into 5G CAS can be performed via OAM instead of via NEF.

This solution works also for the non-roaming case; 5G-CAS stores CAG info for non-roaming UEs as well.

### 6.1.2	Procedures

#### 6.1.2.1	Registration procedure

Figure 6.1.2.1-1: Retrieval of CAG configuration provisioned in the Serving PLMN

1-7.	The same as steps 1, 14a, 14b, and 14c in clause 4.2.2.2.2 of TS 23.502 [3].

8-9.	If HPLMN has enabled retrieval of CAG info in the Serving PLMN (via Service Level Agreement), AMF retrieves the Serving PLMN-specific subscription data containing Allowed CAG list for the UE from 5G CAS.

10-11.	AMF subscribes to the notification of update of CAG info from 5G CAS.

12.	AMF concatenates the Allowed CAG list for the Serving PLMN retrieved from the subscription data in HPLMN and the Allowed CAG list retrieved from 5G CAS in the Serving PLMN and creates an Allowed CAG list. AMF provides this Allowed CAG list to gNB, when applicable, and to UE.

#### 6.1.2.2	Parameter provisioning

Figure 6.1.2.2-1: CAG configuration provisioning to the Serving PLMN

1-4.	The same as steps 1,2,5 and 6 in clause 4.15.6.2 of TS 23.502 [3], except for that CAG information is provisioned. The CAG info is provisioned per UE basis.

NOTE 1:	Control to allow only an authorized administrator to access NEF relies on the CAPIF functionality. That is not part of this solution.

NOTE 2:	Provisioning of CAG info into 5G CAS can be performed via OAM instead of via NEF.

#### 6.1.2.3	Parameter update to UE

This procedure applies when UE has registered and AMF has subscribed to the notification of update of CAG info from 5G CAS during the registration procedure (See steps 10-11 in Figure 6.1.2.1-1).

Figure 6.1.2.3-1: CAG configuration update to UE

1.	5	G CAS sends to AMF a notification of the Serving PLMN-specific subscription data update per UE basis.

2-3.	AMF concatenates the locally stored Allowed CAG list and the Allowed CAG list contained in the notification and creates an Allowed CAG list. Then, the same steps as steps 1, 2a, and 2b in clause 4.2.4.2 of TS 23.502 [3] applies. (When needed, the same step as step 2c in clause 4.2.4.2 of TS 23.502 [3] also applies.)

### 6.1.3	Impacts on services, entities and interfaces

The solution has the following impacts:

5G CAS:

-	This stands for 5G CAg subscriber Server. 5G CAS is a new NF that is in the Serving PLMN and stores the Serving PLMN-specific subscription data for both non-roaming UEs and roaming UEs. 5G CAS supports a subset of service operations that UDM supports.

AMF:

-	AMF checks the local configuration based on Service Level Agreement and accesses 5G CAS when needed. AMF creates an Allowed CAG list for the Serving PLMN per UE by concatenating Allowed CAG lists, one of which is retrieved from HPLMN and the other of which is retrieved in the Serving PLMN.

NEF:

-	NEF receives CAG info related inputs from an administrator and accesses 5G CAS.

NOTE:	OAM can be used instead of NEF.

gNB and UE are not impacted.

********** End of Changes **********