SA WG2 Meeting #S2-162	S2-2405231

15 – 19 April, 2024, Changsha, China	(rev of S2-2404372)

Source:	InterDigital Inc., Samsung

Title:	KI#1, New Solution, Multi-USS Support

Document for:	Approval

Agenda Item: 	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a solution to support multi-USS in 5GC.

# 1	Discussion

This contribution proposes a solution for multi-USS support in 5GC addressing key issue #1.

# 2	Proposal

It is proposed to adopt the following new solution in TR 23.700-59.

* * * * Start of Changes (All Next Text) * * * *

## 6.X	Solution #X: Multi-USS Support in 5GC

### 6.X.1	Key Issue mapping

This solution addresses KI#1, “Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions”, focusing on the following aspect:

-	The scenario of multiple USS serving different geographical areas corresponding to the UAV flight path.

### 6.X.2	Description

The procedure for authentication and authorization of UAV by USS (UUAA) in TS 23.256 [2], clause 5.2 allows a particular USS to authenticate and authorize the UAV when it Registers or establishes a PDU Session. In current release a UAV is served by single USS for the duration of the connectivity between the USS and the UAV.

In this solution it is proposed to enhance the UUAA procedure such that the USS provides the UAS NF with a list of applicable serving USS info. UAS NF stores the list of serving USS info in the UUAA context and UAV UE store the information internally.It is also possible that list of serving USS info is preconfigured in UAS NF and UAV. The serving USS info also includes the serving area of the serving USS info. When a change of current serving USS occurs, the UAS NF is informed by the USS about the change. The UAS NF updates the UUAA context . UAS NF verifies whether a USS is authorized to initiate a change of USS based on the list of serving USS info in the UUAA context.

Editor’s note: It shall be verified by SA3, whether the updating UUAA context in UAS NF from old USS to target USS can be updated without performing UUAA authentication and authorization.

### 6.X.3	Procedures

In 5GS, the following procedures are used for the authorization to support multiple USSs:

- The UE indicates multi-USS capability to in the Registration Request message in the Registration procedures as defined in clause 4.2.2.2 of TS 23.502 [X].

Editor’s note: It is FFS whether multi-USS capability may be informed to USS by application layer.

NOTE : How the serving USS acquire of information of other USSes is out of 3GPP scope.

- The procedure for UUAA-MM as specified in TS 23.256 [2], clause 5.2.2.2 with the following enhancements:

- In step 2 and 3, an indication of multi-USS support is included, respectively by AMF and UAS NF;

- In step 5, USS sends Naf_Authentication_AuthenticateAuthorize Response authentication response message to UAS NF, which includes list of serving USS info (e.g., list of IP addresses, serving area information). The UAS NF stores list of serving USS info in the UUAA context;

- In step 6, UAS NF forwards information received from USS which includes the list of serving USS info which is further forwarded to the UE in step 10 (along UAV authorization information from USS).

- The procedure for UUAA-SM as specified in TS 23.256 [2], clause 5.2.3.2 with the following enhancements:

- In step 1 and 2, an indication of multi-USS support is included, respectively by SMF and UAS NF;

- In step 4, USS sends Naf_Authentication_AuthenticateAuthorize response authentication response message to UAS NF, which includes the list of serving USS info (e.g., list of IP addresses). The UAS NF stores list of serving USS info in the UUAA context;

- In step 5, UAS NF forwards information received from USS which includes the list of serving USS info which is further forwarded to the UE during in step 7 (along UAV authorization information from USS)

-    UAV provides new USS information while triggering any PDU session signalling or registration signalling for UUAA.

In 5GS, the following procedures are used for the UAV tracking with support for multiple USSs:

- The procedures for UAV Tracking as specified in TS 23.256 [2], clause 5.3 with the following enhancements:

- UAV Location Reporting procedure in clause 5.3.2 and UAV presence monitoring procedure in clause 5.3.3, step 1, UAS NF checks that USS is in the list of serving USS info in the UUAA context;

- Obtaining list of Aerial UEs in a geographic area procedure in clause 5.3.4, step 5, the UAS performs a filtering based on the list of serving USS in the UUAA context.

In 5GS, the following procedures are used for the change of current serving USS:

- A procedure similar to USS initiated C2 pairing policy configuration in 5GS as specified in TS 23.256 [2], clause 5.2.5.4.1 with the following differences:

- USS invokes Nnef_AFsessionWithQoS_Update service operation to the UAS NF for the PDU Session used for communication between the UAV and PCF. UAS NF checks that USS is in the list the serving USS in the UUAA context. The request (and following steps) includes the new serving USS IP address (instead of UAVC pairing info/IP address);

- UAS NF triggers Npcf_PolicyAuthorization_Update providing new serving USS IP information;

### 6.X.4	Impacts on services, entities and interfaces

UAS NF:

- Receives multi-USS support indication from AMF/SMF during UUAA procedures.

- Receives from USS and stores in UUAA context a list of serving USS info during UUAA procedures.

- Sends list of serving USS info to AMF/SMF during UUAA procedures.

- Authorizes USS requests (location tracking, change of USS/AF session configuration) based on the list of serving USS info in the UUAA context.

AMF/SMF

- Sends multi-USS support indication to UAS NF during UUAA procedures.

UE

- Provides multi-USS capabilities during Registration procedure.

- Receives list of serving USS info from AMF/SMF during UUAA procedures.

* * * * End of Changes * * * *