3GPP SA WG2 Meeting #162	S2-2405788

Changsha, China, 15-19 April 2024

# Title:	LS on FS_VMR_Ph2 solution impacts to RAN

# Response to:	-

# Release:	Rel-19

# Work Item:	FS_VMR_Ph2

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 would like to inform RAN3 that SA2 had progressed the Rel-19 FS_VMR_Ph2 study and documented solution proposals in TR 23.700-06v0.3.0.

In order to evaluate the solutions and derive principles/conclusions for normative work, SA2 would like to obtain some feedback from RAN3 regarding the following questions:

- Question 1: SA2 has an interim conclusion that the MWAB (MWAB-UE) authorization is based on dedicated slice ID(s) (S-NSSAI(s)). Therefore, from SA2 perspective no MWAB-specific AS layer indication at MWAB-UE's RRC establishment is required. SA2 would like to also point out that if there was a strict need for indication at AS layer, the existing mechanism of including S-NSSAI in RRC connection establishment should be considered. SA2 would like to invite RAN3 to provide the feedback if any scenario considered by RAN3 needs such a MWAB-specific AS layer indication. Note that SA2 considers the MWAB-gNB and MWAB-UE may register and connect to different PLMNs, and the authorization of the MWAB-UE is different from the MWAB-gNB service authorization/configuration and activation by OAM.

- Question 2: For the MWAB (MWAB-UE) authorization result, SA2 could not identify any reason to inform/update that to the NG-RAN serving the MWAB-UE. Therefore, SA2 would like to understand from RAN3's perspective whether the MWAB authorization result needs to be provided to the NG-RAN serving the MWAB-UE.

- Question 3: To support mobility of the MWAB, some solutions assume that the MWAB-gNB can instantiate two cells (with same gNB ID or different gNB ID), and handover connected UEs between the two cells. The different gNB IDs use case is driven by the need to change AMF if the MWAB moves into a geographic area where a different AMF must be chosen to serve UEs. SA2 would like to ask RAN3 to confirm if this can be supported or not.

- Question 4: SA2 discussed the scenario of Xn interface between RAN nodes over the IP connectivity provided by the PDU session of MWAB-UE, and would like to ask RAN3 if this scenario can be supported by RAN3.

SA2 would also like to invite RAN3 to provide any feedback on candidate solutions documented in the TR 23.700-06 with RAN impacts and inform SA2 if any aspect in the solutions cannot be supported within the RAN3 Rel-19 work on Additional Topology Enhancements.

The target completion date of FS_VMR_Ph2 is June 2024 and Rel-19 Stage 2 is planned to be frozen in December 2024. Therefore, feedback/information from RAN3 considering these timelines would be appreciated.

2. Actions:

To RAN3:

ACTION: 	SA2 asks RAN3 to review the questions and provide answers and other feedback accordingly.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #163		27-31 May 2024	Jeju, KR

TSG-SA2 Meeting #164		19-23 August 2024	Maastricht, NL