SA WG2 Meeting ##162	S2-2403871

15 - 19 April, 2024, Changsha, China

3GPP TSG-CT WG1 Meeting #147	C1-241722

# Athens, Greece, 26 February-1 March 2024

# Title:	LS on LCS user plane connection binding to the UE

# Response to:	-

# Release:	Rel-18

# Work Item:	5G_eLCS_Ph3

Source:	CT WG1

To:	SA WG2

Cc:	CT WG4

Contact Person:

Name:	Sunghoon Kim

Tel. Number:	-

E-mail Address:	sunghoon@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

As part of the work on 5G_eLCS_Ph3, CT1 has specified protocol solutions for location service over user plane as defined in new TS 24.572:

Step 1) UPP-CM procedure during which the LMF provides the user plane connection information to the UE.

Step 2) The UE performs user plane connection establishment with the LMF (i.e., TLS connection) based on the information received from Step 1.

Step 3) LCS-UPP procedure to transfer location service messages between the UE and the LMF.

CT1 has found following issues:

- After the TLS connection is established, the UE and the LMF initiate LPP or LCS-SS procedure. For the LMF-initiated case, given that multiple UEs are served by the LMF and multiple TLS connections are active, the LMF needs to determine a TLS connection for the target UE. However, it has not been specified how to link the association between the TLS connection and the UE, e.g., how the LMF knows which TLS connection is for the UE who has received the UPP-CM command message. If the TLS connection can be linked to the ‘wrong UE’ then there could be a risk of location related data from one UE which is sent to another UE.

- Once the binding of the TLS connection to the UE is done, if there is a new LCS service request (e.g., MT-LR) for the same UE, the AMF invokes Nlmf_location_determinelocation service operation to the LMF. However, it has not been specified how the LMF associates the LCS service request to the TLS connection of the UE. In this case, the LMF cannot determine which TLS connection can be used for the LCS service request. Therefore, the LMF cannot reuse the TLS connection of this UE for subsequent LCS service request(s).

2. Actions:

To SA WG2 group.

ACTION: 	CT WG1 kindly asks SA WG2 to provide a resolution for above-described issues and feedback to CT1.

3. Date of Next CT1 Meetings:

CT1#148                          15th April- 19th April 2024                       China, CN

CT1#149                           27th May- 31st May 2024                       India, IN