SA WG2 Meeting ##162	S2-2403904

15 - 19 April, 2024, Changsha, China

3GPP TSG-SA3 Meeting #115	S3-240835

Athens, Greece, 26th February - 1st March 2024

Title:	LS on UUAA Status Information availability for SMF

Response to:	LS C4-230790 on Removal of the uavAuthenticated IE from Create SM Context Request from CT4

Release:	Rel-17/18

Work Item:	ID_UAS

Source:	SA3 (Lenovo)

To:	CT4

Cc:	SA2, CT1

Contact person:

Name:	Sheeba Backia Mary Baskaran

E-mail Address:	smary@lenovo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	[S3-241002] [S3-241003]

# 1	Overall description

Thanks for the clarification question asked in LS C4-230790 related to the need of uavAuthenticated information element.

CT4 Question: CT4 kindly asks SA3 to clarify why the functionality of uavAuthenticated information element is needed.

SA3 Answer: According to SA3 UUAA is to authenticate the UAV for security reasons, and we do not see any security benefits in repeating.  But in the latest SA2 TS 23.256 V18.2.0, texts were removed related to AMF and SMF relations on UUAA execution [See SP-231244]. In its current form UUAA is triggered during registration and PDU session establishment independently.

To align with latest SA2 TS 23.256 V18.2.0, SA3 agreed the Rel.17 and Rel.18 CRs which are attached here for your information.

# 2	Actions

To CT4

ACTION: 	Find the attached latest SA3 CRs agreed for your information.

# 3	Dates of next TSG SA WG 3 meetings

SA3#115AdHoc-e	15 - 19 April 2024	Electronic meeting

SA3#116	20 - 24 May 2024		Jeju (South Korea)

SA3#117	19 - 23 August 2024	Maastricht (Netherlands)