SA WG2 Meeting ##162	S2-2403908

15 - 19 April, 2024, Changsha, China

3GPP TSG-SA3 Meeting #115	S3-240906

Athens, Greece, 26th February - 1st March 2024

Title:	Reply LS on DNS over TLS (DoT)

Response to:	LS (S2-2313793) on Reply LS on DNS over TLS (DoT) and DNS over HTTPS (DoH)

Release:	Rel-18

Work Item:	TEI18

Source:	3GPP SA3

To:	SA2

Cc:

Contact person:	German Peinado

german.peinado@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Agreed CRs S3-240907, S3-240908, S3-240909

# 1	Overall description

SA3 thanks SA2 for the reply LS on DNS over TLS (DoT) and DNS over HTTPS (DoH).

The feedback to the questions provided in the LS is the following:

1) SA2 kindly asks SA3 to specify now the security information in the PCO should be used by the UE.

Response:

Please note that the updated security aspects of DNS in the context of EDGE are described in Annex T of TS 33.501. DNS over TLS as specified in IETF RFC 7858 [83] and RFC 8310 [84] shall be supported by the UE and the EASDF. Accordingly, the following CRs have been agreed: S3-240907, S3-240908, S3-240909.

More specifically, in clauses T.3 and T.4 of TS 33.501, it is specified as well that the EASDF security information (credentials to authenticate the EASDF, supported security mechanisms, port number, etc.) can be preconfigured in the UE by using out of band mechanisms or if the core network is used to configure the security information, this is provided to the UE via PCO. According to the clause 6.4.1.3 of TS 24.501, upon receiving the DNS server security information, the UE passes it to the upper layer. The UE uses this information to send the DNS over TLS. Additionally, the clause 10.5.6.3 of TS 24.008 provides the configuration of the different options of DNS over TLS specified in the RFC 7858.

2) SA2 ask if SA3 have any concerns to allow the security information to be provided by the EASDF to the SMF.

Response:

EASDF security information may either be locally configured in the SMF or retrieved by the SMF from the network (EASDF). If the EASDF and SMF belong to the same security domain there is no need to specify any standard security mechanism for such communication. For data protection purposes, if they belong to different security domains, NDS/IP as defined in TS 33.210 applies.

# 2	Actions

To SA2

ACTION:

SA3 kindly asks SA2 to take the provided information into consideration.

# 3	Dates of next TSG SA WG 3 meetings

SA3#115AdHoc-e	15 - 19 April 2024	Electronic meeting

SA3#116	20 - 24 May 2024		Jeju (South Korea)

SA3#117	19 - 23 August 2024		Maastricht (Netherlands)