3GPP TSG-SA WG2#162	S2-2403985

Changsha, China, 15 – 19 April 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	KI 2.2: New solution for PDU Session Establishment

Document for:	Discussion/Approval

Agenda Item:	19.3

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes a new solution for KI#2.2 to address the scenario in which 3GPP access in not available when a connection via NIN3A needs to be established.

# 1.	Discussion

TS 23.700-54 include solutions that are based on simplified architectures for ATSSS, in which there is no NAS signaling exchange between CN and UE via the (Non-Integrated) Non-3GPP access. See, for example, Solution 2.7 and Solution 2.8 in clauses 6.2.7 and 6.2.8, respectively.

Such solutions assume that the UE uses both 3GPP access and (the Non-Integrated) Non-3GPP access to exchange data with the UPF. However, there can be scenarios in which this is not always true. As an example, we can think of an IoT device (such as a surveillance camera) which is (semi-)stationary, needs only pure Internet for connectivity and does not need to exchange UP data over 3GPP access.

The solution proposed in clause 2 for the scenario above builds on top of the architecture and the principles described in clause 6.2.7 (ATSSS-Lite).

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-54:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	IETF RFC 9298: "Proxying UDP in HTTP".

[7]	IETF RFC 9484: "Proxying IP in HTTP".

[8]	IETF draft-ietf-masque-connect-ethernet: "Proxying Ethernet in HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	IETF RFC 9114: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

[10]	IETF draft-ietf-httpbis-connect-tcp: "Template-Driven HTTP CONNECT Proxying for TCP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[11]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[12]	IETF RFC 9000: "QUIC – A UDP based Multiplexed and Secured Protocol".

[13]	IETF RFC 9001: "Using TLS to Secure QUIC".

[14]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[15]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[16]	IETF draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[17]	3GPP TR 23.700-53: "Study on access traffic steering, switching and splitting support in the 5G system architecture; Phase 3".

[18]	IETF RFC 9369: "QUIC Version 2".

[19]	IETF RFC 9220: "Bootstrapping WebSockets with HTTP/3".

[20]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[x]	3GPP TS 29.509: "5G System; Authentication Server Services; Stage 3".

[y]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

# >>>>NEXT CHANGE<<<<

### 6.2.X	Solution #2.X: ATSSS-Lite session management when UE-UPF communication does not require 3GPP access.

#### 6.2.X.1	Description

To address the scenarios in which a UE is not required to exchange UP data via 3GPP access when ATSSS-Lite (see clause 6.2.7) is used, the concept of NIN3A-only session is introduced. A NIN3A-only session enables MPQUIC/MPTCP communication between UE and UPF only over NIN3A and does not exchange UP data over 3GPP access. There are two options to enable such kind of session:

Option 1: Establish a NIN3A-only session via PDU Session Establishment over 3GPP access. This option is based on the architecture described in clause 6.2.7.1.2 and does not require architectural modifications. The more detailed procedural steps are described in clause 6.2.X.2.1.

Option 2: Establish a NIN3A-only session via in-band signaling over NIN3A. This option requires the UPF (MPQUIC/MPTCP proxy) to communicate with the Authentication Server Function (AUSF, see Figure 6.2.x.1-1 below). The related procedural steps are described in clause 6.2.X.2.2.

Figure 6.2.x.1-1: Architecture to support establishing a NIN3A-only session via in-band signaling (Option 2)

#### 6.2.X.2	Procedures

##### 6.2.X.2.1	Option 1: Establishment of NIN3A-only session via PDU Session Establishment over 3GPP access

The main steps for this option can be summarized as follows:

1.	At registration, the PCF provides the UE with URSP rules indicating that UE may establish NIN3A-only sessions for certain traffic.

2.	The UE requests via 3GPP access the establishment of NIN3A-only PDU session (e.g., by using a dedicated DNN).

3.	The SMF receives PCC rules to enable a NIN3A-only PDU Session for that UE.

4.	The SMF provides to the UE the information for establishment of a NIN3A-only PDU Session via 3GPP access:

A. It provides the UPF IP address/port number and MPQUIC/TLS security material (e.g., CA certificate) to UE to later establish MPQUIC/MPTCP connection over NIN3A.

B. 	It indicates to UPF and UE how to handle the NIN3A-only session, e.g., keep NIN3A connection for a predefined time that is given to the UE/UPF (preconfigured in the UE/UPF or provided by the SMF to the UE/UPF)

5.	The UE establishes MPQUIC/MPTCP connection with the UPF over NIN3A

6.	The UE then requests over 3GPP access the SMF to release the 3GPP access resources for the NIN3A-only PDU session. This does not release UE context in the SMF and in the UPF.

7.	Based on the indication in Step 4.B, the UPF and the UE keep the NIN3A connection for a predefined amount of time and can exchange data over NIN3A.

8.	Before the timer expires, the UE can re-establish the communication over 3GPP access and request the SMF (via PDU Session Modification) to refresh the timer. Step 4.B is then repeated.

9.	When the timer expires, the UE and the UPF release the NIN3A resources for the PDU session. If, at that moment, the 3GPP access is available, the UE can trigger PDU Session release. If the 3GPP access is not available, the UE and the UPF release the resources locally and the UPF notifies SMF.

##### 6.2.X.2.2	Option 2: Establishment of NIN3A-only session via in-band signaling over NIN3A

In this solution, UE establishes a TLS session with the UPF by authenticating UPF based on the UPF certificate; and the UE authenticates to the UPF by performing EAP-AKA' with AUSF via UPF that takes the role of a pass-through authenticator.

The solution re-uses EAP-AKA' [y] between a UE and the AUSF via UPF, which acts as a pass-through authenticator and receives authentication result from AUSF.

Figure 6.2.x.2.2: Establishment of NIN3A-only session via in-band signaling over NIN3A (Option 2)

The main steps of this option can be summarized as follows:

1A.[Optional] The UE may be configured with the address information of the UPF with MPQUIC/MPTCP proxy functionality over Nx (e.g., IP address with port number, FQDN, etc.)

1B. The UE is configured with the TLS security material (CA certificate) of the UPF (MPQUIC/MPTCP proxy).

2.	The UPF is configured to support the MPQUIC/MPTCP proxy functionality and to support NIN3A-only communication.

3.	[Optional] The UE may use the same mechanism as in clause 6.3.6 of TS 23.501 for ePDG/N3IWF discovery to discover the UPF used as MPQUIC/MPTCP proxy.

NOTE 1:	At least one among Steps 1A and Step 3 need to be executed.

4.	The UE triggers the establishment of QUIC/TCP communication over NIN3A, and a TLS session is established between UE and MPQUIC/MPTCP proxy.

5. 	The UE performs EAP-AKA' [y] with AUSF over the established TLS session in step 4, where the MPQUIC/MPTCP proxy acts as a pass-through authenticator. This requires the transport of EAP-AKA' messages over the MPQUIC/MPTCP protocol and the indication of NIN3A access when the MPQUIC/MPTCP proxy invokes authentication service operation to the AUSF.

NOTE 2:	It is assumed that the UPF can use the Nausf_UEAuthentication service defined in TS 23.502 [4] clause 5.2.10.2, and the AUSF uses Nudm_UEAuthentication service defined in TS 23.502 [4] clause 5.2.3.4.

NOTE 3:	The details of how EAP-AKA' authentication is performed via UPF are to be discussed by SA3.

NOTE 4: 	EAP-AKA' messages can be encapsulated over HTTP similarly to the extensions made for SBI in TS 29.509 [x] clause 5.2.2.2.3. The details of how EAP-AKA' messages are encapsulated within MPQUIC/MPTCP messages are to be discussed by CT1.

6.	The establishment of MPQUIC/MPTCP is finalized and data can be transferred over the NIN3A connection.

#### 6.2.X.3	Impacts on services, entities and interfaces

##### 6.2.X.3.1	Impacts of Option 1

UE:

-	Receives enhanced URSP rules from PCF indicating that NIN3A-only sessions are allowed

-	Receives indication from SMF of how long the NIN3A connection can be used. Alternatively, such value is preconfigured in the UE.

-	Requests the SMF to establish a NIN3A-only session and to terminate 3GPP access resource of the PDU Session once MPQUIC/MPTCP parameters for NIN3A connection are received/available.

UPF:

-	Receives indication from SMF of how long the NIN3A connection can be used. Alternatively, such value is preconfigured in the UPF.

SMF:

-	Indicates to UE and UPF for how long the NIN3A connection can be used (optional)

-	Keeps UE context (in SMF and UPF) even after UE requests to terminate 3GPP access resource of PDU Session.

PCF:

-	Provides enhanced URSP rules to UE to indicate which traffic is eligible for NIN3A-only sessions

##### 6.2.X.3.2	Impacts of Option 2

UE (MPQUIC/MPTCP client):

-	capable of exchanging EAP-AKA' messages with UPF

UPF (MPQUIC/MPTCP proxy):

-	capable of relaying EAP-AKA' messages between UE and AUSF

-	support new Ny interface towards AUSF

-	needs to indicate to AUSF that authentication for NIN3A access

AUSF:

-	supports new Ny interface towards UPF

-	needs to know that the authentication is happening for NIN3A

# >>>>END OF CHANGES<<<<