SA WG2 Meeting #162  	  S2-2403997

15 April-19 April 2024, Changsha,China

Source:	ZTE

Title:	TR 23.700-84: VFL Terminology update and 2 EN removal

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

# 1. Discussion

As the NOTE 4 of key issue 2 clearly state that:

NOTE 4:	The existing procedures defined for Horizontal FL in TS 23.288 [5] will be taken into account when studying the procedure for VFL.

So this paper propose to remove the VFL active participant and VFL passive participant naming and use server/client naming in the R18 HFL procedure instead.

# 2. Proposal

---------- Start of change ----------

3	Definitions of terms and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

-	Horizontal Federated Learning (HFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

-  Vertical Federated Learning (VFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Clients for local model training have different feature spaces for the same samples (e.g. UE IDs).

Editor's note: whether different feature space required for VFL is FFS.

-  Label: A label is the property of interest that is to be learned in supervised machine learning.

Editor's note: the label definition is to be reviewed and extended if needed.

-  VFL Server: An NF with labels for a VFL training task that may have related inputs data.

-  	VFL Client: An NF with the required inputs data without the required labels for a VFL training task. There can be multiple Clients in VFL.

Editor's note: whether there is a VFL coordinator is FFS.

Editor's note: the terminology definition will be updated.

---------- End of change ----------