3GPP TSG-WG SA2 Meeting #162	S2-2404008

Changsha, China, April 15- April 19, 2024

Source:	China Mobile

Title:	Update on Sol #10 PDU Set information identification based on MoQ

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM Ph2 /Rel-19

Abstract of the contribution: This paper proposes to update solution #10 for MoQ introduction during PDU Session establishment procedure.

1	Discussion

Solution #10 proposed a solution for identifying PDU Set information from encrypted traffic based on the Media over QUIC (MoQ).

In step 0, the QUIC connection has established between UE and AF/AS, which should be clarified how the QUIC connection establishment is performed when UPF acts as an MoQ relay. This paper updates the QUIC connection establishment procedure during PDU Session establishment.

In addition, the format of OBJECT message has been updated in MoQT[9], which is updated accordingly in this paper.

Editor's note: It is FFS how the PDU set information (e.g. PDU Sequence Number within a PDU Set) can be supported by MoQ.

2	Proposal

It is proposed to adopt the following changes into TR 23.700-70.

* * * * 1st Change (revision marked) * * * *

6.10	Solution #10: PDU Set information identification based on MoQ

6.10.1	Key Issue mapping

This solution addresses the Key issue #2 to Support PDU Set information identification for end-to-end encrypted XRM traffic.

6.10.2	Description

QUIC is a UDP-based, stream-multiplexing, encrypted transport protocol that is widely accepted. However, QUIC natively integrates TLS encryption technology, making it difficult for the network intermediate node to identify the application layer information such as PDU Set information.

Media over QUIC (MoQ) develops a simple low-latency media delivery solution for ingest and distribution of media. MoQ focuses on building protocol mechanisms for publication of media and means to identify and receive the media, which can be used over raw QUIC or WebTransport. MoQ specifies a simple method for clients to authenticate to the relay or server to transmit or receive media.

The basic data element of MOQT is an object. An object is an addressable unit whose payload is a sequence of bytes. Objects are comprised of two parts: metadata and a payload. The format of the OBJECT message is as follows:

-	Track Namespace and Track Name: The track this object belongs to. The track identifier obtained as part of subscription and/or publish control message exchanges. Track Alias may be used to identify the Track Namespace and Track Name.

-	Group ID: The object is a member of the indicated group within the track.

-	Object ID: The order of the object within the group. The IDs starts at 0, increasing sequentially for each object within the group.

-	Object Send Order: An integer indicating the object send order or priority value.

-	Object Payload Length: The length of the following Object Payload. If this field is absent, the object payload continues to the end of the stream.

-	Object Payload: An opaque payload intended for the consumer and SHOULD NOT be processed by a relay.

The metadata is never encrypted and is always visible to relays. So, the PDU-set related information could be identified in the relay (i.e. the UPF).

This paper proposes a solution for identifying PDU Set information from encrypted traffic based on the MoQ. In this solution, UPF acting as a relay node could identify the PDU Set information and place it in MoQ metadata/GTP header.

The solution is based on the existing PDU Session Establishment/ modification with the following enhancement:

AF:	Provide Encrypted Traffic Handling Assistance Information along with the Protocol description.

PCF:	Determine PCC Rules based on Encrypted Traffic Handling Assistance Information and the Protocol description.

SMF:	Provide N4 rule to activate the MoQ relay functionality of UPF.

UPF:	Identify the PDU set information based on MoQ metadata. Maintain two QUIC connections (i.e. one between the UE and UPF, another one between the UPF and the AS) for one QoS flow.

UE:	Support the QUIC/MoQ connection establishment with UPF.

Editor's note:	It is FFS how the PDU set information (e.g. PDU Sequence Number within a PDU Set) can be supported by MoQ.

6.10.3	Procedures

The procedures is used for QUIC connection establishment with encrypted traffic during UE requested PDU Session Establishment procedure. The network could help to identify the PDU set information based on the AF provided assistance information.

Figure 6.10.3-1: PDU Set information identification based on MoQ

1.	The AF/AS may provide Encrypted Traffic Handling Assistance Information (ETHAI) along with the Protocol Description to the NEF/PCF, the ETHAI includes the MoQ relay support indication along with one or more URI of AS, UE GPSI. Those information can be provided via AF triggered traffic influence procedure.

2.	UE triggers the PDU session establishment with Requested DNN.

3.	The AMF selects an SMF and invokes the Nsmf_PDUSession_CreateSMContext Request with including the Requested DNN.

4.	The SMF may perform an SM Policy Association Establishment/Modification procedure with the PCF. The PCF generates PCC rules based on the AF provided ETHAI and Protocol Description, and sends the PCC rules to the SMF, including MoQ relay support indication and AS address.

5.	The SMF determines that the MoQ relay is required according to the Requested DNN, based on PCC Rules from PCF. And the SMF will perform UPF selection considering the MoQ relay capability of UPF.

6.	SMF initiates an N4 Session Establishment/Modification procedure with the selected UPF, and indicates UPF the requested DNN for the specific PDU Session and the AS address received from PCF to establish QUIC connection with AF/AS.

7.	Then the UPF triggers the QUIC connection (connection 1) establishment procedure with the AS identified by the AS address (e.g. URI).

- The UE obtains the MoQ relay address (i.e. UPF address) via application layer. Then the UE initiates the QUIC connection (connection 2) establishment procedure with the UPF based on the UPF address.

- For the downlink direction, the PSA UPF identifies PDU Set information from MoQ metadata. The UPF identifies the MoQ metadata and convert the MoQ metadata into PDU set information and sends the PDU Set information in the GTP-U header to RAN. The PDU Set Information can be extracted from the metadata as following:

-	The PDU Set Sequence Number could be extracted from Object Sequence.

-	Indication of End PDU of the PDU Set could be identified based on the Payload Length.

-	PDU Set Size in bytes could be identified based on Object Payload Length.

-	PDU Set Importance could be identified based on Object Object Send Order.

6.10.4	Impacts on services, entities and interfaces

AF/AS:

-	Provide Encrypted Traffic Handling Assistance Information along with the Protocol description.

PCF:

-	Determine PCC Rules based on Encrypted Traffic Handling Assistance Information and the Protocol Description.

SMF:

-	Provide N4 rule to activate the MoQ relay functionality of UPF.

-  Indicate UPF to establish QUIC connection with AF/AS.

UPF:

-	Identify the PDU set information based on MoQ metadata.

-	Maintain two QUIC connections (i.e. one between the UE and UPF, another one between the UPF and the AS) for one QoS flow.

UE:

-	Support the QUIC/MoQ connection establishment with UPF.

* * * * End of Changes * * * *