3GPP TSG-WG SA2 Meeting #162	S2-2404021

# Changsha, China, April 15- April 19, 2024

Source: 	China Mobile

Title:	Evaluation and Conclusion for KI#1

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: Propose Evaluation/Conclusion for KI#1.

1. Discussion

This proposal proposes the evaluation and conclusion for KI#1 as baseline for normative work.

2. Proposal

It is proposed to capture the following changes in TR 23.700-63.

FIRST CHANGE

7	Overall Evaluation

7.X	Key Issue #1: Selection of UPF providing a selected user plane functionality

There are four solutions (Sol#2, Sol#3, Sol#4, and Sol#6) mapping to Key Issue #1.

This Key Issue has the following aspects to study:

-	Justify use cases that necessitate a need to extend the existing UPF capabilities in case UPF has integrated additional functionalities.

-	How to extend the existing UPF advertising capabilities.

-	The mechanism to enhance the UPF selection to support specific user plane functionalities.

Following use cases have been indicated to extend the existing UPF capabilities:

-	Identifying UPF supporting features as NAT (Sol#2).

-	DDoS protection or Firewall support in UPF (Sol#2).

-	Layer 7 Deep Packet Inspection (DPI) with specific DPI levels, e.g. specific transport levels such as QUIC, MP-QUIC, MP-TCP, MASQUE and/or application such as HTTP, RTP, SIP, WebRTC, etc. (including Sol#2, Sol#4).

-	Different hardware configurations (e.g. CPU or NIC (Network Interface Controller) that brings better capacity or latency characteristics) (Sol#3).

-	Partial support of a standardized feature (Sol#3).

-	Non-standard features implemented in UPF, such as e.g. NAT, firewalling or advanced non standardized reporting or forwarding features that cannot be mapped to current information elements as capabilities (Sol#3).

-	Parental control, with optionally specific controls to be enforced to the traffic (Sol#4);

-	MASQUE proxy functionality (Sol#4).

Following methods have been indicated to extend the existing UPF advertising capabilities based on the extended UPF capabilities:

-	Reusing procedures in clause 4.17.1 and clause 4.17.2 of TS 23.502 [3] to support registering the extended UPF functionalities in NRF (including Sol#2, Sol#3, Sol#4, Solt#6).

-	Extend the baseline UPF capabilities announced in N4 via the PFCP Association Setup (including Sol#3, Sol#4).

-	Defining an open and generic value (e.g. octet string) which can be used for non-standard or partially supported features and configured by operator to extend the baseline UPF capabilities announced in N4 and UPF NF profile in NRF (Sol#3).

Following mechanisms have been indicated to enhance the UPF selection to support specific user plane functionalities:

-	Support SMF provisioning of UPF and UPF discovery via the NRF as well as procedures in clause 4.17.6 of TS 23.502 [3] and clause 4.15.4.5.3 of TS 23.502 [3], respectively (including Sol#2, Sol#3, Sol#4, Sol#6).

-	Enhancing UPF selection logic at SMF by dividing UPF capabilities into required and desired for the PDU Session so that the SMF selects a UPF for the PDU Session supporting all the required functionalities and most of the desired functionalities possible (Sol#4).

-	SMF executes UPF selection based on the status of the supported functionalities in the UPF which has been informed by UPF via the NRF (Sol#6).

NEXT CHANGE (all the text is new)

8	Conclusions

8.X	Key Issue #1: Selection of UPF providing a selected user plane functionality

The following aspects are concluded as principles for normative work:

-	Reusing procedures in clause 4.17.1 and clause 4.17.2 of TS 23.502 [3] to support registering the extended UPF functionalities in NRF.

-	The following UPF functionalities are added in UPF profile stored in NRF:

-	NAT functionality, and the type of the NAT functionality e.g. static NAT, pooled NAT, NAPT, easy IP, and NAT server.

-	Packet Inspection functionality, i.e., Layer 7 DPI with specific DPI levels, e.g. specific transport levels such as QUIC, MP-QUIC, MP-TCP, or application such as HTTP, RTP, SIP, WebRTC, etc..

-	DDoS protection or Firewall.

-	DNS snooping.

-	Enhancing UPF selection logic at SMF by dividing UPF capabilities into required and desired for the PDU Session so that the SMF selects a UPF for the PDU Session supporting all the required functionalities and most of the desired functionalities possible.

-	SMF executes UPF selection based on the status of the supported functionalities in the UPF which has been informed by UPF via the NRF.

# * * * * End of changes * * * *