SA WG2 Meeting #162	S2-2404137

April 15-19, 2024, Changsha, P.R.China                            (was S2-2402609)

Source: 	vivo, CATT, LG Electronics

Title: 	Update the Solution#4 for KI#1: Handling awareness of supported TAIs when RAN is onboard

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes to update solution#4 for KI#1.

# 1 Discussion

In TR23.700-29 v0.4.0, solution #2 and solution#4 for KI#1 both are proposed with similar solutions about how to handle the feeder link issue when RAN onboard. One aspect is how to manage the interface when RAN orbits away, the other aspect is how to reduce signaling overhead caused by frequent TAI list updating, i.e. add a “time period” dimension in the mapping relationship between “Global RAN ID, TAI list”. To make the solutions more generic and easier to understand, it is proposed to rearrange the 2 solutions:

- Solution#2: merge aspects of the NG/S1AP interface management (updated with S2-2404686)

- Solution#4: merge aspects of TAI list management (updated with S2-2404137)

Besides, this solution update also solves EN on “How to page the UE via the pre-configured mapping relationship when ISL is available is FFS”. Compared to transparent mode, the regenerative payload can take advantage of ISL regarding paging. Using ISL, a satellite without a direct connection to the ground station (feeder link) can still provide service to UE. By incorporating information about which satellites are connected through ISL, along with details like the TAI list and the satellites' valid service periods, the AMF can effectively send paging requests to UEs serviced through ISL-connected satellites..

# 2 Proposal

It is proposed to update solution #2 for KI#1 of TR 23.700-29.

# * * * * First change * * * *

## 6.4	Solution #4: Handling awareness of supported TAIs when RAN is onboard

### 6.4.1	Description

This is a candidate solution for Key Issue 1 - "Support of Regenerative-based satellite access ".

In contrast to Rel-17/Rel-18, where transparent payload was defined for satellite access and backhaul, the introduction of regenerative payload in Rel-19 involves deploying gNB on board, leading to impacts on the interface management between the RAN and the 5GC, especially considering paging. It is necessary to enable the AMF to avoid sending any paging requests to this RAN if the RAN is orbiting away.

This solution aims to repurpose the existing mechanism with minor enhancements, i.e. preconfigure a "TAI timetable" for each RAN node in the AMF/MME to facilitate more efficient RAN context management for the AMF to avoid sending paging requests to a RAN that is far away and how to enable the paging via satellites having ISL connections.

Figure 6.4.1-1: Example of mapping relationship between RAN ID, supporting TAI list and valid period in case of one satellite

Regarding the preconfigured "TAI timetable" per RAN node, the current mechanism supposes that RAN updates the broadcast TAC in real-time according to the ephemeris and confirms the broadcast TAI list is associated with the geographical area covered by the satellite beam. When RAN updates the TAI list, RAN will inform AMF of the updates. Even though the interactions between RAN and AMF are very frequent, in the context of R17/E18, the cost of interactions is limited because the interactions are via ground link (e.g. fibre, cable, etc.). However, in R19, such interactions will be over feeder link, whose bandwidth is scarce. Besides, the ephemeris data can be updated also very frequently in R17/R18. Therefore, a mechanism is needed to reduce the frequent interactions by:

-	Quantifying the broadcasting TAI list refreshing cycle to reduce the frequency of interactions. As shown in Figure 6.4.1-1, e.g. at T1, RAN broadcasts TAI 1-12, at T1+δ, RAN broadcasts TAI 3-13, at T2, RAN broadcasts TAI 6-18 and etc. RAN only updates the TAI list at the quantified time point. The period for updating the Supported TA List may be pre-configured (e.g. up to the operator's policy) or up to the implementation

-	Integrating valid period in the mapping relationship between RAN ID and supporting TAI list. Such mapping relationship can also be stored at AMF by pre-configuration/RAN provision, as RAN orbiting has a predictable trajectory, such mapping relationship information can be reused/valid regularly. The AMF/MME can calculate the TA(s) that the RAN node can serve based on ephemeris information and can update the TAC based on the reported TA list by the RAN node. In this case, the RAN nodes may not notify the AMF/MME of the supported TA List.

NOTE:	The mapping relationship for the RAN ID includes RANs connected via ISL (i.e. TAI and the valid period of the satellite connected with the ISL)

By preconfiguring such per RAN node TAI timetable, AMF/MME can also stop interactions with a RAN node that is not valid, e.g. stop sending paging requests via a RAN node that is not available.

In conclusion, this solution proposes enhancements to reduce the messages exchanged for TAI/CGI reports over N2/S1-AP interface .

### 6.4.2	Procedures

It is proposed that AMF/MME and RAN both are pre-configured with the mapping relationship among Global RAN Node ID, supported TA list, and valid period. In contrast to the existing mechanism outlined in TS38.413 [12] and TS36.413 [14], the novel addition is the inclusion of the "valid period" parameter to the TA list. Leveraging this timeframe information, the AMF/MME can assess the RAN node's availability, especially in scenarios involving RAN orbiting with a satellite, and cease interactions when the RAN node falls outside the valid period.

To enhance the accuracy of the TAI timetable, RAN and AMF/MME may employ existing NG/S1 interface management mechanisms to update the TAI timetable, as illustrated in clauses 6.4.2.1, 6.4.2.2, and 6.4.2.3

#### 6.4.2.1	TA management during the NG setup procedures

Figure 6.4.2.1-1: TA management during the NG setup procedure

The purpose of the NG Setup procedure is to exchange application level data needed for the RAN node and the AMF/MME to correctly interoperate on the NG/S1 interface This procedure can be reused to update the TAI timetable, e.g. when RAN detects the TAI timetable is different from the pre-configuration.

1.	The RAN node initiates the procedure by sending an NG/S1 SETUP REQUEST message including the appropriate data (e.g. updates of the mapping relationship if it is different from the pre-configuration) to the AMF/MME.

2.	The AMF/MME responds with an NG/S1 SETUP RESPONSE message including the appropriate data, as defined in clause 8.7 of TS 38.413 [12] and in clause 8.7 of TS 36.413 [14]. The AMF/MME will update the mapping relationship and the starting time of the valid period if needed.

#### 6.4.2.2	TA management during the RAN configuration update procedure

Figure 6.4.2.2-1: TA management during the RAN configuration update procedure

The purpose of the RAN Configuration Update procedure is to update application-level configuration data needed for the RAN node and the AMF/MME to interoperate correctly on the NG/S1 interface. This procedure can be reused to update the TAI timetable when RAN detects the TAI timetable is different from the pre-configuration after the NG/S1 interface is already set up.

- If the on-board RAN node will not be able to provide services to any TA in the TA list last provided to the AMF/MME, the on-board RAN node may initiate the RAN/ENB Configuration Update procedure to update the supported TA list.

- The AMF may update this NG-RAN's mapping relationship, valid period, supported TA List, and the corresponding list of supported slices for each TA and use them for subsequent registration area management of the UE based on received Supported TA List IE. The AMF/MME may take Tracking Area contained in the Supported TA List IE into account to optimise signalling towards this RAN node.

#### 6.4.2.3

#### TA management during the AMF/MME configuration update procedure

Figure 6.4.2.3-1: TA management during the AMF configuration update procedure

The purpose of the AMF/MME Configuration Update procedure is to update application level configuration data needed for the RAN node and AMF/MME to interoperate correctly on the NG/s1 interface. This procedure can be reused by AMF/MME to request the TAI timetable from RAN.

1.	AMF/MME sends CONFIGURATION UPDATE message by including:

a)	Request Supported TA List indicator.

2.	RAN sends CONFIGURATION ACKNOWLEDGE message by including:

a)	Supported TA List IE;

b)	valid period.

The AMF may update the mapping relationship and the starting time of the valid period.

### 6.4.3	Impacts to Services, Entities and Interfaces

AMF:

-	Preconfigure mapping relationship among Global RAN Node ID, supporting TAI list and valid period, including the mapping relationship of RANs connected via ISL.

-	Update mapping relationship, especially the valid period starting time-based on the message received from RAN over NG interface.

-	Request the TAI timetable when is necessary

RAN:

-	Preconfigure mapping relationship among Global RAN Node ID, supporting TAI list and valid period, including the mapping relationship of RANs connected vis ISL.

-	Send updates of mapping relationship to AMF to minimize the volume of signalling.

-	Support of reporting the supported TA list to AMF/MME when is requested

# * * * * End of changes * * * *