3GPP TSG-WG SA2 Meeting #162	S2-2404176

# Changsha, China, April 15th – 19th, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5361 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for EAS discovery procedure via (V-)EASDF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause T.4 of TS33.501, it defines to reuse the DNS over DTLS specified to protect the discovery message between UE and V-EASDF. In the case of HR with Session Breakout (HR-SBO) roaming scenarios, during the PDU session establishment or modification procedure, the V-SMF provides the V-EASDF security information via Nsmf_PDUSession_Create/ Nsmf_PDUSession_Update to H-SMF when the V-SMF determines to use a V-EASDF for EAS discovery, and the H-SMF provides the V-EASDF security information to UE via PCO if HR SBO is authorized and UE support DNS over TLS. However, how the SMF obtains the EASDF security information is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the SMF can obtain the EASDF security information via NF discovery procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SMF can not obtain the EASDF security information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2; 6.2.6.2; 7.2.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502 CR 4777 TS23.548 CR 0221 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2.2	SMF

The Session Management function (SMF) includes the following functionality. Some or all of the SMF functionalities may be supported in a single instance of a SMF:

-	Session Management e.g. Session Establishment, modify and release, including tunnel maintain between UPF and AN node.

-	UE IP address allocation & management (including optional Authorization). The UE IP address may be received from a UPF or from an external data network.

-	DHCPv4 (server and client) and DHCPv6 (server and client) functions.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The SMF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Selection and control of UP function, including controlling the UPF to proxy ARP or IPv6 Neighbour Discovery, or to forward all ARP/IPv6 Neighbour Solicitation traffic to the SMF, for Ethernet PDU Sessions.

-	Configures traffic steering at UPF to route traffic to proper destination.

-	5G VN group management, e.g. maintain the topology of the involved PSA UPFs, establish and release the N19 tunnels between PSA UPFs, configure traffic forwarding at UPF to apply local switching, N6-based forwarding or N19-based forwarding, manage traffic forwarding in the case that a SMF Set or multiple SMF Sets are serving a 5G VN.

-	Termination of interfaces towards Policy control functions.

-	Lawful intercept (for SM events and interface to LI System).

-	Support for charging.

-	Control and coordination of charging data collection at UPF.

-	Termination of SM parts of NAS messages.

-	Downlink Data Notification.

-	Initiator of AN specific SM information, sent via AMF over N2 to AN.

-	Determine SSC mode of a session.

-	Support for Control Plane CIoT 5GS Optimisation.

-	Support of header compression.

-	Act as I-SMF in deployments where I-SMF can be inserted, removed and relocated.

-	Provisioning of external parameters (Expected UE Behaviour parameters or Network Configuration parameters).

-	Support P-CSCF discovery for IMS services.

-	Act as V-SMF with following roaming functionalities:

-	Handle local enforcement to apply QoS SLAs (VPLMN).

-	Charging (VPLMN).

-	Lawful intercept (in VPLMN for SM events and interface to LI System).

-	Support for interaction with external DN for transport of signalling for PDU Session authentication/authorization by external DN.

-	Instructs UPF and NG-RAN to perform redundant transmission on N3/N9 interfaces.

-	Generation of the TSC Assistance Information based on the TSC Assistance Container received from the PCF.

-	Support for RAN feedback for BAT offset and adjusted periodicity as defined in clause 5.27.2.5.

NOTE:	Not all of the functionalities are required to be supported in an instance of a Network Slice.

In addition to the functionalities of the SMF described above, the SMF may include policy related functionalities as described in clause 6.2.2 of TS 23.503 [45].

In addition to the functionality of the SMF described above, the SMF may include the following functionality to support monitoring in roaming scenarios:

-	Normalization of reports according to roaming agreements between VPLMN and HPLMN; and

-	Generation of charging information for Monitoring Event Reports that are sent to the HPLMN.

The SMF may also include following functionalities to support Edge Computing enhancements (further defined in TS 23.548 [130]):

-	Selection of EASDF and obtain DNS security information of the EASDF and provision of its address to the UE as the DNS Server for the PDU session;

-	Usage of EASDF services as defined in TS 23.548 [130];

-	For supporting the Application Layer Architecture defined in TS 23.558 [134]: Provision and updates of ECS Address Configuration Information to the UE;

-	For supporting the HR-SBO as defined in clause 6.7 of TS 23.548 [130].

The SMF and SMF+ PGW-C may also include following functionalities to support Network Slice Admission Control:

-	Support of NSAC for maximum number of PDU sessions as defined in clauses 5.15.11.2, 5.15.11.3 and 5.15.11.5.

-	Support of NSAC for maximum number of UEs as defined in clauses 5.15.11.3 and 5.15.11.5.

The SMF may also include following functionalities:

-	Providing per-QoS flow Non-3GPP QoS assistance information to the UE (e.g. PEGC) and formulation of the CN PDB based on non-3GPP delay budget from UE (e.g. PEGC) as described in clause 5.44.3.4.

-	Support of PDU Set based handling as described in clause 5.37.5.

In addition to the functionalities of the SMF described above, the SMF may also include functionalities to support Network Slice Replacement as described in clause 5.15.19.

The SMF may also include functionalities to support indirect UPF event exposure service subscription on behalf of the consumer NF(s) as described in clause 4.15.4.5 of TS 23.502 [3].

# * * * * Second change * * * *

#### 6.2.6.2	NF profile

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

-	NF capacity information.

-	NF priority information.

NOTE 1:	This parameter is used for AMF selection, if applicable, as specified in clause 6.3.5. See clause 6.1.6.2.2 of TS 29.510 [58] for its detailed use.

-	NF Set ID.

-	NF Service Set ID of the NF service instance.

-	NF Specific Service authorization information.

-	if applicable, Names of supported services.

-	Endpoint Address(es) of instance(s) of each supported service.

-	Identification of stored data/information.

NOTE 2:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in clause 5.2.7.2.2 of TS 23.502 [3]. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g. DNN or DNN list, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Location information for the NF instance.

NOTE 3:	This information is operator specific. Examples of such information can be geographical location, data centre.

-	TAI(s).

-	NF load information.

-	Routing Indicator, Home Network Public Key identifier, for UDM and AUSF.

-	For UDM, AUSF and NSSAAF in the case of access to an SNPN using credentials owned by a Credentials Holder with AAA Server, identification of Credentials Holder (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used for access to an SNPN using credentials owned by a Credentials Holder, identification of Credentials Holder (i.e. the realm if Network Specific Identifier based SUPI is used or the MCC and MNC if IMSI based SUPI is used); see clause 5.30.2.1.

-	For AUSF and NSSAAF in the case of SNPN Onboarding using a DCS with AAA server, identification of DCS (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used as DCS in the case of SNPN Onboarding, identification of DCS (i.e. the realm if Network Specific Identifier based SUPI, or the MCC and MNC if IMSI based SUPI).

-	One or more GUAMI(s), in the case of AMF.

-	For the UPF, see clause 5.2.7.2.2 of TS 23.502 [3].

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of internal group identifiers, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	PCF Group ID, range(s) of SUPIs for PCF.

-	HSS Group ID, set(s) of IMPIs, set(s) of IMPU, set(s) of IMSIs, set(s) of PSIs, set(s) of MSISDN for HSS.

-	For NWDAF, the following information are supported:

-	Analytics ID(s) (possibly per service).

-	NWDAF Serving Area information (i.e. list of TAIs for which the NWDAF can provide services and/or data).

-	Supported Analytics Delay per Analytics ID (if available).

-	NF types of the NF data sources, NF Set IDs of the NF data sources, if available.

-	Analytics aggregation capability (if available).

-	Analytics metadata provisioning capability (if available).

-	ML model Filter information parameters include S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s).

-	ML Model Interoperability indicator (if available) per Analytics ID(s).

-	FL capability information per analytics ID including FL capability type (i.e. FL server and/or FL client, if available).

-	Time interval supporting FL (if available).

-	Accuracy checking capability for ML model accuracy monitoring or Analytics Accuracy Monitoring (if available).

-	Roaming exchange capability (if available).

NOTE 4:	The NWDAF's Serving Area information is common to all its supported Analytics IDs.

NOTE 5:	The Analytics IDs supported by the NWDAF may be associated with a Supported Analytics Delay i.e. the Analytics report can be generated with a time (including data collection delay and inference delay) in less than or equal to the Supported Analytics Delay.

NOTE 6:	The determination of Supported Analytics Delay, and how the NWDAF avoid updating its Supported Analytics Delay in NRF frequently is NWDAF implementation specific.

-	Event ID(s) supported by AFs, in the case of NEF.

-	Event Exposure service supported event ID(s) by UPF.

-	Application Identifier(s) supported by AFs, in the case of NEF.

-	Range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF, in the case of NEF.

NOTE 7:	This is applicable when NEF exposes AF information for analytics purpose as detailed in TS 23.288 [86].

NOTE 8:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 9:	The NRF may store a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s) and PCF Group ID and SUPI(s), to enable discovery of UDM, UDR, AUSF and PCF using SUPI, SUPI ranges as specified in clause 6.3 or interact with UDR to resolve the UDM Group ID/UDR Group ID/AUSF Group ID/PCF Group ID based on UE identity, e.g. SUPI (see clause 6.3.1 for details).

-	IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [58], Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, Range(s) of SUPIs or Range(s) of GPSIs or a BSF Group ID, in the case of BSF.

-	SCP Domain the NF belongs to.

-	DCCF Serving Area information, NF types of the data sources, NF Set IDs of the data sources, if available, in the case of DCCF.

-	Supported DNAI list, in the case of SMF.

-	For SNPN, capability to support SNPN Onboarding in the case of AMF and capability to support User Plane Remote Provisioning in the case of SMF.

-	IP address range, DNAI for UPF.

-	DNS security information, in the case of EASDF.

-	Additional V2X related NF profile parameters are defined in TS 23.287 [121].

-	Additional ProSe related NF profile parameters are defined in TS 23.304 [128].

-	Additional MBS related NF profile parameters are defined in TS 23.247 [129].

-	Additional UAS related NF profile parameters are defined in TS 23.256 [136].

-	Additional Ranging based services and Sidelink Positioning related NF profile parameters are defined in TS 23.586 [180].

-	For additional information in PCF profile, see clause 5.2.7.2.2 of TS 23.502 [3].

# * * * * Third change * * * *

### 7.2.25	EASDF Services

The following NF services are specified for EASDF.

Table 7.2.25-1: NF Services provided by EASDF

| Service Name | Description | Reference in TS 23.548 [130] |
| --- | --- | --- |
| Neasdf_DNSContext | This service enables the consumer to create, update and delete DNS context in EASDF, or subscribe to DNS message reporting from EASDF, or provide the DNS security information of EASDF. DNS contexts in EASDF include rules on how EASDF is to handle DNS messages. | 7.1.2 |
| Neasdf_BaselineDNSPattern | This service enables the consumer to create, update and delete BaselineDNSPattern in EASDF. | 7.1.3 |

# * * * * End of changes * * * *