3GPP TSG-WG SA2 Meeting #162	S2-2404179

Changsha, China, Apr 15 – 19, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, Sol#22: Update on the handling UL PDU Set QoS parameters

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: The paper proposes to clarify the notification from RAN on the UL PDU Set based handling support.

# 1. Introduction/Discussion

In last SA2#161 meeting, sol#22 is agreed for KI#1 about the UL PDU Set QoS handling considering e.g. UE’s indication via UAI.

Alternative 2 in the solution is similar with how current SMF accepts/rejects QoS Flow establishment/modification using PDU Session management procedure. However, in current solution RAN replies to SMF only when the PDU Set handling is impossible, which is not in line with traditional QoS Flow establishment/modification.

It’s proposed to further clarify alternative 2 that the NG-RAN notifies the SMF whether the UL PDU Set handling is accepted for a QoS flow by considering, e.g., UE capability for UL PDU Set handling, during PDU Session Establishment/Modification procedure.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change * * * *

## 6.22	Solution #22: < Handling of UL PDU Set QoS parameters >

### 6.22.1	Key Issue mapping

This solution addresses key issue #1.

### 6.22.2	Description

As approved in RAN R2-2312136, the PDU Set QoS handling depends on UE’s indication via UAI: when PDU Set identification is possible for a QoS flow, this is indicated to the gNB by the UE.

In the uplink, the UE needs to be able to identify PDU Sets and Data Bursts dynamically, including PSI. How this is done is left up to UE implementation but when possible for a QoS flow, this is indicated to the gNB via UE Assistance Information.

According to the existing procedure at RAN side, the NG RAN can understand whether UL PDU Set handling is possible or not for a QoS flow, but the CN is not aware of the UE supporting UL PDU Set handling or not. E.g. the CN may still send the UL PDU Set QoS parameters to the RAN even though the UL PDU Set handling is not possible. But the UL PDU Set handling is not applicable in this case and the CN has no idea about this situation.

If the AF has subscribed the QoS notification, it is beneficial and needed for CN to understand the UL PDU Set QoS is not fulfilled due to the UL PDU Set handling is not possible and notify the AF.

The principles of this solution are:

- Alternative1:

  - The NG RAN notifies to the SMF that UL PDU Set handling is possible for a QoS flow when the NG RAN understand the UL PDU Set handling is possible for a QoS flow.

  - The SMF may provide UL PDU Set QoS to the NG RAN upon knowing the UL PDU Set handling is possible for a QoS flow.

NOTE:   Before knowing that UL PDU Set handling is possible for a QoS flow, the SMF doesn’t need to send the UL PDU Set QoS parameter to the NG RAN.

Editor's note:	Alternative1 assumes the UE can indicate UAI to the RAN before RAN receives UL PDU Set QoS. It needs to evaluate whether it is realistic.

- Alternative2:

  - The SMF provides UL PDU Set QoS to the NG RAN without knowing the UL PDU Set handling is possible for a QoS flow.

- Upon reception of the UL PDU Set QoS, the NG RAN notifies the SMF whether the UL PDU Set handling is  accepted for a QoS flow by considering, e.g., UE capability for PDU Set handling.

NOTE:   If the AF has subscribed “QoS targets can no longer (or can again) be fulfilled”, the CN can notify the above to the AF.

### 6.22.3	Procedures

#### 6.22.3.1	Alternative1

Figure 6.X.3.1-1: Procedure

1.	When the NG RAN knows the UL PDU Set handling is possible for a QoS flow, the NG RAN notifies UL PDU Set handling is possible for a QoS flow to the SMF.

2.	The SMF may update the QoS profile for the QoS flow to provide UL PDU Set QoS to the NG RAN upon knowing the PDU Set handling is possible for a QoS flow.

NOTE:   Before knowing whether UL PDU Set handling is possible for a QoS flow, the SMF doesn’t need to send the UL PDU Set QoS parameter to the NG RAN.

#### 6.X.3.2	Alternative2

Figure 6.X.3.2-1: Procedure

1.	The SMF provides UL PDU Set QoS to the NG RAN for a QoS flow as legacy.

2.	Upon reception of the UL PDU Set QoS, the NG RAN notifies the SMF whether the UL PDU Set is  accepted for a QoS flow by considering, e.g., UE capability for PDU Set handling..

NOTE1:   If the AF has subscribed “QoS targets can no longer (or can again) be fulfilled”, the CN can notify above to the AF.

NOTE2:   How the NG RAN knows whether the UL PDU Set handling is possible or not for a QoS flow can be left to the RAN implementation.

### 6.22.4	Impacts on services, entities and interfaces

Alternative1

NG RAN：

-	When the NG RAN knows the UL PDU Set handling is possible for a QoS flow, the NG RAN notifies UL PDU Set handling is possible for a QoS flow to the SMF.

SMF:

-	Upon knowing the PDU Set identification is possible for a QoS flow, provide UL PDU Set QoS to the NG RAN.

NOTE:   Before knowing that UL PDU Set handling is possible for a QoS flow, the SMF doesn’t need to send the UL PDU Set QoS parameter to the NG RAN in this alternative.

No UE impacts.

Alternative2

NG RAN：

-	Upon reception of the UL PDU Set QoS, the NG RAN notifies the SMF whether the UL PDU Set is   accepted for a QoS flow by considering, e.g., UE capability for PDU Set handling..

CN:

-	Notifies to the above event to AF if the has subscribed the notification.

No UE impacts.

# * * * * End of changes * * * *