3GPP TSG-SA2 Meeting #162	S2-2404186

Changsha, China, April. 15-19, 2024 	(is revision of S2-2401960)

Source:	Lenovo

Title:	New solution for KI#1, Protocol stacks of backhaul link using a PDU session to support the N2/N3 interface for MWAB node

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract of the contribution: protocol stacks for wireless backhauling of N2/N3 interface for MWAB node

1	Discussion

During SA2#161 meeting, several solutions are introduced to address architecture enhancements to support MWAB operation, MWAB authorization handling and also the N2/N3 interface establishment procedure etc. However, the basic protocol stacks have not been captured. In this paper, the protocol stacks of backhaul link to support the N2/N3 interface of MWAB via IP connectivity provided by BH PDU sessions are introduced.

2	Proposal

* * * * Start of Change * * * *

## 6.x	Solution #X: Protocol Stacks of backhaul link to support the N2/N3 interface for MWAB node

### 6.x.1	Key Issue mapping

This solution is for Key Issue #1, which addresses the protocol stacks of the backhaul link using a PDU session to support the N2/N3 interfaces for a MWAB node.

### 6.x.2	Description

Based on the Non-Roaming MWAB architecture for 5GS in Figure 4.1-1, BH PDU sessions are established between MWAB-UE and UPF for MWAB-UE (i.e., MWAB-UE-UPF) within BH-5GC to transfer N2/N3 signaling/data, where N2 interface is between MWAB-gNB and AMF within 5GC serving UE and N3 interface is between MWAB-gNB and UPF for UE (i.e., UE-UPF) within 5GC serving UE. In this solution, the protocol stacks for wireless backhauling of N2/N3 interface of MWAB via IP connectivity provided by PDU sessions are introduced.

### 6.x.3   Procedures

#### 6.x.3.1   Protocol Stacks of backhaul link to support the N2 interface for MWAB node

Figure 6.x.3.1-1 illustrates Protocol Stacks of backhaul link to support the N2 interface for MWAB node. MWAB-UE connects to BH gNB. A PDU session between MWAB-UE and MWAB-UE-UPF is established as a backhual link to support the overlayed N2 interface. The N2 interface terminates at MWAB-gNB and AMF respectively, which includes NGAP and SCTP/IP layer protocols between them.

Figure 6.x.3.1-1: Protocol Stacks of backhaul link to support the N2 interface for MWAB node

#### 6.x.3.2   Protocol Stacks backhaul link to support the N3 interface for MWAB node

Figure 6.x.3.2-1 illustrates Protocol Stacks backhaul link to support the N3 interface for MWAB node. A PDU session between MWAB-UE and MWAB-UE-UPF is established as a backhual link to support the overlayed N3 interface. The N3 interface terminates at MWAB-gNB and UE-UPF respectively, which includes GTP-U and UDP/IP layer protocols between them. UE connects to MWAB-gNB. A PDU session between UE and UE-UPF is realized by 1) the Uu DRB between UE and MWAB-gNB, and 2)the NG-U between MWAB-gNB and UE-UPF on top of the backhauling PDU session between MWAB-UE and MWAB-UE-UPF.

Figure 6.x.3.2-1: Protocol Stacks of backhaul link to support the N3 interface for MWAB node

### 6.x.4	Impacts on services, entities and interfaces

The impacts will be captured in the related procedures of establishing the backhaul links to support N2/N3 interface.

* * * * End of Change * * * *