SA WG2 Meeting #162	S2-2404264

April 15 – 19, 2024, Changsha, China

Source:	Ericsson

Title:	KI#2.2: Evaluation

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes evaluation for KI#2.2.

# Discussion

All solutions proposed for KI#2.2 are based on the principle that the UE registers and triggers MA PDU Session Establishment in 3GPP access. The solutions differ however in how the UE communicates with UPF to carry user-plane data.

This paper proposes an evaluation and comparison of the solutions for KI#2.2.

# Proposal

It is proposed to update TR 23.700-54 as follows:

**** First Change ****

## 7.2	Overall Evaluation for ATSSS_Ph4

Editor's note:	This clause will provide evaluation of different solutions for ATSSS_Ph4.

### 7.2.X 	Evaluation for KI#2.2

The solutions proposed for KI#2.2 are summarized on Table 7.2.X-1 (AMF impacts to support capability negotiation and SMF selection has not been included for simplicity since it is similar in all solutions).

Table 7.2.X-1: Comparison of solutions for KI#2.2

|  | Solution # |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2.2 | 2.6 | 2.7 | 2.8 |
| Simplification | Eliminates NAS and N2 via non-3GPP. | Eliminates NAS and N2 via non-3GPP. | Eliminates NAS and N2 via non-3GPP. Eliminates IKEv2 / IPSec via non-3GPP | Eliminates NAS and N2 via non-3GPP. Eliminates IKEv2 / IPSec via non-3GPP |
| 5GC impacts | SMF, UPF, (ePDG). New interfaces (re-using EPC interfaces for ePDG) | SMF, UPF, AUSF. New interface SMF-AUSF. PFCP impacts to carry EAP payload | SMF, UPF. | SMF, UPF. |
| UE-UPF interface protocol | IKEv2/IPSec + ATSSS protocols (MPTCP, MPQUIC) | IKEv2/IPSec + ATSSS protocols (MPTCP, MPQUIC) | ATSSS protocols (MPTCP, MPQUIC) | ATSSS protocol (MPQUIC) |
| Registration | 3GPP access only | 3GPP access only | 3GPP access only | 3GPP access only |
| New information provisioned to UE via 3GPP access | ePDG IP addresses + | IKEv2/IPSec server IP addresses | MPTCP, MPQUIC proxy addresses | MPQUIC proxy addresses |
| Supported Steering Functionalities | ATSSS-LL, MPTCP, MPQUIC | ATSSS-LL, MPTCP, MPQUIC | MPTCP, MPQUIC | MPQUIC |
| Double encryption over non-3GPP | Yes for MPQUIC. No for ATSSS-LL, MPTCP. | Yes for MPQUIC. No for ATSSS-LL, MPTCP. | No | No |

Based on the comparison in Table 7.2.X-1, Solutions 2.7 and 2.8 achieve the biggest simplification, in particular on the UE/modem but also in the network, and has the smallest impacts to 5GC, at the expense of not supporting ATSSS-LL. Solutions 2.2 and 2.6 achieve some simplification (avoiding NAS via non-3GPP access) but at the same time introduce new requirements in UPF and also new interfaces. The existing rel-18 solution for ATSSS with ePDG can fulfil similar functionality as Solutions 2.2 and 2.6.

**** End of Changes ****