3GPP TSG SA WG2 Meeting #162	S2-2404484

Changsha, China, April 15 – 19, 2024	(was S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4793 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to HR-SBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.548 table 6.2.3.4-1 NOTE 2 defines The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. For HR-SBO roaming scenario, External Group Identifier and Internal Group Identifier, cannot be used by AF in VPLMN. The Internal Group Identifier should not be exposure to external AF and above 2nd sentence for HR-SBO roaming scenario with V-NEF as service consumer of Nnef_UEId_Get operation while External/Internal Group Identifier cannot be used in such scenario, means no definition to include External Group Id as input and Internal Group Id as output in Nnef_UEId API. While clause 5.2.6.27.2 Nnef_UEId_Get operation including External Group Id and Internal Group Id which is NOT aligned with definition in TS 23.548, hence needs to be directly removed. And clause 4.3.6.5 added optionally Private UE IP address, HPLMN DNN and S-NSSAI for HR-SBO Session in response of Nnef_UEId_Get operation, while not present in clause 5.2.6.27.2 Nnef_UEId_Get operation description. TS 23.548 clause 6.5.2.6.2 ECS Address Configuration Information Provision from AF via NEF in VPLMN defines "The AF provides non-PDU Session specific ECS Address Configuration Information via NEF in VPLMN to 5GC is defined in this clause.", and there is no DNN, S-NSSAI, External Group Id defined in clause 5.2.6.37.2 Nnef_ECSAddress_Create service operation, while 5.2.6.37.4 Nnef_ECSAddress_Delete service operation including DNN, S-NSSAI and External Group Id which is not aligned with above clauses, hence need to be removed. AF Transaction Id needs to be added in clause 5.2.6.37.2 and 5.2.6.37.4. Clause 5.2.6.37.1 defines "This procedure is defined only for the support of HR-SBO, i.e. where the ECS Address is not associated with the subscription of an user of the PLMN operating the V-NEF and V-SMF. ", and upon HR-SBO is HR PDU Session actually established by H-SMF with the DNN and S-NSSAI belong to HPLMN, no DNN and S-NSSAI has be provisioned in the Nnef_ECSAddress_Creat service operation, hence also needs to correct clause 5.2.6.37.5 to remove the not matched DNN and S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.2.6.27.2remove the not applied External Group Identifier(s) and Internal Group Identifier(s), add Private UE IP address, HPLMN DNN and S-NSSAI for HR-SBO Session as Outputs, Optional. Clause 5.2.6.37.2add AF Transaction Id in input. Clause 5.2.6.37.4add AF transaction Id in input and remove the not appliced parameters. Clause 5.2.6.37.5remove the not matched DNN, S-NSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear and Not aligned definitions on HR-SBO, which cannot be correctly implemented in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.27.2, 5.2.6.37.2, 5.2.6.37.4, 5.2.6.37.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 5.2.6.27.2	Nnef_UEId_Get operation

Service operation name: Nnef_UEId_Get

Description: Get the UE identifier.

Inputs, Required: GPSI or UE address (i.e. IPv4/IPv6 address or MAC address).

Inputs, Optional: DNN, S-NSSAI, Port number (e.g. TCP or UDP port), IP domain, Application port ID, MTC Provider Information, AF Identifier.

Outputs, Required: Result, GPSI either as an AF specific UE Identifier represented in the form of an External Identifier, or in the form of a MSISDN or SUPI.

NOTE 1:	SUPI can only be exposed to roaming partners.

NOTE 2:	The AF specific UE Identifier in GPSI form of MSISDN can only be exposed to a trusted AF when allowed and authorized by the operator as described in clause 4.15.10A.

Outputs, Optional: Private UE IP address, HPLMN DNN and S-NSSAI for HR-SBO Session.

# *** 2nd Change ***

##### 5.2.6.37.2	Nnef_ECSAddress_Create service operation

Service operation name: Nnef_ECSAddress_Create

Description: The NF consumer requests to create ECS Address Configuration Information.

Inputs, Required: ECS Address Configuration Information, AF Transaction Id.

Inputs, Optional: AF-service-identifier.

Outputs, Required: Result Indication.

Outputs, Optional: None.

# *** 3rd Change ***

##### 5.2.6.37.4	Nnef_ECSAddress_Delete service operation

Service operation name: Nnef_ECSAddress_Delete

Description: The NF consumer requests to delete ECS Address Configuration Information.

Inputs, Required: AF Transaction Id

The AF Transaction Id identifies the NF Service Consumer request for ECS Address Configuration Information to be deleted.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

# *** 4th Change ***

##### 5.2.6.37.5	Nnef_ECSAddress_Subscribe service operation

Service operation name: Nnef_ECSAddress_Subscribe

Description: Provided by the V-NEF for NF consumers to explicitly subscribe the notification of changes of ECS Address Configuration Information.

Inputs, Required: Event ID, Notification Target Address (+Notification Correlation ID).

Inputs, Optional: None.

Outputs, Required: Result Indication. When the subscription is accepted: Subscription Correlation ID (reference of the subscription).

Outputs, Optional: None.

*** End of Changes ***