3GPP SA WG2 Meeting #162	S2-2404494

Changsha, China, April 15-19, 2024														(was S2-2402262)

Source:	Ericsson

Title:	KI #2, New Solution: Device ID(s) for AIoT Devices

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: The contribution discusses and proposes a new solution for Ambient IoT device IDs.

1. Introduction

Ambient IoT devices are IoT devices powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor). It can have, e.g., lower complexity, smaller size, reduced capabilities and lower power consumption than previously defined 3GPP IoT devices. The data rate of Ambient IoT devices is usually low.

In KI#2 of TR 23.700-13 v0.2.0, the study objectives include:

-	Study how to identify Ambient IoT Device or group of devices and how to format the identifier.

This pCR proposes a solution to study the aspects above.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v0.2.0:

* * * Start of Change * * * *

## 66.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #1 |  | X |  |
| #2 |  | X |  |
| #3 | X | X | X |
| #4 | X | X | X |
| #5 |  | X | X |
| #6 | X | X | X |
| #7 | X | X |  |
| #8 | X | X |  |
| #9 | X | X | X |
| #10 | X | X |  |
| #11 |  |  | X |
| #12 | X | X | X |
| #X |  | X |  |

* * * Next of Change (ALL TEXTS ARE NEW) * * * *

## 6.X	Solution #X: Device ID(s) for AIoT Devices

### 6.X.1	Description

This solution addresses device ID aspect of Key Issue #2.

In 5GS, there are the following identifiers which are described in TS 23.501 [4]:

- Subscription Permanent Identifier (SUPI)

- Subscription Concealed Identifier (SUCI)

- Permanent Equipment Identifier (PEI)

- 5G Globally Unique Temporary Identifier (5G-GUTI)

- Generic Public Subscription Identifier (GPSI)

Internal-Group Identifiers and External-Group Identifiers are used to identify a group of UEs.

The different identifiers in 5GS are used for different purposes. Depending on the use cases and required functionality for supporting AIoT, different AIoT device IDs should be introduced.

To be able to perform authentication and authorization procedures in 5GS, it is necessary to introduce SUPI like and SUCI like IDs for AIoT devices. The SUCI like AIoT device ID is a concealed format of SUPI like AIoT device ID, which is used to transmit the device ID from AIoT devices towards network, similar as SUCI in registration request.

The SUPI can be a network-specific identifier containing the NAI based user identification. The SUPI like AIoT device ID can take this option and have the form username@realm. The realm part of the NAI includes MCC, MNC, and optional NID.

The SUCI like AIoT device ID should also take the form of a NAI as the SUCI today. And using the Routing Indicator assigned by the home network operator to route to the proper AUSF and UDM instances capable of handling authentication.

Editor's note:	Further content and details of the SUPI/SUCI like AIoT device ID is FFS, including whether one or multiple IEs are used.

For 5G-GUTI like AIoT device ID, similar as 5G-GUTI, is allocated by the CN after authentication and authorization. The 5G-GUTI like AIoT device ID is passed from CN towards an AIoT device, and the AIoT device stores it and then use it in the following communication towards network to identify the authenticated AIoT device.

Depending on AIoT device capabilities, the 5G-GUTI like AIoT device ID can be used.

Editor's note:	Further content and details of the 5G-GUTI like AIoT device ID, including whether one or muliple IEs are used.

The PEI is an identifier of the ME and is used e.g. to check whether the ME been stolen etc.

Editor's note:	Send LS to SA1 to understand whether PEI like AIoT device ID is needed or not.

To be able to communicate with external AF, a GPSI like AIoT device ID needs to be introduced. It is the NEF which perform the mapping between the SUPI like AIoT device ID and GPSI like AIoT device ID. The AF utilizes the GPSI like AIoT Device ID in requests towards the CN, and the NEF maps it to the SUPI like AIoT device ID, which is used inside CN. And when the CN responds or notifies the AF, the NEF maps the SUPI like AIoT device ID back to GPSI like AIoT device ID.

In RFID, RFID tags use EPC ID to communicate with RFID readers. It may make sense to introduce the EPC in GPSI like AIoT Device ID, to minimize the integration with RFID applications.

The GPSI can be defined as an External Identifier and have the form "<Local Identifier>@<Domain Identifier>". It could be an option to introduce EPC ID in the <Local Identifier>, while the <Domain Identifier> can be the domain name of the AIoT service provider.

Editor's note:	Further content and details of the GPSI like AIoT device ID, including whether one or muliple IEs are used and whether GPSI can be used as is.

Regarding the group of AIoT devices, Internal Group Identfier and External Group Identifier can be reused. And the NEF can perform the mapping between the Internal Group Identifiers and External Group Identifiers.

Editor's note:	It is FFS whether the differentiation of Internal Group Identifier and External Group Identifier is needed or not.

### 6.X.2	Procedures

The device IDs for AIoT devices are used in an equivalent way as current ones for UEs. No new procedures introduced due to AIoT device IDs. They are used in the inventory, command, (and also other) procedures for AIoT devices:

-	SUCI like device ID is sent from AIoT devices to network to identify the device when unauthenticated.

-	SUPI like device ID is used in 5GC to uniquely identify the device.

-	5G-GUTI like device ID is allocated by the core network and sent back to the device. It is used to page the device and identify the device if allocated.

-	PEI like device ID: as an identifier of ME, it is to be checked whether it is needed.

-	GPSI like device ID is used externally between AF and 5GC.

-	Interneal Group Identifer is used to identify a group of AIoT devices internally.

-	External Group Identifier is used to identify a group of AIoT devices in AF.

### 6.X.3	Impacts on services, entities and interfaces

Editor's note:	The services, entities and interfaces are FFS.

* * * End of Change * * * *