3GPP TSG-SA WG2 Meeting #S2-162	S2-2404620

# Changsha, China, April 15  –  19, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0367 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transfer of MBS assistance information during UE mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Per RAN3 LS(S2-2403894/R3-241145) it is agreed that gNB to signal to SMF whether it supports the Multicast reception in RRC inactive state in order to have SMF send the PDU session modification only for UEs moving from a RAN node not supporting this feature to a target RAN node. Hence, PATH Switch ACK is used for SMF providing available MBS Assistance Information to target NG-RAN for Xn-based handover or Connection Resume from an MBS supporting source RAN node towards an MBS supporting target RAN node. And for the case of Xn-based handover or Connection Resume from an non-MBS supporting source RAN node towards an MBS supporting target RAN node, a PDU session modification is used after the completion of the handover or Connection Resume. So, in general the existing procedures of Xn/N2 HO and Connection resume are used with having MBS assistance information included in MBS session related information from SMF Hence, this is already covered by the following bullet because it is applied to the clauses 7.2.3.2(Xn based handover from MBS supporting NG-RAN node), 7.2.3.3(N2 based handover from MBS supporting NG-RAN node), 7.2.3.4(Xn/N2 based handover from non-MBS supporting NG-RAN node), 7.2.3.6(Xn/N2 based handover for inactive MBS session) and 7.2.3.7(Connection Resume in RRC Inactive procedure). - If there is signalling of MBS session related information from SMF to target RAN node, the SMF includes MBS Assistance Information in the N2 SM information. Hence, the following 3 bullets are not required any more. - For N2-based handover, when the SMF provides the MBS session related information as part of the associated PDU Session context data as specified in the existing procedures, the SMF also includes the MBS Assistance Information in the N2 SM information. - For Xn-based handover or Connection Resume from a non-MBS supporting source RAN node towards an MBS supporting target RAN node, the SMF provides MBS session related information as part of the associated PDU Session context data as specified in the existing procedures and the SMF also includes the MBS Assistance Information in the N2 SM information. - For Xn-based handover or Connection Resume from an MBS supporting source RAN node towards an MBS supporting target RAN node, depending on configuration, the SMF may provide the MBS Assistance Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The duplicated 3 bullets are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with RAN specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.2.3.8	Mobility procedures to enable delivery of multicast MBS session data to UEs in RRC_INACTIVE state

7.2.3.8.1	General

The procedures in clauses 7.2.3.2, 7.2.3.3, 7.2.3.4, 7.2.3.6 and 7.2.3.7 apply with the following additions:

-	If the MBS assistance information is available at the SMF for an MBS session that the UE joined,

-	If there is signalling of MBS session related information as part of the associated PDU Session context data as specified in the existing procedures from SMF to target RAN node, the SMF includes MBS Assistance Information in the N2 SM information.

-	For Xn-based handover or Connection Resume, the MBS session information transferred from source NG-RAN towards target NG-RAN also include MBS assistance information for the MBS session if such information is available at the source RAN node as specified in TS 38.300 [9].

NOTE 1:	In deployments where not all the MBS supporting NG RAN nodes support delivery of multicast MBS session data to UEs in RRC_INACTIVE state, a source NG-RAN node not supporting delivery of multicast MBS session data to UEs in RRC_INACTIVE state will not provide MBS assistance information for the MBS session to the target NG-RAN node.

NOTE 2:	It is assumed that a RAN node not supporting delivery of multicast MBS session data to UEs in RRC_INACTIVE state ignores the MBS assistance information for the MBS session.

For a UE that is receiving multicast MBS data in CM-CONNECTED with RRC_INACTIVE state, the scenarios for UE mobility are as follows:

-	UE moves to new cell within the RAN Notification Area (RNA).

-	UE moves outside the current RAN Notification Area but within the current Registration Area (RA).

-	UE moves out of the current Registration Area.

# * * * * End of changes * * * *