3GPP TSG-WG SA2 Meeting #162	S2-2404640

# Changsha, China, April 15th – 19th, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4797 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and alignments for member UE selection assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current general description and the basic procedure of the Assistance for Member UE selection service are missing some details. Furthermore, the terminology is not fully aligned and the clause headings are inconsistent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifcation of details, corrections and terminology alignment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear statements and inconsistent terminology. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.0, 4.15.13.1, 4.15.13.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.15.13	Assistance for Member UE selection

#### 4.15.13.0	General

Assistance for Member UE selection allows the AF to request the 5GC to filter a list of AF provided target member UEs according to AF provided criteria (Member UE filtering criteria) and to provide one or more list(s) of candidate UEs (and additional information) as response to the AF.

With the initial subscription request, the AF provides a list of target member UEs in the form of a list of GPSIs or a list of UE IP addresses and at least one Member UE filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve from 5GC NFs the information for each UE in the list of target member UEs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria requested by the AF. The NEF finally provides the one or more list(s) of candidate UEs and possibly additional information to the AF together with a Subscription Correlation ID.

The AF may subsequently update the subscription by sending further subscription requests to the NEF with the Subscription Correlation ID but without the list of target member UEs. The AF may update the parameters (e.g. expiry time, time window for selecting the candidate UEs) for previously provided Member UE filtering criteria. The AF may also update the Member UE filtering criteria by adding new Member UE filtering criteria or removing part of the subscribed Member UE filtering criteria.

The Member UE selection assistance capability can be used to assist the AF to select the list of member UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how the AF leverages the 5GC network exposure for Member UE selection is described in (informative) Annex I.

AF may also delete the previous subscription for Member UE selection assistance as described in clause 4.15.13.7.

# * * * * Next change * * * *

#### 4.15.13.1	Member UE selection assistance subscribe procedure

This clause describes the Member UE selection assistance subscribe and update procedure that is generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: Member UE selection assistance subscribe and update procedure

1.	AF subscribes the Member UE selection assistance by sending a Nnef_MemberUESelectionAssistance_subscribe request including a list of target member UEs, one or more member UE filtering criteria as listed in Table 4.15.13.2-1 and optionally, time window(s). Subsequently, the AF may only update the Member UE filtering criteria of the subscription as described in clause 4.15.13.0 by invoking Nnef_MemberUESelectionAssistance_subscribe and providing a Subscription Correlation ID, i.e. the AF does not provide the list of target member UEs again.

NOTE:	The time window(s) for selecting the candidate UE(s) is used by the NEF when subscribing/requesting to NWDAF. The NEF maps the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations.

2a.	[CONDITIONAL] If the AF request does not contain a Subscription Correlation ID, the NEF verifies the authorization of the AF request and identifies which information needs to be collected for each UE in the list of target member UEs and executes the corresponding service operations based on the Member UE filtering criteria provided by the AF, e.g. events, analytics ID(s), notifications, etc.

2b.	[CONDITIONAL] If the AF request contains a Subscription Correlation ID, the NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF updates the Member UE filtering criteria and/or their parameters (as described in clause 4.15.13.0) for the list of target member UEs received in initial subscription (i.e. no Subscription Correlation ID in the subscription request).

3.	NEF interacts with different 5GC network functions to collect the required information for each UE in the list of target member UEs. The set of interactions between the NEF and the 5GC NFs depend on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, the NEF consolidates all the information to derive the list(s) of candidate UEs which fulfil the Member UE filtering criteria in the AF request. The NEF may derive recommended time window(s), considering the validity period(s) of the analytics used for Member UE selection criteria. The recommended time window(s) may be a subset of the time window(s) received from the AF. In different recommended time windows, the list of candidate UEs which fulfil the Member UE filtering criteria may be different.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UEs and possibly additional information. See clause 5.2.6.32.4 for details.

# * * * * Next change * * * *

#### 4.15.13.7	Member UE selection assistance unsubscribe procedure

This clause describes the procedure to delete the subscription for Member UE selection assistance in NEF.

Figure 4.15.13.7-1: Member UE selection assistance unsubscribe procedure

1.	AF requests to delete the previous subscription for Member UE selection assistance by sending Nnef_MemberUESelectionAssistance_unsubscribe request including the Subscription Correlation ID.

2.	NEF interacts with different 5GC network functions to delete the subscription(s) generated for each UE in the list of target member UEs. The set of interactions between NEF and 5GC NFs are dependent on the Member UE filtering criteria that AF had provided in clause 4.15.13.1.

3.	NEF sends a Nnef_MemberUESelectionAssistance_unsubscribe response to the AF with the operation execution result indication.

# * * * * End of changes * * * *