3GPP TSG-SA2 Meeting #162	S2-2404684

# 15 - 19 April, 2024, Changsha, China												revision of S2-xxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4800 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to assistance for member UE selection functionality and service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Changes in 4.15.13.0 intend to clarify that AF does not provide a new list of target member UEs when either the list of filtering criteria changes or the value of the existing filtering criteria change. Other changes are editorial. In addition, regarding the change in 5.2.6.32.4: As specified in clause 4.15.13.4 of TS 23.502, in step 11-12, ‘If step 10 identifies any UE which is moving out of the FL coverage area, the NEF may further notify the AF of the given UE which is moving out the coverage’. Based on the description, the NEF should inform AF of the UEs that cannot fulfil UE location filtering criteria or the UEs that are not recommended anymore for FL. However, using the current Nnef_MemberUESelectionAssistance_Notify, the NEF can only include one or more list(s) of candidate UEs to the AF. It is possible for the NEF to indicate the UEs that not recommended for FL to the AF. Furthermore, there are potential large account of UEs will participate the FL. In the case if only a small amount of UEs (e.g.2 out of 100) cannot fulfil the UE location filtering criteria, the NEF must indicate 98 candidates UEs to the AF, which waste resources significantly. Similar scenarios may also happen during the member UE update, e.g. only a small amount of UEs cannot fulfil the updated filtering criteria. In order to assist the AF to determine the UEs to participate FL in a resource efficient way, the Nnef_MemberUESelectionAssistance_Notify should be allowed indicate list(s) of UEs that are recommended. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections in General description Introduce the one or more list(s) of UEs that are not recommended into Nnef_MemberUESelectionAssistance_Notify. The NEF will either send the candidate UEs or the UEs that are not recommended to the AF, based on NEF implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear functionality description. Also, not clear for the Nnef_MemberUESelectionAssistance_Notify consumer whether the list of UEs are candidate UEs or the UEs that are not recommended for FL. The AF cannot choose the right UEs for FL. Member UE selection functionality cannot operate in resource efficient manner. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.0, 5.2.6.32.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

#### 4.15.13.0	General

With the initial subscription request, the AF provides a list of target UEs in the form of a list of GPSIs or a list of UE IP addresses and at least one Member UE filtering criterion as part of the service operation input parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve from 5GC NFs the information for the UEs in the list of target member UEs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria provided by the AF.

AF may subsequently update the Member UE filtering criteria values and notify the NEF with Subscription Correlation ID. AF may also request to update the Member UE filtering criteria by adding new Member UE filtering criteria or removing some of the Member UE filtering criteria. In both cases, AF does not include the list of target member UEs. AF may also request to update the parameters (e.g. expiry time, time window for selecting the candidate UEs) for the subscribed Member UE filtering criteria with Subscription Correlation ID.

The Member UE selection assistance capability can be used to assist the AF to select the list of member UEs to support application service (e.g. FL operation) as described in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how the AF leverages the 5GC network exposure for Member UE selection is described in (informative) Annex I.

AF may also unsubscribe the previous subscription for Member UE selection assistance information as described in clause 4.15.13.7.

* * *Next Change * * *

##### 5.2.6.32.4	Nnef_MemberUESelectionAssistance_Notify service operation

Service operation name: Nnef_MemberUESelectionAssistance_Notify

Description: NEF reports the Member UE selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information.

Inputs, Conditional Required:

At least one of the following inputs is required:

-	One or more list(s) of candidate UEs in the form of a list of GPSIs or a list of UE IP addresses.

-	One or more list(s) of UEs that are not recommended in the form of a list of GPSIs or a list of UE IP addresses

Inputs, Optional: Recommended time window for performing the application operation per list of candidate UEs as described in clause 4.15.13.1, specific values of the parameters that NEF gathered for the Member UE filtering criteria per candidate UE, a number for each filtering criterion that indicates the amount of UEs in the initial list which do not meet the criterion.

NOTE:	This number can be an indication for AF to revise one or multiple of the corresponding filtering criteria.

Outputs, Required: Operation execution result indication.

* * *End of Changes * * *