3GPP TSG-WG SA WG2 Meeting #162	S2-2404688

Changsha, China, 15th –19th April, 2024                        (revision of S2-240xxxx)

Source:	CATT

Title:	Update sol#38 for exposure of additional events for the S&F operation

Document for:	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This paper proposes an update for sol#38 for FS_5GSAT_ARCH_Ph3 KI#2.

1	Discussion

This paper proposes an update for sol#38 to expose more monitoring events for the S&F operation.

2	Proposal

It is proposed to include the following changes in TR 23.700-29 V0.4.0.

* * * * Start of Changes * * * *

6.38	Solution #38: Exposure of Registration in S&F Mode

6.38.1	Description

This solution addresses following aspect of KI #2:

-	What enhancements on the related UE and network procedures are needed to support S&F Satellite Operation?

When a UE registers to the network in S&F mode and is available to send/receive data on specific occasions only (e.g. when satellites fly-over and later connect to ground station), it may be desirable to expose information below to an application function. E.g.  the AF may send a message to the UE and expect UE will reply in a certain time, monitored by a guard timer. AF may need to re-adjust all such applicable timer if the UE is in the S&F mode, thus such information will help an AF adjust its logic accordingly as described.

1)	UE registered in S&F Mode: If the UE is registered in S&F mode, the AF may want to adjust the frequency, size of data, message acknowledgement handling, ack timers etc.

2)	When the Feeder-Link is available: This will help AF schedule delivery as and when Feeder-Link is available. Additionally feeder link availability duration. This can help AF to know till when a data packet can be delivered to the network in the current pass.

3)	Expected delivery time: The delivery of data will require following events:

a)	Feeder-Link Availability to deliver data to EPC/5GC and further to the Satellite

b)	Service-Link Availability to deliver data to UE and receive acknowledgement

c)	Feeder-Link availability to deliver acknowledgement to EPC/5GC and further back to the AF

It may be useful to provide expected delivery time, potential round trip(if UE sends an ack on reception of this data) to AF so that retry etc. can be scheduled accordingly

4)	UE reachability in S&F Mode: If the UE has registered in S&F mode, and UE can be reachable (e.g., CM-CONNECTED state), the AF may know when to deliver the DL data to the UE. If the UE cannot be reachable in S&F mode (maybe the service-link is unavailable), the AF may not deliver the DL data and adjust the DL data delivery policies (e.g., pending the DL data, extend the acknowledgement timers, etc.).

5)	S&F data storage quota: If the S&F data storage quota usage status per Satellite can be monitored and exposed to the AF, the AF may re-provisioning the maximum S&F data storage quota per Satellite and/or adjust the DL data delivery policies when the allocated S&F data storage quota is closely used up (e.g., lower the DL data rate, pending the DL data).

Accordingly, it is proposed to define new monitoring events and expose those via NEF/SCEF to AF.

* * * * Next Changes * * * *

6.38.2	Procedures

The procedure of clause 4.15.3.2 of TS 23.502[3] can be reused to depict the exposure procedure for the following monitoring events. The main enhancement is the satellite ID may be one of the Event Filter Information.

It is proposed to add following events to be exposed by AMF/MME, UDM/HSS via NEF/SCEF to AF:

6.38.2-1 List of monitoring events for S&F operations

| Event | Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Registration in S&F Mode | Detected when the UE registers in S&F Mode. The event report may include an indication of S&F Mode. | Ground-AMF/Ground-MME/UDM/HSS |
| Feeder Link Available | Network detects that Feeder Link is available for AF to send data to a UE. The event report may include duration for which the feeder link is available. Alternatively, the AF may query the network to determine when/frequency the feeder-link will be available next. | Ground-AMF/Ground-MME |
| Expected Delivery/Delay Time | The expected delay to deliver the data to the UE. This information may be informed to the AF based on query, or when data sent by AF is cached into EPC/5GC, or along with the "Feeder Link Available" event. The potential round trip time if UE acknowledges to the sent data packet | Ground-AMF/Ground-MME |
| UE reachability in S&F Mode | Detected UE reachability status (e.g., CM-CONNECTED state) when it registers in S&F Mode. | Ground-AMF/Ground-MME |
| S&F data storage quota (NOTE 1). | Network detects the S&F data storage quota usage status per satellite. | NF(s) used for store data for the S&F operation |
| NOTE 1: It only applies for one Satellite scenario. |  |  |

Editor's note:	Whether any existing monitoring events can be used or enhanced to achieve the events described in this solution is FFS.

6.38.3	Impacts on services, entities and interfaces

AF/UDM/AMF/HSS/MME/SCEF/NEF: Support of new monitoring events.

* * * * End of Changes * * * *