3GPP TSG-SA WG2 Meeting #162	S2-2404721

15 – 19 April 2024, Changsha, China											(revision of S2-2402790)

Source:	Xiaomi

Title:	KI#2: Solution #21 Update to resolve EN

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: The contribution proposes solution#21 update to Key Issue #2: support of store and forward satellite operation in EPS.

1. Introduction

This contribution proposes solution#21 update to Key Issue #2: Support of store and forward satellite operation in EPS.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-29-040.

* * * Start of change * * * *

6.21	Solution #21:	Support of store and forward satellite operation in EPS with eNB on board

6.21.1	Description

This solution resolves KI#2 about supporting store and forward satellite operation for delay-tolerant/non-real-time services. The solution is based on architecture with eNB on board and all EPC functionality are deployed on the ground, as shown in the following figure:

Figure 6.21.1-1: Regenerative based satellite architecture with eNB on board

The following basic principles and assumptions are applied to the solution:

-	The full eNB is on board satellite, only one satellite serves the UE access, all EPC functionalites are deployed on the ground.

-	The UE subscription information contains store and forward information (e.g. data storage quota, data forward priority, etc), if the UE subscribed S&F capability.

-

-	When eNB detects the feeder link is not available, it performs uplink data/signalling storage and forwards the stored uplink data/singalling if feeder link becomes available.

-	When MME detects the feeder link is not available, it performs downlink data/signalling storage or it indicates S-GW to perform downlink data storage. The MME forwards the stored downlink data/singalling or it indicates S-GW to forward the stored downlink data to eNB if the feeder links becomes available.

-	When eNB detects the service link is not available, it performs downlink data/singalling storage and forwards the stored downlink data/signalling if service link becomes available.

-	Architectural assumptions and principles defined in clause 4 in 23700-29 is applied to this solution.

* * * Next change (with only changes to attach procedure)* * * *

6.21.2	Procedures

1	Attach procedure.

Figure 6.21.2-1: UE attach without PDU connectivity procedure based on regenerative satellite architecture

The attach procedure for supporting store and forward feature is described as below:

- An eNB on board the satellite shall broadcast Store and Forward mode if supporting S&F satellite operation. Besides, whether it supports CP or UP CIoT EPS optimisation shall be broadcasted as well.

- A UE, camping on the eNB, reads the related SIB message. It knows the satellite access being accessed is operating in S&F mode. If the UE supports S&F capability and wants to access via the satellite access to Winitiate delay tolerant service, it initiates the attach procedure by sending an attach request (S&F capability, preferred network behavior) message together with RRC parameters to the onboard eNB.

The UE supporting S&F capability is included in the attach request message. The UE indicated CP/UP CIoT EPS Optimisation in preferred network behaviour. In the RRC message, the UE indicates its support of the CIoT EPS Optimisations. In this case, ESM message container may be omitted in the attach request message. If the ESM message container is omitted, the MME shall not establish a PDN connection as part of the attach procedure, step 11 and 17 are not executed.

For the case of UE attaching with CP CIoT EPS Optimisation with no user plane establishement, steps 12-16 are replaced by S1 AP NAS Transport and RRC Direct Transfer messages that transport the NAS attach accept and NAS attach complete messages.

- After receving attach request message, if the on board eNB detects feeder link is unavailable, the on board eNB stores the message for the UE.

- The on board eNB sends an attach notification message to the UE to inform the UE that the attachment is under S&F mode and attach response will be delayed. After receiving the notification, the UE will not try to send attach request again to the on board eNB until the service link becomes available.

- If the on board eNB detects feeder link is available, it forwards the attach request in a S1 AP message together with eNB operating in S&F mode to the MME.

- If no UE context for the UE exists in the MME, if the attach request was not integrity protected, or if the check of the integrity failed, authentication and NAS security setup is needed. The MME request authenction vectors from HSS for the UE using IMSI.

- The MME sends the authentication vectors to the eNB for the UE. The eNB may store the information for the UE if the service link is unavailable.

Editor’s note:	How to handle the case that the on board eNB stores the UE related authentication information without security mechanism is created is upto SA3.

- If the UE receives attach notification in step 3, and if the UE detects the service link becomes available, the UE resend the attach request to the on board eNB.

- When receiving the attach request, the eNB sends the stored authentication vectors to the UE.

- The UE handles the authentication vectors and returns the authentication response to the eNB. The eNB stores the authentication response if the feeder link is unavailable.

- The eNB sends the stored authentication response message to the MME when feeder link becomes available. The MME authenticates the UE using the authentication vectors.

Editor’s note:	Whether there is any impact on the authentication procedure to the UE in case of S&F satellite operation is applied is upto SA3 work.

- If there is no valid subscription context for the UE in the MME, or if the MME has changed since the last detach, the MME sends an update location request (MME identity, IMSI, etc) message to the HSS. The HSS acknowledges the update location message by sending an update location ack (IMSI, Subscription data) message to the MME. If the UE supports S&F capability, the Subscription data contains the subscribed S&F information, e.g. data storage quota, data forward priority, etc.

- If the UE provided supporting S&F capability, the on board eNB provided operating in S&F mode, and if the MME supports S&F mode, the MME stores S&F operating in the UE context in MME. The MME sends an attach accept (supported network behaviour) together with S&F subscription information (e.g. data storage quota, data forward priority) in a S1 AP NAS Transport message to the on board eNB. The MME indicates the CIoT EPS Optimisations and S&F mode it accepts in the supported network behaviour information. The MME may take the UE reporting S&F capability, the on boarding reporting S&F mode, UE subscription, the network capability into account to determine applying S&F mode for the UE. The on board eNB stores the S&F subscription information in the UE context.

- The on board eNB sends a RRC Direct Transfer containing the attach accept message to the UE when service link becomes available.

- The UE sends a RRC Direct Transfer containing the attach complete message to the on board eNB.

- The on board eNB forwards the attach complete message to the MME in an uplink NAS Transprot message when feeder link becomes available.

Editor’s note:	How to handle the NAS timer to support the UE attach in S&F operation depends on CT1 decision.

2	MO data transmission using control plane signalling based on S&F operation

Figure 6.21.2-2: MO data transmission using control plane signalling based on S&F operation

The MO data transmission based on S&F operation is described as below:

0.	The UE is ECM-IDLE.

1.	The UE establishes a RRC connection or send the RRC early data request message as defined in TS 36.300 [13] and sends as part of it an integrity protected NAS PDU. The NAS PDU carries the EPS bearer ID and encrypted Uplink Data. The UE may also indicate a NAS Release Assistance Information in the NAS PDU whether no further uplink or downlink data transmissions are expected, or only a single downlink data transmission (e.g. Acknowledge or response to uplink data) subsequent to this uplink data transmission is expected

2.	If the on-board eNB detects feeder link is unavailable, it stores the NAS DATA PDU based on the data storage quota. How to handle the data if the quota is over can be based on operator policy. E.g. discard the data and inform the event to UE.

3.	After a period of time, if the on-board eNB detects the feeder link becomes available, then it forwards the stored NAS DATA PDU based on the data forward priority. For example, if there is network congestion during the feeder link recovery, the data with high priority can be transmitted first.

4.	The on-board eNB forwards the NAS DATA PDU in the S1-AP intial UE message to the MME.

5.	The MME checks the integrity of the incoming NAS PDU and decrypts the data. If the S11-U connection is not established, the MME sends a Modify Bearer Request to SGW to establish S11-U connection with S-GW. The MME sends uplink data via the S11-U connection to the S-GW and further to P-GW.

6.	If the NAS Release Assistance Information from the UE indicates no downlink data is expected, or if the MME is not aware of pending MT traffic and S1-U is not established, step6-8 skips. Otherwise, downlink data may arrive at the P-GW and the P-GW sends the downlink data to the MME via the S-GW.

7.	If the MME detects feeder link is unavailable, it stores the downlink data based on the data storage quota.

8.	When the feeder link is detected becomes available, the MME encrypts and integrity protects to the stored downlink data.

9.	The MME forwards the stored downlink data to the eNodeB in a S1-AP Downlink NAS Message based on data forward priority. Data encapsulation may be applied in the transmission. The MME may indicates acknowledgment is requested as per UE subscription. If the NAS Release Assistance Information in step 1 indicated that downlink data was expected and if MME does not aware of additional MT traffic or S1-U bearer, the MME sends a S1 UE Context Release Command immediately after the S1-AP to indicate eNB to release the RRC connection promptly after successfully sending data to the UE.

10.	The eNB receives the downlink data. If it detects the service link is unavailable, it stores the downlink data. The storeage is based on the storeage quota.

11.	If the service link is detected available, the eNB sends a RRC downlink data message including the downlink data to the UE. If the eNB receiving the S1-AP message in step 9 was followed by an S1 UE Context Release Command, an S1 release procedure is enforced after the downlink data is transmitted to the UE and the acknowledgement is sent to the MME by the eNB.

12.	Other steps of the MO data transport in Control Plane CIoT EPS Optimization defined in clause 5.3.4B.2 of TS 23.401 [5] are performed. During these steps, if feeder link or service link is detected unavailable, the Store and Forward feature as described in steps 2-3, 7-8, or 10-11 will be performed will be performed at eNB or MME accordingly.

3	MT data transmission using control plane signalling based on S&F operation:

Figure 6.21.2-3: MT data transmission using control plane signalling based on S&F operation

The MT data transmission based on S&F operation is described as below:

1.	When the S-GW receives a downlink data for a UE, it forwards the downlink data to the MME as described in steps 1-2, 7-11 in clause 5.3.4B.3.

2.	If the MME detects feeder link is unavailable, it stores the downlink data based on the data storage quota.

3.	After a period of time, the MME detects the feeder link becomes available.

4.	The MME sends Paging message as specified in step 3a of clause 5.3.4.3.

5.	The eNB receives the Paging message. When eNB detects service link is unavailable, it records the paging information, e.g. the UE location information. The eNB indicates the paging message is delay due to the service link unavailability. A waiting time is also notified to the MME which can be set based on ephemeris information.

6.	The eNB detects the service link becomes available. It uses the paging information received in step 4 to send paging to the UE.

7.	The eNB sends Paging message to the UE as specified in step 4a of clause 5.3.4.3.

8.	Upon reception of paging indication, the UE sends Control Plane Service Request NAS message over RRC Connection request. The Control Plane Service Request NAS message does not trigger data radio bearer establishment by the MME.

9.	The eNB stores Control Plane Service Request NAS message if the feeder link is unavailable.

10.	When the feeder link recovers, the eNB sends Control Plane Service Request NAS message over S1-AP initial message.

11.	When the MME receives Control Plane Service Request NAS message, it can send the stored downlink data to the eNB based on the data forward priority using a NAS PDU carried by a Downlink S1-AP message. The downlink data may be encrypted and integrity protected by the MME.

12.	If the on-board eNB detects service link is unavailable, it stores the downlink data based on the data storage quota.

13.	After a period of time, the on-board eNB detects the service link becomes available.The stored downlink data is delivered to the UE via a Downlink RRC message based on the data forward priority. This is taken as implicit acknowledgment of the Service Request message sent in step 8.

14.	Other steps of the MT data transport in Control Plane CIoT EPS Optimization defined in clause 5.3.4B.3 of TS 23.401 [5] are performed. During these steps, if feeder link or service link is detected unavailable, Store and Forward feature as described in steps 2-3, 9, 12 will be performed at eNB or MME accordingly.

******the remaining description of this clause remains unchanged************

* * * Next change * * * *

6.21.3	Impacts to Services, Entities and Interfaces

The on-board eNB has the following impacts:

-	Reports S&F capability to MME

-	Receives and stores S&F subscription information for the UE

-	Detects the available/unavailable of service link/feeder link

-	Performs S&F opteration for data/signalling based on UE S&F subscription information

-	Broadcasts S&F mode information if it supports.

-	Sends attach notification to UE when feeder link is unavailable

The MME has the following impacts:

-	Determine to apply S&F capability for a UE based on eNB S&F capability and UE subscription information

-	Sends UE S&F subscription information to the eNB

-	Detects the available/unavailable of feeder link

-	Performs S&F operation for data/signalling based on UE S&F subscription information

The S-GW may have the following impacts:

-	Performs S&F operation for data based on MME indication

UE may have the following impacts:

- Receiving and identifying S&F mode information from eNB

- Sends its S&F capability to the MME

- Receive attach notification and resend attach request if service link becomes available

* * * End of change * * * *