3GPP TSG-WG SA2 Meeting #162	S2-2404793

Changsha, China, April 15th – 19th, 2024	(revision of S2-2402289)

# Title:	KI#1&2&3: Sol #y Definition of network energy related information KPIs;

# Source:	Huawei, HiSilicon

# Document for:	Approval

Agenda Item:	19.4

Abstract:	The essential elements to be defined are the network energy related information KPI per UE and PDU session. These are the basis for al KIs, i.e. for the exposure , profile and definition of mechanism for energy saving and efficiency.

# 1. Introduction

The document proposed the following principle for evaluation of energy related KPI per UE, PDU session and QoS flow granularity:

The scope of this contribution which is the revision of merged S2-2402289 and S2-2402822 is :

- Propose procedure for the estimation of Energy consumption and energy Efficiency

- The estimation is performed per UE, PDU session and QoS flow granularity

- The estimation proposed is base for the evaluation of the contribution of all NFs for completeness, but the paper propose some simplification for this release.

# 2. Consideration based TS 22.216 requirements

The TS 22.216 requirements in 6.15a refer to the support energy consumption monitoring, to the capability to limit the maximum energy consumption  and to enable the energy credit control.

For such reasons this contribution proposes only the KPI for the evaluation of Energy Consumption and Energy efficiency.

The energy related information per UE and PDU session is evaluated as follows

- The sum of the contributions of gNB, UPF serving the UE and PDU session

- The contribution of N3GPP access node and transport network is considered as a constant contribution to be added to the energy related value. The contribution may be set equal to 0 (zero) or to a value determinated by PLMN based on considerations outside the scope of this SID (for example considering the EC of the overall network component in the previous evaluation period (e.g. previous year, previous months, etc) divided by the number of data volume transmitted in the previous evaluation period. The value may be pre-configured in the 5G CN or passed by OAM.

- For satellite the following considerations apply:

  - The energy related contribution for element on-board of a satellite, such as gNB or UPF, is considered equal to 0 (zero), since the satellite is powered directly in the space

  - When a satellite is used as backhaul of 3GPP system the component at earth are considered as transport network elements and considered as described in bullet 2

  - The contribution of UPF when deployed at ground for satellite RAT is considered as part of UPF contribution, since it is not distinguisible from the UPF used for a NR RAT.

- The contribution of 5G CN

- The contribution of energy related information, e.g. EC, of AF when deployed by operator is as a constant contribution to be added to the energy related value. The contribution may be set equal to 0 (zero) or to a value determinated by PLMN based on considerations outside the scope of this SID. When the AF is deployed by a 3rd party is not considered.

- The contribution of UE is not considered as part of PLMN

- The standalone EPC and the effect of 4G BTS is not in the scope of the SID therefore is proposed to not consider in this release of the SID.

# 2 Text proposal

The document proposes to add the following changes in the TR

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| 1 | x |  |  |  |
| 2 | x |  |  |  |
| 3 | x |  | x |  |
| 4 | x |  |  |  |
| 5 | x |  | x |  |
| 6 | x |  |  |  |
| 7 |  | x |  |  |
| 8 |  | x |  |  |
| 9 |  | x |  |  |
| 10 |  | x |  |  |
| 11 |  | x | x |  |
| 12 |  |  | x |  |
| 13 |  |  | x |  |
| 14 |  |  | x |  |
| 15 |  | x | x |  |
| X | X | X | X |  |

# * * * * Second change * * * * all new text. The revision marks show the difference in respect the previous version of the document

## 6.x	Solution #y: Energy related KPIs

The solution provide the KPIs for the parameters related to energy information which can be considered as for exposure, for definition and evaluation of energy saving and energy efficiency procedure and for energy as a services.

### 6.y.1	Key Issue mapping

The solution is applicable to all KIs, since it defines the KPIs to be used to evaluate the energy related information.

### 6.y.2	KPI definition

The KPI are numerical representation of the Energy Consumption, Energy Efficiency, Carbon Intensity and Renewable ratio evaluated per the following granularity:

-	per UE

-	per PDU session

-	per QoS flow

For the definition of the KPI is assumed that the Energy Consumption is per node granularity. The KPI definition does not depend by whether the EC per node is provided by OAM or evaluated directly from the NF. The requirement is that the EC is provided on a measurement period which is known and any other information required to perform the evaluation per UE, PDU session or QoS flow is available for the same measurement period.

The principles of this solution are:

-	the contribution of the UE to EC, Carbon Emission and renewable ratio is considered outside the scope of this SID.

-	the contribution of the 4G network to EC, Carbon Emission and renewable ratio is considered outside the scope of this SID.

-	the EC, Carbon Emission and renewable ratio per UE, PDU session and QoS flow granularity is obtained as the sum up of the contribution of the serving NF involved in the managing of the specific UE, PDU session and QoS flow. Therefore it is required and assumed that the NF performing the evaluation shall determine which are the serving NFs.

-	the EC, Carbon Emission and renewable ratio contribution of gNB, UPF and Core network Function per UE, PDU session and QoS flow granularity is evaluated as a proportion of the EC, Carbon Emission and renewable ratio of the whole NF to manage the specific UE, PDU session and QoS flow. How the proportion is performed is left to implementation.

-	the EC, Carbon Emission and renewable ratio contribution of Core network per UE, PDU session and QoS flow granularity is evaluated as sum of a proportion of the EC, Carbon Emission and renewable ratio of AMF, SMF which manage the specific UE, PDU session and QoS flow. How the proportion is performed is left to m implementation. In addition it is added a constant contribution representing the sum of other NF which is determined by PLMN based on considerations outside the scope of this SID.

-	the EC, Carbon Emission and renewable ratio contribution of NTN, Non-3GPP access node, Transport network , per UE, PDU session and QoS flow granularity are considered a constant value contribution which is determined by PLMN based on considerations outside the scope of this SID

.NOTE: the solution proposes also how the contribution of this element might be considered for completeness and for potential possible future usage.

### 6.y.3 Energy Consumption per UE

The Energy Consumption is evaluated per NFs, per physical node and per slices (as defined in TS 28.554 [6] clause 6.3.2.1) and described in Annex 1.

The Energy Consumption per UE represents the overall Energy Consumption of the Mobile Network due to the traffic provided to the given UE in a period of time defined as

ECUE, Estimated=ECUE+ECUE,gNB+ECUE,UPF+ECUE,TNGF+ECUE,N3IWF+ECUE,CN+ECUE,4G +ECUE,NTN+ECUE,transport

where:

-	ECUE represents the EC of the UE in the evaluation period, i.e. of the smartphone. This release of the study this contribution is considered outside the scope of this SID. For more details see clause 6.y.3.1.

-	ECUE,gNB represents the EC contribution of the gNB(s) due to the traffic and signaling of the UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.2.

-	ECUE,UPF represents the EC contribution of the UPF(s) due to the traffic and signaling of the UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.3.

-	ECUE,TNGF represents the EC contribution of the TNGF(s) due to the traffic and signaling of the UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.5.

-	ECUE,N3IWF represents the EC contribution of the N3IWF(s) due to the traffic and signaling of the UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.5.

-	ECUE,CN represents the EC contribution of the CN NF(s), e.g. AMF, SMF, PDF, UDM, etc which controls the UE and performs the signaling of the UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.4.

-	ECUE,4G represents the EC contribution of the 4G Network where the UE can be connected during the evaluation period. This release of the study this contribution is considered outside the scope of this SID.

-	ECUE,NTN represents the EC contribution of the NTN RAT and CN part where the UE can be connected during the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.6.

-	ECUE,Transport represents the EC of the backhaul network involved in the transport of the mobile network traffic in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.7.

The above equation considers the sum of different contributions in order to take into account two different factor, the different combination of NFs which are involved for each UE for providing the services and controlling the UE and the time variation.

Different UEs which are using the mobile network can be served by different NF, as shown in Figure 6.y.3.1-1 when a UE1 has 2 PDU sessions with 2 UPFs and UE 2 only 1 PDU session (figure a) or UE1 is also connected to N3GPP access (Figure b) or the UE has a MA PDU session.

(a)

(b)

(c)

Figure 6.y.3.1-1:Example of UEs making usage of the mobile network

The second factor to be considered is the time variation, for example if the evaluation period if 1 hour, and the UE has been connected to the gNB A for 15 minutes, to gNB B for 40 minutes and to gNB C for 5 minutes, or any other conditions change , e.g. the UE1 move 1 PDU session to N3GPP, then start a MA PDU, move and change the UPF etc, etc, therefore the ECs due to the UE needs to be evaluated for the same period of time consistently. It should be noted that more the evaluation period is longer, more the services and UE positions can change in the time therefore more EC contribution from different NFs shall be considered in order to have an appropriate evaluation of the EC.

#### 6.y.3.1 Energy Consumption of UE

The ECUE contribution includes the consumption of the UE battery due to related to the UE layer (i.e. the transmission and reception of the data and signaling from/to the mobile network and all functionalities related to the CN interaction such as NAS termination. PDU session, etc, etc,) and then all EC of the UE battery related to the service/application which is running via 3GPP network. In order to allow the CN to consider this UE contributions to the overall EC, the UE has to communicate directly to the CN.

For this release it is assumed that Energy Consumption of the UE is not considered.

#### 6.y.3.2 Energy Consumption of gNB per UE

The solution assumes the EC is provided for the gNb on a measurement period.

In order to evaluate the EC contribution due to gNBs, we need to consider that the UE may change gNB during the evaluation period . therefore the EC of all gNBs where the UE has been connected need to be summed.

ECUE,gNB=gNBECUE,gNBj

The EC contribution of gNB j to UE traffic is an average of the total EC of the gNB j. The function f(x,y..) is left to implementation and it can consider a different set of information related to the gNB available at the NF which makes the evaluation

ECUE,gNBj=fx,y,..*ECgNBj

Where

-	The EC contribution of a gNB ECfgNB is obtained by summing up the Energy Consumption of all the Network Functions (NF) that constitute the gNB as defined in TS 28.544 clause 6.7.3.4.

-	The function f(x,y..) is left to implementation and it can consider a different set of information related to the gNB available at the NF which makes the evaluation, for example it can be a proportion of the UE data volume to the total gNB data volume.

, The

Fx,y=FUE Data Volume,  gNB Total Data Volume= UE Data VolumegNB Total Data Volume

Where the UE data volume and the gNB data volume is the Data volume at N3 interface measured in UPF. The UE data Volume is the same volume used for charging and represent the traffic of the UE’s PDU session within the GTP, while the gNB traffic is the sum of the all traffic on the N3 interface from the gNB.

#### 6.y.3.3 Energy Consumption of UPF per UE

The OAM provides the EC on a measurement period for the all single UPFs in the network. The EC related to the UPF is intended to be the EC of the physical node hosting the UPF or the EC of the NF representing the virtual UPF hosted in a physical node. From SA2 point of view the EC corresponds to the UPF managing the PDU session.

In order to evaluate the EC contribution due to UPFs, the EC of all UPFs managing the PDU session of the UE (i.e. PSA UPF , I-UPF, ..) shall be considered, for example for the UE1 in Figure 6.y.3.1-1 the contribution of UPF1 and UPF2 needs to be summed.

The total EC contribution of UPF to UE traffic is the sum of the contribution of each UPFs transferring the UE traffic (PDU session) as follow

ECUE,UPF=UPFECUE, UPFj

Where the EC contribution of UPF j to UE traffic is proportional to the total EC of the UPF j ECUPF.

The function f(x,y..) is left to implementation and it can consider a different set of information related to the UPF available at the NF which performs the evaluation

ECUE,UPFj=fx,y,..*ECUPFj

The EC includes the dynamic contribution due to the operation performed on the data traffic, the control operation and the related signaling toward the SMF, and the static energy consumption due to the fact that the UPF is running (as any equipment the EC is not null also in case of lack of traffic). Furthermore, the actions performed to the traffic of different UEs can be very different for example due to PDU session type, different QoS to be managed, whether DPI is used or not, whether the traffic is TSN or not, etc.  Therefore, the EC for UE can only be estimated based on available information at the network function performing the evaluation. The estimation can be done considering different element, for example proportionally to the Data traffic of the UE, proportionally to the slice based on PDU sessions, etc, etc.

For example the same approach based on data traffic as specified in TS 28.554 clause 6.7.3.3 for evaluation UPF contribution to network slice consumption may be used,

Fx,y=FUE Data Volume,  UPF Total Data Volume== UE Data VolumeUPF Total Data Volume

Alternative the The function f(x,y..) can be left to implementation and it can consider a different set of information related to the UPF available at the NF which performs the evaluation

It shall be noted that an UPF can also manage the traffic of PDU session that are currently over N3GPP access and over NTN RAT, in that case even if the contribution of access part is not considered, in the above evaluation this is considered by default. This contribution may be eliminated from the data traffic, for example if the function f(x,y,..) does not take into account the traffic, PDU sessions over a N3GPP access and over NTN RAT, but the ECUPFj provided by OAM includes the consumption due to such traffic which can not be distinguished when EC is measured.

#### 6.y.3.4 Energy Consumption of Core Network per UE

The energy consumption for the CN ECUE,CN needs to consider the contribution of all NFs. Generally speaking the current core network NF are deployed as virtual functions hosted in a physical server and from the deployment point of view the AMF and SMF are more numerous than PCF, UDR, UDM and LCS, NEF, etc.

In literature for 4G, roughly the energy consumption split between RAN and CN is 70-80% RAN, 15% CN, 5% data center, but literature also mentioned that 5G EC is higher than 4G. Furthermore generally speaking, the Core Network Function may be deployed as physical nodes or as virtual functions hosted on a servers hardware. The latter is the most common deployment considering the nature of the 5G Core Network functionalities.

The TS 28.554 defines the energy consumption of the 5G CN in clause 6.7.3.2 as sum up of energy consumption of Network Function ECNF . The ECNF  is defined in TS 28.554 clause 6.7.3.1 as sum of the energy consumption of physical node (PNF) and/or virtual function (VNF) which compose the NF.

The TS 28.554 in clause 6.7.3.3 when consider the contribution of the slice the proportion of the EC of AMF to be assigned to a slice is based on the number of UE since the AMF has a NAS per UE and manage the mobility, while for the SMF is proportional to the PDU sessions, since they represent the element which is managed.

The Example of equations to estimate the EC for all NF are reported in the table 6.y.3-1

The AMF and SMF are NF more often involved in managing the UE activity, while network function may depend by the services and supported features, such as for LCS or MBS or NEF for exposure. Therefore we consider in this release of the study only the contribution of AMF and SMF is considered, while for the other NFs is considered a constant value contribution which is determined by PLMN contribution based on considerations outside the scope of this SID.

Therefore the total EC contribution of CN NF to UE is the sum of the contribution of serving AMF and serving SMF plus a constant value as follow

ECUE,NF=ECUE,AMF+SMFECUE, SMFj+ECUE,NF'

Where

- ECUE,AMF is EC contribution of serving AMF evaluated as proportion of EC of the serving AMF (ECUE,AMF) for the specific UE as below. The function f(x,y..) is left to implementation and it can consider a different set of information related to the UPF available at the NF which performs the evaluation

ECUE,AMF=fx,y,..*ECAMF

- ECUE,SMFj is EC contribution of serving SMF evaluated as proportion of EC of the serving AMF (ECUE,SMFj) for the specific UE as below. The function f(x,y..) is left to implementation and it can consider a different set of information related to the UPF available at the NF which performs the evaluation

ECUE,SMFj=fx,y,..*ECSMFj

- ECUENF’ is EC contribution of other core network function represented as a constant value contribution which is determined by PLMN contribution based on considerations outside the scope of this SID

It is considered only the contribution of serving AMF , since the UE is served only by 1 AMF when connected to 3GPP. There is only a limited roaming scenario where a UE may be served by 2 AMFs when connected to 3GPP and N3GPP which is not considered in this release of specification.

Table 6.y.3-1: Energy consumption

| Entity | Energy Consumption KPI per UE |
| --- | --- |
| AMF | EC of the AMF divided by the maximum/mean number of registered subscribers in the measurement period, |
| SMF | EC of SMF multiplied by the ratio between the number of PDU session of the UE and the total number of PDU sessions in the measurement period. |
| PCF | EC of PCF multiplied by the ratio between the number of Policy events (AM policy, SM policy, UE policy, Background data transfer policy, and Event exposure) of UE and the total number of Policy events in the measurement period |
| UDM | EC of UDM divided the average number of registered subscribers through UDM in the measurement period. |
| NEF | Editor’s note: how to estimate the EC per UE granularity is FFS. |
| NRF | Editor’s note: how to estimate the EC per UE granularity is FFS. |
| NSSF | Editor’s note: how to estimate the EC per UE granularity is FFS. |
| SMSF | Total number of measurements related to MO SMS message delivery, MT SMS message delivery, registration procedure, or select performance measurements for specific aspect(s) as defined in clause 5.12 of TS 28.552. |
| UDR | EC of UDM divided the average number of registered subscribers through UDR in the measurement period. |
| ECS | Editor’s note: how to estimate the EC per UE granularity is FFS. |
| EES | Editor’s note: how to estimate the EC per UE granularity is FFS. |
| LMF | EC of LMF multiplied by the ratio between the sum of the Location determination, Location notification, and Location context transfer for the specific UE divided the total number of location measurement |
| NWDAF | Editor’s note: how to estimate the EC per UE granularity is FFS. |

#### 6.y.3.5 	Energy Consumption of N3GPP access network per UE

The ECUE,TNGF and ECUE,N3IWF contributions represent the EC due to the amount of traffic which is carried over the WLAN towards the N3IWF and then in the 3G CN. It should be noted that for the N3GPP traffic, the EC contribution of the N3GPP Access , e.g. the WLAN AP, possible fixed access line and the transport network can not be evaluated since they may be outside the operator domain or within the operator domain , but not in Mobile network part. The N3IWF generally is managing a great number of UE at the same time, they are currently not numerous and often dedicated to support VoWifi services which use a small amount of data, therefore this solution proposes that:

-	the Energy Consumption contributions of N3GPP access nodes ECUE,TNGF and ECUE,N3IWF are considered a constant value contribution which is determined by PLMN contribution based on considerations outside the scope of this SID.

-	The ECUE,TNGF and ECUE,N3IWF value may be pre-configured in the 5G CN or passed by OAM.

-	The value may assume the value 0 (zero) and it may be considered directly in the during the charging procedure in charging system

NOTE 1:	for example the EC of the N3IWF and TNGF may be a configured as value proportional to the bit rate to be added as function of the actual data value of the user or it may be constant value equal for all UE to be added irrespectively of the data volume.

#### 6.y.3.6 	Energy Consumption of Transport network per UE

In order to evaluate the ECUE,Transport contribution to a each UE, it is needed to know the EC of each transport nodes involved in carrying the traffic of the UE by the OAM, which usually is managed by a separate system or in several scenario by a different operators, since the mobile operator may lease transport line or capacity. The contribution of EC of transport network is variable by operator to operator. therefore this solution proposes that :

-	the Energy Consumption contributions of the transport network ECUE,Transport is a constant value contribution which is determined by PLMN contribution based on considerations outside the scope of this SID.

-	The ECUE,Transport value may be pre-configured in the 5G CN or passed by OAM.

-	The value may assume the value 0 (zero) and it may be considered directly in the during the charging procedure in charging system.

NOTE 2:	For example the EC of the overall network component in the previous evaluation period such as previous year, previous months, etc, divided by the number of data volume transmitted in the previous evaluation period in order to have a value proportional to the bit rate to be added as function of the actual data value of the user or it may be constant value equal for all UE to be added irrespectively of the data volume.

#### 6.y.3.7 Energy Consumption of NTN network per UE

The contribution to energy consumption of the Satellite ECUE,NTN when the gNB or UPF are onboard is considered equal to 0 (zero), since the satellite consumption does not impact the power electricity on ground.  This solution proposes that

-	when the gNB and or UPF are onboard the Energy Consumption contribution to UE ECUE,NTN is equal to 0 (zero)

-	The ECUE,NTN contribution of the ground station is a considered as part of transport network contribution which is determined by PLMN contribution based on considerations outside the scope of this SID.

-	When the traffic from NTN is terminated in an UPF on ground, this contribution is considered as part of UPF contributions ECUE,UPF

#### 6.y.3.8 Energy Consumption of 4G network per UE

The standalone EPC and the effect of 4G BTS is not in the scope of the SID therefore is proposed to not consider the ECUE,4G contribution in this release of the SID.

### 6.y.4. Energy Consumation per PDU session

The Energy Consumption per PDU represents the overall Energy Consumption of the Mobile Network due to the traffic provided to the given PDU Session for a given UE in a period of time defined as

ECPDU ses, Estimated=ECPDU Ses+ECPDU Ses,gNB+ECPDU Ses,UPF+ECPDU Ses,TNGF+ECPDU Ses,N3IWF+ECPDU Ses,CN+ECPDU Ses,NTN+ECPDU Ses,transport=

where:

-	ECPDU ses represents the EC of UE due to the PDU session in the evaluation period, i.e. of the smartphone. This release of the study this contribution is considered outside the scope of this SID. For more details see clause 6.y.3.1.

-	ECPDU Ses,gNB represents the EC contribution of the gNB(s) due to the traffic and signaling of the PDU Session in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.2.

-	ECPDU Ses,UPF represents the EC contribution of the UPF(s) due to the traffic and signaling of the PDU Session in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.3.

-	ECPDU Ses,TNGF represents the EC contribution of the TNGF(s) due to the traffic and signaling of the PDU Session in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.5.

-	ECPDU Ses,N3IWF represents the EC contribution of  the N3IWF(s) due to the traffic and signaling of the PDU Session in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.5.

-	ECPDU Ses,CN represents the EC contribution of  the CN NF(s), e.g. AMF, SMF, PDF, UDM, etc which controls the UE and performs the signaling of the PDU Session in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.4.

-	ECPDU Ses,4G represents the EC contribution of the 4G Network where the PDU Session can be connected during the evaluation period. This release of the study this contribution is considered outside the scope of this SID.

-	ECPDU Ses,NTN represents the EC contribution of of the NTN RAT and CN part where the PDU Session can be connected during the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.6.

-	ECPDU Ses,Transport represents the EC of the backhaul network involved in the transport of the mobile network traffic in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.7

#### 6.y.4.1 Energy Consumption of UE per PDU Session

For this release it is assumed that Energy Consumption of the UE is not considered based on same motivations described in clause 6.y.3.1

#### 6.y.4.2 Energy Consumation of gNB per PDU Session

The same approach defines in clause 6.y.3.2 is used and therefore the EC contribution of gNB to PDU session traffic is the sum of the average of the total EC of the serving gNB j

ECPDU Ses,gNB=gNBECPDU Ses,gNBj=gNBF(x,y,..)ECgNBj

#### 6.y.4.3 Energy Consumption per PDU session per UPF

The same approach defines in clause 6.y.3.3 is used and therefore the EC contribution of UPF to PDU session traffic is the sum of the proportion of the total EC of the serving UPF

ECPDU Ses,UPF=UPFECPDU ses, UPFj=UPFg(x,y,..)EC UPFj

#### 6.y.4.4 Energy Consumption per PDU session per CN

The same approach defines in clause 6.y.3.4 is used and therefore the EC contribution of CN to PDU session traffic is the sum of the proportion of the total EC of the serving AMF, serving SMF and a constant value representing the contribution of the other CN NFs

ECPDU Ses,NF=ECPDU Ses,AMF+SMFECPDU Ses, SMFj+ECPDU Ses,NF'

#### 6.y.4.5 	Energy Consumption of N3GPP access network per PDU Session

The same approach defines in clause 6.y.3.5 is used.

#### 6.y.4.6 	Energy Consumption of NTN network per PDU Session

The same approach defines in clause 6.y.3.6 is used.

#### 6.y.4.7 	Energy Consumption of Transport network per PDU Session

The same approach defines in clause 6.y.3.7 is used.

### 6.y.5 Energy Consumption per QoS flow

The Energy Consumption per QoS flow represents the overall Energy Consumption of the Mobile Network due to the traffic provided to the given QoS flow of a given PDU session for a given UE  in a period of time defined as

ECQoS Flow, Estimated=ECQoS Flow+ECQoS Flow,gNB+ECQoS Flow,UPF+ECQoS Flow,TNGF+ECQoS Flow,N3IWF+ECQoS Flow,CN+ECQoS Flow,NTN+ECQoS Flow,transport=

where:

-	ECQoS Flow represents the EC of UE due to the PDU session in the evaluation period, i.e. of the smartphone. This release of the study this contribution is considered outside the scope of this SID. For more details see clause 6.y.3.1.

-	ECQoS Flow,gNB represents the EC contribution of the gNB(s) due to the traffic and signaling of the QoS flow of a given PDU session for a given UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.2.

-	ECQoS Flow,UPF represents the EC contribution of the UPF(s) due to the traffic and signaling of the QoS flow of a given PDU session for a given UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.3.

-	ECQoS Flow,TNGF represents the EC contribution of the TNGF(s) due to the traffic and signaling of the QoS flow of a given PDU session for a given UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.5.

-	ECQoS Flow,N3IWF represents the EC contribution of  the N3IWF(s) due to the traffic and signaling of the QoS flow of a given PDU session for a given UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.5.

-	ECQoS Flow,CN represents the EC contribution of  the CN NF(s), e.g. AMF, SMF, PDF, UDM, etc which controls the UE and performs the signaling of the QoS flow of a given PDU session for a given UE in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.4.

-	ECQoS Flow,4G represents the EC contribution of the 4G Network where the QoS flow of a given PDU session for a given UE can be connected during the evaluation period. This release of the study this contribution is considered outside the scope of this SID.

-	ECQoS Flow,NTN represents the EC contribution of of the NTN RAT and CN part where the QoS flow of a given PDU session for a given UE can be connected during the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.6.

-	ECQoS Flow,Transport represents the EC of the backhaul network involved in the transport of the mobile network traffic in the evaluation period. The evaluation of this contribution is defined in clause 6.y.3.7

#### 6.y.4.1 Energy Consumption of UE per QoS flow of PDU session of UE

For this release it is assumed that Energy Consumption of the UE is not considered based on same motivations described in clause 6.y.3.1

#### 6.y.4.2 Energy Consumation of gNB per QoS flow of PDU session of UE

The same approach defines in clause 6.y.3.2 is used and therefore the EC contribution of gNB to PDU session traffic is the sum of the average of the total EC of the serving gNB j

ECQoS flow,gNB=gNBECQoS Flow,gNBj=gNBF(x,y,..)ECgNBj

#### 6.y.4.3 Energy Consumption of UPF per QoS flow of PDU session of UE

The same approach defines in clause 6.y.3.3 is used and therefore the EC contribution of UPF to PDU session traffic is the sum of the proportion of the total EC of the serving UPF

ECQoS flow,UPF=UPFECQoS flow, UPFj=UPFg(x,y,..)EC UPFj

#### 6.y.4.4 Energy Consumption of CN per QoS flow of PDU session of UE

The same approach defines in clause 6.y.3.4 is used and therefore the EC contribution of CN to PDU session traffic is the sum of the proportion of the total EC of the serving AMF, serving SMF and a constant value representing the contribution of the other CN NFs

ECPQoS flow,NF=ECQoS flow,AMF+SMFECQoS flow, SMFj+ECQoS flow,NF'

#### 6.y.4.5 	Energy Consumption of N3GPP access network per QoS flow of PDU session of UE

The same approach defines in clause 6.y.3.5 is used.

#### 6.y.4.6 	Energy Consumption of NTN network per QoS flow of PDU session of UE

The same approach defines in clause 6.y.3.6 is used.

#### 6.y.4.7 	Energy Consumption of Transport network per QoS flow of PDU session of UE

The same approach defines in clause 6.y.3.7 is used.

### 6.y.6 Energy Efficiency KPI

Editor’s note: the Energy Efficiency KPI is FFS.

### 6.y.7 Carbon Emission

The Carbon Emission is assumed to be provided by OAM per NFs and per physical node/.

The Carbon Emission is directly connected to the Energy Consumption via the mix of power sources used to produce the electric power used for each element. Therefore the same methodology defined in the clause 6.y.3 and 6.y.4 applies where the Carbon Emission replaces the EC. Thus the same formulas apply replacing the CE term to EC term. In the following is defined the  Carbon Emission (CE) per UE represents the overall Carbon Emission of the Mobile Network due to the traffic provided to the given UE in a period of time defined. For each contribution the description in clause 6.y.3 applies. The calculation for the evaluation of the Carbon Emission (CE) per PDU session is omitted since it can be derived from definitions described in clause 6.y.4.

NOTE : 	Carbon intensity is a measure of how clean our electricity is. It refers to how many grams of carbon dioxide (CO2) are released to produce a kilowatt hour (kWh) of electricity.

CEUE, Estimated=CEUE+CEgNB+CEUE,UPF+CEUE,TNGF+CEUE,N3IWF+CEUE,CN+CEUE,4G +CEUE,NTN+CEUE,transport=

where:

-	CEUE represents the CE of the UE in the evaluation period, i.e. of the smartphone. This release of the study this contribution is considered outside the scope of this SID.

-	CEUE,gNB represents the CE of the gNB(s) due to the traffic and signaling of the UE in the evaluation period.

-	CEUE,UPF represents the CE of the UPF(s) due to the traffic and signaling of the UE in the evaluation period.

-	CEUE,TNGF represents the CE of the TNGF(s) due to the traffic and signaling of the UE in the evaluation period.

-	CEUE,N3IWF represents the CE of the N3IWF(s) due to the traffic and signaling of the UE in the evaluation period.

-	CEUE,NF represents the CE of the CN NF(s), e.g. AMF, SMF, PDF, UDM, etc which controls the UE and performs the signaling of the UE in the evaluation period.

-	CEUE,4G represents the CE of the 4G Network where the UE can be connected during the evaluation period.

-	CEUE,NTN represents the CE of the NTN RAT and CN part where the UE can be connected during the evaluation period.

-	CEUE,Transport represents the CE of the backhaul network involved in the transport of the mobile network traffic in the evaluation period.

In this clause we do not intend to duplicate the description of the calculation of KPI as defined in clause 6.y3 per the per UE granularity, in clause 6.y4 for the EC per PDU session granularity and in clause 6.y.5 for the EC per QoS flow . As explanation we refer to the case of evaluation of Carbon emission per UFP per UE granularity

The CE contribution of UPF j to UE traffic is proportional to the total Carbon emission of the UPF j ECUPF.

CEUE,UPFj=fx,y,..*CEUPFj

where

Fx,y=FUE Data Volume,  UPF Total Data Volume== UE Data VolumeUPF Total Data Volume

An alternative approach can be based on the Carbon Factor which represents the amount of Carbon Emission for unity of Energy for the production of a given mix of electric power. In this case the carbon emission can be obtained as

CEUE,UPFj=Cf*ECUE, UPFj

Where

- ECUE, UPFj is the EC of UPF per UE evaluated as defined in clause 6.y.3.3

- Cf is the Carbon intensity related to the production of the electric energy used by UPFj provided by OAM

The evaluation of the other contributions per different granularity can be obtained in the same manner.

### 6.y.8 Renewable Ratio KPI

The Renewable Ratio represents the ratio of power supply which is generated by renewable energy and by source not renewable.  The solution assumes that Renewable Ratio is provided by OAM per each NFs and per each physical node

The Renewable Energy ratio is a measurement of the ratio of source used for generate the electric power used in a physical location, therefore it does not change with the energy consumed, since it can be assumed that the ratio stay constant over a period of time.

The Ratio in reality may change every period due to the change of mix of energy used during the different time window or during the period of day, e.g.  no solar power in during the night or in cloudy days. Generally speaking we can imagine different scenario from a ratio which is result of an averaging of previous period or based on averaging due to the contract between the operator and the power supplier. Now also assuming the most variable condition , we can assumed that in the measurement period the Renewable energy ratio will corresponds to a value, e.g. 20% of Renewable in 15 Minutes or in 1 hours. Now this ratio is not influenced by the EC of the UE or by the traffic of the UE 1 and UE2, it is still the 20%, what is change is the absolute number of the Energy consumption produced with Renewable wource.

Let assume that we the following scenario:

-	UE1 consumes 10 Mbit of data

-	UE2 consumes 500 Mbit of data

-	The EC of the UPF is 300 Joule and the total data volume is 2500 Mbits

-	Renewable energy ratio is 20%

The result using the estimation defined in in clause 6.3.3 are:

-	EC of UE1 is 1.2 J  ( 10/2500*300 J)

-	EC of  UE2 is 6 J (500/2500*300 J)

-	Renewable ratio for UE1 is 20%  since it is percentage of mix of energy used to produce the 300J consumed by the UPF

- 	Renewable ratio for UE2 is 20% since it is percentage of mix of energy used to produce the 300J consumed by the UPF

Therefore to make a proportion with the data volume is logically incorrect . The correct evaluation is to estimate the absolute value of energy consumption due to renewable and due to non-renewable which is

- EC due to Non-renewable of UE1 = 80%*1.2 J = 0.96 J , EC due to Renewable of UE1 = 20%*1.2 J =0.24 J

- EC due to Non-renewable of UE2 = 80%*6 J = 4.8 J , EC due to Renewable of UE1 = 20%*6 J=1.2 J

It shall be noted the absolute number is different, but the percentage is the same. Therefore, the effect may only be measured when the on a billing period the different UE make use of different mixing vue to usage of network in different location. Unfortunately, it shall be noted that this is not influenceable by user which consume what is consume based on operator decision of which is the power mix used in the location where the UE is using the resources that operator has decided to assigned to him.

### 6.x.9	Conclusion

The KPIs described in this solution need to be evaluated in the relevant NFs which is responsible for the evaluation. The scope of the solution to propose a set of KPIs independently by which will be the NF which will do the evaluation. Furthermore the KPI provide an indication of which should be the input information required by such NF in order to perform the evaluation.

### 6.x.10	Impacts on existing services, entities and interfaces

The KPI described in this solution does not have an impact per se, since the impact is on the NF where the evaluation will be performed. Furthermore, the impact is on the information which needs to be provided to this NF where the evaluation needs to be performed. All these impacts are related and described as part of the solution where the evaluation is proposed to be performed.

# * * * * End of changes * * * *