3GPP TSG-WG SA2 Meeting #162	S2-2404843

Changsha, Hunan Province, China, April 15-19, 2024

Source:	Ericsson

Title:	KI#3: Updating Solution#9 to remove some ENs

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes updates to Solution#9 tackling KI#3 on NTZ to address some of the ENs, the update relies on the LS Reply from TSG RAN (RP-240810).

# 1.	Discussion

Solution#9 contains a number of editor’s notes, and some of them are expected to be address based on Reply LS-es from RAN WGs and ETSI that have been sent earlier (see S2-2403859) while the others can be removed by on the relevant assumption.

TSG RAN has provided the Reply LS (see RP-240810) with the following content:

RAN would like to thank SA2 for their LS RP-240031 on NTZ. RAN has discussed the LS and has provided some clarifications and answers to your questions. In general, RAN would like to clarify that the operation on No-Transmit Zone was not in scope Rel-18 UAV work. Furthermore, currently RAN doesn’t have a Rel-19 WI on UAV or on NTZ’s operation. Approval of new Rel-19 RAN work are planned to be discussed in RAN105, at which point a new Rel-19 WI/SI NTZ operation may be discussed. Therefore, RAN hasn’t yet discussed any technical details or reached consensus on technical issues and requirements. The answers to SA2 questions are found below: Question 1: - As per the ECC ruling, UAV UE(s) need to comply with NTZ restrictions (i.e. no-transmit zones for spectrum compatibility purposes when aerial UE (UAV) operating in the relevant frequency bands) based on its location in and around any NTZ-applicable area. What is the expectation/assumption, if any, RAN may have regarding per UE level or RAN node level NTZ information needed to enforce/apply NTZ? Answer 1: As mentioned, RAN doesn’t have a WI approved for this topic to discuss and has not yet discussed technical details and requirements. Therefore, at this point in time RAN doesn’t have any expectations/assumptions. From RAN perspective, it needs to be further clarified whether and how (if necessary) RAN needs to be involved in enforcing the regulation and whether it is RAN’s responsibility to enforce/apply NTZ regulation. Question 2: - Is RAN WG(s) planning to consider potentially how to restrict UAV UE’s initial cell connection if a cell is operating in the NTZ area (e.g. for Pre-Rel19 UAV UEs, and Rel-19 UAV UEs which do not have the latest/updated NTZ information)? Answer 2: Even though there is no Rel-19 WID plan on NTZ, RAN assumes the delivery of the NTZ information can be done via mechanisms outside the scope of RAN for Pre-Rel-19 aerial UEs. RAN assumes that if the UE is expected to follow the regulation, the UE shall ensure it does not operate without valid NTZ information. Question 3: - Is RAN WG(s) planning to investigate what and if any kind of information may be needed from 5GC to enable any such control? Answer 3: As previously discussed, RAN doesn’t have an agreed plan on what would need to be investigated and what kind of information may be needed to enable such control. The requirement on what type of control and from what entity would need to be further clarified. Question 4: - Is RAN WG(s) planning to implement any reporting of spurious UAV UE (those who do not follow frequency restrictions)? Can SA2 assume that RAN have ability of height detection mechanism to know at which height the UAV UE is operating, and if a certain frequency is restricted at a certain height, it can be reported to the core network or UAV UE. Answer 4: Since we haven’t started any Rel-19 WI we do not have an agreed plan or consensus whether there will be any reporting of spurious UAV UEs. Regarding ability of height detection mechanism, from Rel-18, a UAV UE is supposed to know its height. However, the RAN Nodes have limited knowledge in time and space on UE’s height and/or position, and such knowledge depend on UE reporting and supported capabilities. In general, as of Rel-18 RAN doesn’t have enough information to enforce/apply NTZs to UAV UEs, and would like to further understand the requirements. If RAN will be asked to enforce/apply these restrictions, the feasibility and complexity of solution will need to be further discussed in RAN WGs.

Based on the received reply and considering that RAN hasn’t yet discussed any technical details or reached consensus on technical issues and requirements and will not do until/if Rel-19 WI/SI NTZ operation is approved, some of the ENs are addressed in the following matter.

Editor's note:	Additional/what parameters need to be considered to properly describe NTZ and actual restrictions/enforcement aspects and roles of NTZ in the core and radio network are FFS.

Consideration: RAN has replied that “…currently RAN doesn’t have a Rel-19 WI on UAV or on NTZ’s operation…Therefore, RAN hasn’t yet discussed any technical details or reached consensus on technical issues and requirements.” Hence, SA2 needs to make an assumption. The current solution considers that NTZs can be described using parameters such as frequency, latitude and longitude, altitude. The roles of 5GC NFs (NG-RAN, AMF, UDM, NEF etc.) is also currently identified and described in the solution. Hence, any additional parameters or aspects can be included at the normative phase and for instance based on RAN work progress. Therefore, the proposal is to add a note about that.

Editor's note:	Coordination with RAN WGs are required to progress the solution to ensure that RAN (eNB/gNB) has inputs required to enforce UE compliance with NTZ.

Consideration: Once RAN WGs start the work on this, they can inform SA2 (via an LS for instance) and SA2 will consider any additional parameter/input requested by the RAN. Furthermore, any additional parameter that SA2 considers to be required by RAN for this purpose can be identified and included during the normative phase. The proposal is to capture this in a note.

Editor's note:	Whether to use the existing service operation (e.g. Naf_Authentication_Notification) or to design a new for this specific purpose is FFS.

Consideration: This EN applies to Option#1 (node-level signalling) for NTZ information delivery to RAN. During the conclusion phase, one or two/three options might be adopted. If Option#1 is selected as the way forward, the exact service operation can be identified at the normative phase, although it now seems that to reuse “Naf_Authentication_Notification request” is a good option. Therefore, the proposal is to update Step 1 description in order to use the Naf_Authentication_Notification request as an example.

Editor's note:	What actions and how RAN uses the delivered NTZ information is up to RAN WGs to study, and it is FFS.

Consideration: This EN does not impact SA2 solution since RAN WGs will consider a new Rel-19 WI/SI NTZ operation and work on it if agreed. Such EN does not contribute to any aspect to the current SA2 solution and therefore can be removed if a note is added clarifying the use of NTZ information is expected to be handled by RAN WGs. Especially, since TSG RAN confirmed in their reply that if required they will study solutions to fulfill NTZ requirements: “If RAN will be asked to enforce/apply these restrictions, the feasibility and complexity of solution will need to be further discussed in RAN WGs.”

Editor's note:	It is FFS whether and how (e.g. based on which information) the indication to enforce NTZ is provided by AMF to RAN.

Consideration: This EN applies to Option#1 (node-level signalling) for NRZ information delivery to RAN. For this purpose, it is proposed to use an N2 message from AMF to all relevant NG-RAN nodes, however, whether the presence of NRZ information is sufficient for an NG-RAN node to determine that it now needs to enforce the NTZ or an additional indication is required/beneficial to be included is the normative work. Therefore, the proposal is to add a note about this aspect.

Editor's note:	It is FFS if any additional parameter(s) is needed or existing subscription is sufficient.

Consideration: This EN applies to UE’s subscription data at UDM. The solution proposes to include an indication that the UE/UAV is compliant with the NTZ requirements. However, during the normative phase there might be the ways or parameters included in the subscription, therefore, the proposal is to add a note that exact parameters can be determined at the normative phase and remove the EN.

# 2.	Text proposal

It is proposed to agree the following changes to 23.700-59:

>>>> Start of Changes <<<<

## 6.9	Solution #9: Enabling NTZ support for aerial UEs

### 6.9.1	Key Issue mapping

This solution addresses KI#3 aspects.

### 6.9.2	Description

The solution assumes that an aerial UE indicates its capability to respect no-transmit zones (NTZ), and UE's subscription data has a record about that, as Operators have to enforce the NTZ respect, and the subscription-based approach is one possibility, as described below.

Several options/alternatives are considered when comes to obtaining NTZ information - preconfiguration at the relevant network nodes (e.g. gNodeB (gNB)/eNodeB(eNB), AMF/MME) as well as a network-based configuration of NTZ parameters (e.g. geographical area in form of coordinates (i.e. latitude and longitude), restricted frequency band(s), altitude/elevation etc.).

NOTE X:	Additional parameters, if any, to properly describe NTZs can be included during the normative phase.

In order to perform network-based configuration of NTZ parameters, it is assumed that there needs to be operator's AF with trust relation for this purpose. This AF can be part of UTM (Uncrewed Aerial System Traffic Management) , which is outside operator's trust domain, and therefore, the AF will invoke a service operation towards an UAS NF / NEF so that the NTZ information can be transferred towards RAN nodes (i.e. gNBs in case of 5GC and eNB in case of EPC), core network (CN) entities and to the affected aerial UEs.

Editor's note:	Whether UEs in NTZs are allowed to use UL for emergency support is FFS and depends on LS reply to S2-2403859.

Current solution assumes that the aerial UEs supporting NTZ will not require emergency services support.

Until otherwise indicated, it is assumed that the NTZ does not allow UEs to send uplink data but allows reception of downlink data from the network. Also, Aerial UEs within NTZ(s) are able to communicate with the network for mobility/registration procedures for the purposes of keeping connectivity to the network.

The main principles/steps for the considered solution options/alternatives.

1.	Provisioning RAN (gNB/eNB) with a set of NTZ information (e.g. geographical area in form of coordinates (i.e. latitude and longitude), restricted frequency band(s), altitude/elevation etc.), see Figure 6.9.3-1:

1(a).	RAN is provided with NTZ information via a node-level signalling from operator's AF via UAS NF/NEF, PCF and AMF using the AM Policy Association Establishment/Modification procedures (as described in clauses 4.16.1 and 4.16.2 of TS 23.502 [4]).

1(b).	RAN node(s) are configured by OAM with the existing NTZ information that can be requested by the AF/UTM from network's nodes in the area they serve (e.g. based on RAN node location, Tracking Area(s) identified by a list of tracking area identities (TAI) or a list of cell identities)

1(c):	AMF/MME is pre-configured (e.g. local configuration) with NTZ information; and AMF/MME provides the NTZ information to relevant gNB/eNB nodes via non-UE associated signalling (e.g. during NG interface Setup/Reconfiguration procedure), UE-associated signalling (e.g. during Initial Context Setup/Modification procedure or PDU session resource management procedures).

To provide RAN nodes with new/updated NTZ information from the AF/UTM (i.e. triggered by UTM), a node-level signalling from operator's AF/UTM can be used for Option 1(a), 1(b) and 1(c), whereas reprovisioning via OAM can be done only for Option 1(b).

2.	UE's subscription data include an additional record at UDM, indicating that the UE/UAV is compliant to NTZs requirements. This allows the operator and the network to enforce NTZ respect and to deliver the NTZ information only to UEs with the corresponding subscription. This indication is made available/delivered to RAN nodes (gNB/eNB) so they can enforce the NTZ respect. Additionally, UE's subscription data at UDM can indicate which set(s) of NTZ are allowed to be disobeyed by high-priority UEs (e.g. first responders).

3.	Providing the NTZ information to a UE with the corresponding subscription:

3(a).	RAN (gNB/eNB) sends the NTZ information to a UE using the RAN defined procedure.

3(b).	The NTZ information is sent to a UE from a core network node (e.g. UAS NF/NEF via an AMF) or from the serving AMF using Non-Access Stratum (NAS) signalling during UE's registration in a Registration Accept message.

The UE/UAV stores the received NTZ information until the new NTZ information is provided or deleted, e.g. by explicit signalling from the core network (for instance, via AMF NAS signalling) due to request from the AF/UTM.

Editor's note:	It is FFS if SMF/UPF/PGW-U need to be impacted due to enforcement of no UL data transmission and Aerial UE movement in and out of NTZ(s).

NOTE X1:	Any additional inputs that RAN WGs will be required to progress their work can be considered by SA2 once RAN WGs achieve progress in this work can be provided at the normative phase or via incoming/outgoing LS-es.

### 6.9.3	Procedures

The main procedural steps to provision RAN nodes (gNB/eNB) with the NTZ information is shown in Figure 6.9.3-1.

Editor's note:	The current procedures assume 5GC signalling. The adaptation for EPC is FFS.

Figure 6.9.3-1: Procedure to provision RAN nodes (gNB/eNB) with the NTZ information

Option 1: node-level signalling.

1.	An AF (UTM) sends to the UAS NF/NEF a request (e.g. an Naf_Authentication_Notification request) to provide new/updated information about NTZ for the UE/UAV. The AF/UTM includes GPSI, CAA-Level UAV ID, PDU Session IP address if available and the NTZ information in the re-authentication/authentication data update request.

NOTE 1:	The similar request can be used by the AF/UTM to send an update about the NTZ information when e.g. the AMF/MME is preconfigured with the set of NTZ information or when the previously delivered information is not valid anymore.

2.	The UAS NF/NEF translates the AF-provided NTZ information (e.g. geographical area in form of coordinates (i.e. latitude and longitude), restricted frequency band(s), altitude/elevation etc) into 3GPP identifiers, e.g. a list of Tracking Area Identifiers (TAIs) or a list of cell IDs, RFSP Index.

3A or 3B.	The UAS NF/NEF discovers a PCF handling AM Policy for the UE (Option 3A) or a serving AMF (Option 3B); for that the NEF invokes the NRF discovery service and uses UE's identity.

3C.	The UAS NF/NEF uses UDM service (Nudm_UECM_Get operation as specified in clause 5.2.3.2.4 of TS 23.502 [4]) to get an NF ID of the AMF serving the UE, i.e. with UE Context. In congestion with that, the UAS NF may invoke another service operation to UDM to retrieve UE's subscription data and check whether the UE is compliant to NTZs requirements allowing the operator and the network to enforce NTZs.

If the UE's subscription does not have the NTZ indication, the UAS NF notifies the AF/UTM by sending the response message and include the information about the results inside this message.

NOTE 2:	It is up to UTM to decide for which purpose this information can be used; it is outside 3GPP scope.

4A.	The UAS NF/NEF sends to the discovered PCF an Npcf_AMPolicyAuthorization_Create/Update request containing the translated NTZ information, e.g. a list of TAs/Cell IDs, and the original AF-provided NTZ information (prior the translation by the NEF).

5A.	After receiving the NTZ information, the PCF takes policy decision and then may initiate the AM Policy Association Modification procedure for the UE/UAV as described in clause 4.16.2.2 of TS 23.502 [4] to provide the AMF with the NTZ information.

The PCF does not initiate the AM Policy Associations Modification and rejects (i.e. responds with the failure cause) the Npcf_AMPolicyAuthorization_Create/Update request in cases when the PCF does not receive information about the UE's capability to respect NTZ during the AM Policy Association Establishment (see step 16 in clause 4.2.2.2.2 of TS 23.502 [4]). The PCF responds to the Npcf_AMPolicyAuthorization_Create/Update request (not shown in Figure 6.9.3-1), and the UAS NF/NEF responds to the AF (using Naf_Authentication_Notification response, not shown in Figure 6.X.3-1) to inform the UTM that the UE/UAV does not have capability required.

4B and 4C.	If the UE's subscription data contains the indication that the UE is compliant with NTZ requirements, the UAS NF/NEF sends an Nnef_Authentication_Notification request to the AMF/MME, containing the translated NTZ information, e.g. a list of TAs/Cell IDs, the original AF-provided NTZ information to the target AMF. If the UE subscription does not indicate the NTZ compliance, the UAS NF/NEF responds to the AF/UTM to notify that the UE/UAV does not have capability required.

6A and 6B.	If the UE subscription check (i.e. compliance to NTZ requirements) has not been performed earlier in the procedure, the AMF/MME checks, based on the subscription data retrieved during the registration, whether the UE is compliant with NTZ requirements.

7.	Based on the NTZ information, the AMF determine the relevant RAN node IDs and then sends N2 messages containing the NTZ information and include an indication to enforce (whenever required NTZ respect) to all applicable RAN nodes.

NOTE 3:	It is expected that RAN WGs exploit the delivered NRZ information to study potential solutions to enforce/apply NTZ restrictions, for instance, RAN can use the NTZ information to adjust information broadcasting, e.g. prevent UEs from moving from RRC_IDLE/INACTIVE states to RRC_CONNECTED state.

Option 2: OAM configuration.

1.	RAN node(s) are configured by OAM with the existing NTZ information that can be requested by the AF/UTM from network's nodes in the area they serve (e.g. based on RAN node location, Tracking Area(s) identified by a list of tracking area identities (TAI) or a list of cell identities).

2.	AMF sends an N2 message with indication that the RAN needs to enforce NTZ after the Registration Completion.

Option 3: AMF preconfiguration with NTZ information.

0.	AMF is preconfigured with sets of NTZ information that can be requested by the AF/UTM.

1.	To request a specific NTZ requirements, the AF/UTM may use e.g. an index (a number) to a specific set; Steps (some or all), 3B/C, 4B/C, 6A, and 7 as in Option 1 are executed.

2.	Delivery of the NTZ information together with the indication to enforce NTZ during NG interface Setup/Reconfiguration procedure, during Initial Context Setup/Modification procedure or PDU session resource management procedures.

The main procedural steps to provide UEs with the NTZ information is shown in Figure 6.9.3-2.

Figure 6.9.3-2: Procedure to providing UEs with the NTZ information

Option 1: RAN Mechanism

1.	UE preforms registration procedure as described in clause 4.2.2.2.2 of TS 23.502 [4].

2.	AMF retrieves the subscription data from the UDM and checks whether the UE is compliant with NTZ requirements. If so, the AMF/MME sends N2 message with the indication to RAN that it shall enforce the NTZ whenever required.

3.	RAN sends the received NTZ information, how this is done is FFS and need coordination with RAN WGs.

Option 2: Registration Accept

1.	UE to RAN: UEs sends a Registration Request message and include parameters as specified in clause 4.2.2.2.2 of TS 23.502 [4]. Additionally, if the UE has capabilities to respect no-transmit zones, the UE includes the indication about this capability to the network.

2.	RAN to AMF: Once RAN selects an AMF, the RAN sends an N2 message containing N2 parameters (specified in step 3 of clause 4.2.2.2.2 of TS 23.502 [4]) and the received Registration Request message from the RAN (as described in step 1). The AMF stores information UE's support for NTZ consideration in the UE Context.

3.	If the AMF sees in the registration request information about UEs capability to respect NTZ, the AMF check whether UE subscription data includes indication about UEs compliance with NTZ requirements. If the UE is compliant with NTZs, the AMF may verify the UE's location before replying with a Registration Accept/Reject message.

4.	If the UE is allowed to operate at its present location, the AMF sends to the UE a Registration Accept message and includes inside the message the previously received NTZ information.

If the UE is not allowed to operate at its present location, the AMF may either: (1) send a Registration Reject message with a cause value indicating that the UE is not allowed to operate at the present UE location, or alternatively, (2) send a Registration Accept message in which the AMF includes a UE Radio Capability ID and/or RFSP Index the UE is allowed to operate.

5.	If the UE receives a Registration Reject message with cause value indicating the UE/UAV is not allowed to operate in the present location due to NTZ requirements, the UE may attempt to perform an Emergency Registration.

The overall NTZ procedure is as shown in Figure 6.9.3-3, and it includes:

Figure 6.9.3-3: Procedure to support UE compliance with the provided NTZ information

1.	NTZ information and the indication to enforce NTZ delivery to the relevant RAN nodes as described in Figure 6.9.3-1.

2.	NTZ information delivery to UEs with the corresponding subscription, as described in Figure 6.9.3-2.

3.	UE location determination. As an option, and depending on UTM requirements, the AMF may use the UE mobility event notifications to get information about UE's presence in Area(s) of Interest, as specified in clause 5.3.4.4 of TS 23.501 [5].

4.	In case of UE's presence in the AoI: once the AMF detects the UE's presence in the NTZ, the AMF sends an N2 message with a new value of 'Index to RAT/Frequency Selection Priority' (RFSP Index) to RAN nodes. Based on the received RFSP Index, the NG-RAN nodes decide about redirecting UEs to different frequency layers or RATs (as specified in clause 5.3.4.3.1 of TS 23.501 [5]).

Editor's note:	How to detect UE transmit in NTZ and how to handle corresponding UL traffic is FFS.

5.	If there is any change in the provisioned NTZ information, the RAN nodes and UEs need to be re-provisioned with the newest NTZ information.

6.	For UEs/UAV in CM_CONNECT with RRC_INACTIVE state or in CM_IDLE, the AMF may initiate, based on the local policy, the Network Triggered Service Request procedure as described in clause 4.2.3.3 of TS 23.502 [4] before executing steps 3-5. If the UE is not updated while it was in IDLE, the AMF updates the UE next time it becomes available gain (e.g. RRC_CONNECTED).

### 6.9.4	Impacts on services, entities and interfaces

Impact depends on which options/alternatives selected for delivering NTZ information to UEs and RAN nodes and for updating them about any change from the AF/UTM. Overall, the impacts could be the following:

AF:

-	Invoking a Naf_Authentication_Notification or a new service to request NTZ compliance from the network,

-	Receiving notifications when UEs enter or disobey NTZ requirements.

UAS NF/NEF:

-	Discovering a PCF handling AM policy,

-	Discovering AMF serving the UE,

-	Retrieving information about UE's subscription and checking whether the UE can comply with NTZ,

-	Translating the NTZ information to 3GPP identifiers such as TA, NG-RAN node identifiers, cell IDs.

PCF:

-	Invoking AM Policy Association Modification due to NTZ.

AMF:

-	Determining UE's location/presence in the area of interest,

-	Checking UE's subscription for NTZ compliance,

-	Informing PCF about UE's capability to respect NTZ,

-	Informing RAN nodes about enforcing NTZ requirements,

NOTE X2:	Any additional indication, if any, need to be provided via node-level signalling to RAN nodes from 5GC can be determined during the normative phase or based on RAN WGs request.

-	Informing PCF about UE's capability to respect NTZ,

-	Sending notifications when UE enters the NTZ.

RAN:

-	delivering NTZ information to the relevant UEs,

-	Enforcing NTZ compliance.

UE/UAV:

-	respecting NTZ requirements,

-	announces its capability to comply with NTZ requirements.

UPF:

-	Handling (e.g. discard, etc.) UL data traffic for UEs in NTZ (FFS if/when needed).

UDM:

-	Inclusion of NTZ enforcement indication and provide to AMF.

NOTE X3:	During the normative phase it can be determined whether an explicit indication that a UE is compliant with NTZ requirements need to be included among UE subscription data or implicit indication is sufficient. Any additional parameters, if any, can be considered to extend UE subscription data in order to address NTZ requirements.

>>>> End of Changes <<<<