3GPP TSG-WG SA2 Meeting #162	S2-2404922

Changsha, China, April 15th – 19th, 2024	(revision of S2-240xxxx)

Title:	[DRAFT] LS reply on PDN connectivity request for UAS services

Response to:	LS (S2-2403874/C1-241837) on PDN connectivity request for UAS services from CT1

Release:	Release 18

Work Item:	UAS_Ph2

Source:	[Huawei to be] SA2

To:	CT1

Cc:

Contact Person:

Name:	Runze Zhou

Tel. Number:

E-mail Address:	zhourunze AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

SA2 thanks CT1 for the LS on PDN connectivity request for UAS services.

Regarding CT1 question “whether the UE can establish the PDN connection for UAS service after the completion of the attach procedure during which the UUAA-SM was successfully performed.”, SA2 understanding is:

- During the attach procedure, UE may include the Service Level Device Identity (i.e. the CAA-Level UAV ID of the UAV) in the Attach Request message, if the UE intends to perform UAV operation. The procedure is described as in clause 5.2.3.3, TS 23.256

- However, if the UE does not request UAV operation during the attach procedure, it may also trigger to establish PDN connectivity any time after the completion of the attach procedure. In this case, UE initiates a PDN Connectivity Request by including the Service Level Device Identity (CAA-Level-UAV ID) , and optionally a dedicated APN/DNN for UAS service. Based on the received parameters, the SMF+PGW-C retrieves the Session Management Subscription Data from the UDM+HSS using the Nudm_SDM_Get service operation, and based on that the provided APN/DNN is dedicated for aerial services (have aerial service indicator set), it determines to invoke UAS NF/NEF service operation for UUAA Authentication/Authorization. If the provided APN/DNN is dedicated for aerial services but Service Level Device Identity (CAA-Level UAV ID) is not provided, the SMF+PGW-C shall reject the establishment of the PDU Session.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks CT1 take the above information in the future work.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #163		27th May – 31st May, 2024			Jeju, KR

TSG-SA2 Meeting #164		19th – 23rd August, 2024			Maastricht, NL