SA WG2 Meeting #162	S2-2404989

Changsha, China, April 15 –19, 2024	(was S2-2402944)

Source:	CableLabs

Title:	KI#4: New Sol: Policy Control of individual non-3GPP devices behind a UE/5G-RG

Document for:	Approval

Agenda Item:	19.8

Work Item / Release: 	FS_UIA_ARC / Rel-19

Abstract: This paper proposes a solution for KI #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG.

# Introduction

TR 23.700-32 includes agreed KI#4 as follows:

## 5.4	Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

### 5.4.1	Description

This key issue will study whether and how 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG and whether and how to provide policy control for the traffic associated with individual non-3GPP devices.

The use case that is associated with this key issue is the case where non-3GPP devices behind a UE or 5G-RG need to be identified.

The objective of this key issue is how an identifier is used by the network to control the traffic to/from UE or 5G-RG when the traffic is associated with the non-3GPP devices. This objective differs from existing support for AUN3 devices in TS 23.316 [7] because the objective is to enable the non-3GPP devices to be identified and to use only the subscription of the UE or 5G-RG to access the 5GC (i.e. the UE or 5G-RG should have to maintain only a NAS Context itself and not for each non-3GPP device).

Solutions to this key issue will address:

-	whether and how the 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG. (e.g. in order to charge the individual non-3GPP devices),

-	whether and how to provide policy control for the traffic of individual non-3GPP devices connecting behind a UE or 5G-RG. Including whether and how to trigger policy control for the traffic of individual non-3GPP devices via PCF and NEF APIs, including whether and how to support concurrent services with differing QoS requirements launched by the different non-3GPP devices, and

-	whether and how the operator can optionally restrict the number of simultaneously active User Identifiers per UE or 5G-RG.

NOTE 0: 	The User Identifiers in the above bullet means the identifiers of the devices behind gateway UE or 5G-RG.

NOTE 1:	Changes to the layer 1 or layer 2 protocols of non-3GPP devices are not in scope of this study. It is assumed that the non-3GPP devices do not support 5G-AKA authentication procedures nor separate NAS connections with the 5GC for each non-3GPP device (e.g. like for AUN3 devices).

NOTE 2:	Conclusions related to an 5G-RG should be shared with the Broadband Forum (BBF) and CableLabs.

NOTE 3:	Solutions for Ethernet PDU Session should work in the presence of randomized MAC addresses.

# 2. Proposal

This contribution proposes to include Solution X in TR 23.700-32 clause 6.X and to update Table 6.0-1 accordingly.

Start of changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.101: "Service aspects; Service principles".

[3]	3GPP TS 22.115: "Service aspects; Charging and billing".

[4]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[8]	3GPP TR 22.904: "Study on user centric identifiers and authentication (Release 16)".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[x]	Liaison Statement from IEEE 802.11 Working Group to Wireless Broadband Alliance, https://www.ieee802.org/11/Liaisons/2024-01-04-Liaison%20statement%20from%20IEEE%20802-11%20Working%20Group%20to%20WBA%20re%20P802.11bh.pdf

Next change

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> |
| #1 | X | X | X |  |
| #2 | X |  |  |  |
| #3 | X | X |  |  |
| #4 | X |  |  |  |
| #5 | X | X |  |  |
| #6 | X |  |  |  |
| #7 | X |  |  |  |
| #8 |  | X |  |  |
| #9 |  | X |  |  |
| #10 |  | X | X |  |
| #11 |  | X | X |  |
| #12 |  | X | X |  |
| #13 |  | X |  |  |
| #14 |  | X |  |  |
| #15 |  |  | X |  |
| #16 |  |  | X |  |
| #X |  |  |  | X |

Next change (all new text)

## 6.X	Solution #X: RG-initiated PDU Modification to support differentiated QoS for device behind RG

### 6.X.1	Key Issue mapping

This solution addresses Key Issue #4 on how to provide policy control of individual non-3GPP device behind a UE or 5G-RG.

5GC may have information on a QoS policy applied to a non-3GPP device behind a 5G-RG or UE where a User Identifier is used between 5GC and the 5G-RG/UE to identify the QoS policy, and maps that policy to non-3GPP access between the non-3GPP device and the 5G-RG/UE. 5GC may store the QoS policy based on network exposure with an external operator portal or AF.

### 6.X.2	Description

### Rel-18 WWC study/work item identified and specified a solution to handle non-3GPP devices behind 5G-RG based on Connectivity Group IDs, where a Connectivity Group ID is applied based on a SSID or Ethernet port. Typically, there are only a few SSIDs available (e.g., SSID#1 for home devices and SSID#2 for guest devices), thereby leading to certain limitations in Rel-18 features to allow providing different QoS/parental control over devices sharing a same SSID.

To provide differentiated QoS provisioning for non-3GPP devices behind a 5G-RG/UE, 5G-RG/UE should be able to identify individual non-3GPP devices as well as 5GC should be able to provision separately for individual non-3GPP device. To allow separate provision by 5GC for each non-3GPP device, a User Identifier (for non-3GPP devices) is necessary, where the User Identifier may be different from a MAC address of the non-3GPP device. Various known implementation techniques such those based on OpenWrt or CableLabs Micronets could be considered for a User Identifier/identification. In recent progress in IEEE 802.11bh working group (D1.0 draft) [x] to support various use cases with randomized MAC addresses, IEEE identified two solutions, namely, device ID and identifiable random MAC address (IRM). Using device ID mechanism, a 5G-RG or UE may provide an identifier to a non-3GPP device that may be used by the non-3GPP device where the device ID is constant in ESS regardless of the MAC address used by the non-3GPP device. Using IRM, a non-3GPP device may provide a random MAC address to a 5G-RG or a UE where the IRM MAC address will be used as transmitted address. For example, when the device ID mechanism is used, the device ID may be used as a User Identifier of the non-3GPP device. When IRM is used, the operator may assign a User Identifier of the non-3GPP device that is associated with a current IRM address and is updated by the 5G-RG or the UE when changes occur. Another example of a User Identifier is based on EAP credentials if a non-3GPP device supports EAP authentication, where EAP authentication may be performed outside of 5GC.

Such a User Identifier known to a 5G-RG or UE will be mapped to the local non-3GPP device information (e.g., device ID or current MAC address by 802.11bh, or credentials used in WLAN) by the 5G-RG or UE, though the exact mechanisms of how the 5G-RG maps the User Identifier to local non-3GPP device information would be implementation-specific or operator-choice, which are outside of 3GPP scope.

To support this, however, the 5G-RG is pre-configured with or receives policy from the 5GC on QoS policy for non-3GPP devices, where the QoS policy for each non-3GPP device has the following information:

- QoS parameters for the device (e.g., bit rate requirements)

- User Identifier

Based on non-3GPP device identification by the 5G-RG or UE, a separate QoS flow can be configured for each non-3GPP device. Based on Rel-18 feature of Connectivity Group IDs, a group of non-3GPP devices may share a same connectivity group e.g., based on SSID. The 5G-RG may establish a separate PDU session for each connectivity group. Each device in each connectivity group may be further differentiated based on separate QoS flow as shown in Figure 6.X.2-1.

Figure 6.X.2-1: Differentiated QoS for each non-3GPP device

### 6.X.3	Procedures

6.X.3.1	Procedures for provisioning of non-3GPP device information

Fig. 6.X.3.1-1 Provisioning of non-3GPP device information

Steps 1-4 follow Rel-18 WWC procedures for NAUN3 devices as specified in TS 23.316 [7] –

- 5G-RG registers to 5GC as specified in clause 7.2.1.1 of TS 23.316 [7].

- The non-3GPP device establishes an L2 connection (e.g., WLAN connection) with the 5G-RG.

- The non-3GPP device is authenticated via local authentication (e.g., PKS). The non-3GPP device may be authenticated using EAP authentication via external EAP-server depending on the device capability.

During steps 2-3, 5G-RG maps the non-3GPP local information (e.g., a current MAC address) to the User Identifier.

- A PDU Session with a default QoS flow is established based on the URSP rule matching a Connectivity Group as specified in clause 4.10b of TS 23.316 [7].

- The admin of the 5G-RG subscription registers a policy for the non-3GPP device to an operator portal (e.g., via AF, register parental control for kids iPad). During the policy registration, a User Identifier of the non-3GPP device is determined (e.g., either by assigning by operator portal or 5G-RG provides the User Identifier) and stored in a 5G-RG associated with the non-3GPP device. For example, based on a device ID technique, the non-3GPP device and the 5G-RG use the device ID which maps to the User Identifier in the operator portal for future communications. In case IRM is used, the 5G-RG maintains a mapping of random MAC address to User Identifier, so that when the non-3GPP device uses a random MAC address, the 5G-RG identifies the non-3GPP device and maps it to the User Identifier. The 5G-RG information would be known to the operator portal (e.g., 5G-RG GPSI is known to the operator portal).

- Upon creating a policy for the non-3GPP device, the AF (operator portal) updates the policy associated with the 5G-RG based on the policy configuration via NEF procedure (e.g., extending Service specific parameter procedure in clause 4.15.6.7 of TS 23.502 [5] for non-3GPP policy information). The NEF uses Nudr_DM _Create/Update/Delete service operation to update parameters.

6.X.3.2	Procedures for provisioning of non-3GPP device information

Fig. 6.X.3.2-1 Policy control for non-3GPP device

During Steps 1-2, 5G-RG detects the non-3GPP device with the User Identifier is connected. For example, 5G-RG can detect the User Identifier during the Wi-Fi association or authentication procedure.

3-7. PDU session modification procedure based on clause 4.3.2.2 of TS 23.502 [5] with addition of indication of User Identifier and device info (such as IP address of the device) to apply appropriate QoS flow to the non-3GPP device. 5G-RG uses the User Identifier and device info of the non-3GPP device to map traffic to corresponding QoS Flow.

### 6.X.4	Impacts on services, entities and interfaces

NEF:

- Extensions to the NEF Service Parameter Service to allow an AF to provision non-3GPP device information.

UDR:

- Extensions to the Application Data in UDR to store non-3GPP device information (e.g., User Identifier, policy for non-3GPP).

PCF:

- Ability to retrieve non-3GPP device information from UDR and take it into account for policy decisions.

5G-RG:

- Ability to include User Identifier and device info in the PDU Session Modification Request.

- Ability to receive and configure QoS rules for different non-3GPP devices.

NOTE: It is assumed that the 5G-RG can identify a device behind it based on an implementation-specific solution or a solution outside of 3GPP scope. Details are left up to BBF and CableLabs.

End of changes