SA WG2 Meeting #162	S2-2405056

April 15 - 19, 2024 Changsha, China	(was S2-240X4250)

Source:	Ericsson, Samsung

Title:	KI 1: Update to Solution 1

Document for:	Discussion/Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes an update to solution to KI #1, solution 1.

# 1. Proposal

It is proposed to accept the following changes to TR 23.700-77.

FIRST CHANGE

## 6.1	Solution #1: Architectural Enhancements in Support of exposure of IMS events using HSS Subscription

### 6.1.1	Description

In this solution, DCSF subscribes to events defined in Table AA.2.4.2.2-1 in Annex AA.2.4.2 of TS 23.228 [5] and other possible IMS events related to a given IMS user of interest via HSS. When the event is fulfilled, the IMS AS notifies the event to DCSF directly.

Additionally, in this solution, it is also proposed that a similar approach is used for trusted and untrusted Application Servers to subscribe to IMS events of interest via the NEF. When the IMS exposure event is related to a given IMS user, the NEF subscribes to the corresponding IMS AS allocated to the IMS user via the HSS. When the event is fulfilled, the IMS AS notifies the IMS exposure event to the NEF.

The list of supported events is defined in Key issue 2.

To support the above, once an IMS subscriber successfully registers in IMS and if at least one IMS AS capable of supporting IMS DC is selected and IMS DC is enabled in the subscriber profile in HSS for this IMS subscriber, then IMS AS registers in HSS the IMS AS instance allocated for the subscriber using an enhanced Nhss_ImsUECM service over N71 for that purpose.

In this release MMTEL AS registers its instance in HSS.

For events specified in Annex AA.2.4.2 of TS 23.228 [5] and other possible IMS events, DCSF subscribes with HSS for such an event using a new service Nhss_ImsEE over N72. If HSS finds that the IMS DC is enabled for the IMPU (s), HSS subscribes with IMS AS using a new service Nimsas_ImsEE_ Subscribe Request over N71.

For any IMS event, an AF subscribes to NEF for an IMS event for a subscriber via a new service Nnef_ImsEE, over N33. The NEF in turn forwards such a subscription request to HSS via a new service Nhss_ImsEE over Nxx.

HSS subscribes to the event with the identified IMS AS instance via a new service Nimsas_ImsEE, over N71. For unregistered UEs, the HSS keeps the subscription request and subscribes to the requested IMS event when the UE registers in IMS.

Upon detection of the requested IMS event, the IMS AS notifies the event directly to the NEF/DCSF via a new service Nimsas_ImsEE, over Nyy for NEF, or DC1 for DCSF.

HSS is aware of events producers as follows:

- IMS AS registers in HSS its instance ID serving the successfully IMS registered UEs. The IMS AS includes in the registration request the list of events (or event categories) the IMS AS instance supports. HSS stores this information together with the IMS AS instance ID serving the successfully registered IMS UE. When HSS receives a subscription request for an event (or event category) pertaining to a registered IMS UE, HSS validates the IMS instance serving the UE for the requested event (or event category) and if supported by the registered IMS AS, HSS sends the subscription request to the IMS AS instance.

Optimization of Subscription request to IMS events

In order to optimize subscription requests to IMS events, it is possible to group a collection of individual events in an event category (e.g. DC events as defined in TS 23.228 Table AA.2.4.2.2-1). This enables an NF consumer to subscribe to the event category, rather than to all individual events in the event category. The NF consumer can receive a notification for any individual event (s) within the event category. An event category is allocated an identity just like any individual event. Additionally, an event category is treated like any event from an event producer point of view in terms of declaration for support as depicted above. HSS and NEF are transparent to an event vs an event category.

The architecture depicting the solution is shown in Figure 6.1.1-1 below.

Figure 6.1.1-1 : IMS Event Framework

Figure 6.1.1-1includes the following reference points:

- N33 is enhanced to enable an AF to subscribe to, and receive notifications about the requested IMS event of interest via the NEF.

- A new reference point between NEF and HSS is introduced to enable NEF to subscribe to HSS for IMS related events for a specific subscriber. This reference point is labelled Nxx.

- N72 is enhanced to allow the DCSF to subscribe to DC Session Control related events via the HSS.

NOTE 1: DCSF can also subscribe to other IMS events of interest.

- N71 is enhanced to enable HSS to receive registration of an AS instance allocated for a specific subscriber upon initial IMS registration and to subscribe to the IMS AS instance of a specific subscriber for any IMS event of interest.

NOTE 2: IMS AS supports subscription to IMS events of interest including events defined in TS 23.228 [5]. The existing Subscribe service operation place holder in TS 23.228 [5] in Nimsas_SessionEventControl is not used in this solution.

- A new reference point between IMS AS and NEF is introduced to enable IMS AS to send notifications regarding directly to the NEF. This reference point is labelled Nyy.

- DC1 is enhanced to allow the DCSF to receive notifications for IMS related events.

Normative phase will define Diameter impacts on legacy interfaces impacted by the solution.

### 6.1.2	Procedures

#### 6.1.2.1 DCSF Subscription to IMS Events Via HSS

To subscribe to IMS events defined in Table AA.2.4.2.2-1 in Annex AA.2.4.2 of TS 23.228 [5] and any other IMS event, DCSF subscribes to HSS for that purpose by sending the Nhss_ImsEE_Subscribe Request and includes the  requested event, as well as the subscriber Identity (e.g IMPU) in the request. When any of the requested events is fulfilled, an Nhss_ImsEE_Notify operation is generated including the relevant information for the event.

Figure 6.1.2.1-1 depicts a typical call flow for DCSF subscribing to IMS events via HSS.

6.1.2.2-1 DCSF Subscription to IMS Events

1) UE performs initial IMS Registration.

2) S-CSCF performs third party Registration with the IMS AS instance allocated to the registering UE.

3)	IMS AS registers in HSS using Nhss_ImsUECM_Registration operation the IMS AS instance allocated to the UE. The IMS AS includes in the registration request the list of events (or event categories) the IMS AS instance supports.

4) 	DCSF subscribes to the event by sending to HSS a Nhss_ImsEE_Subscribe Request. The request includes the subscriber IMPU.

5) HSS in turn locates the IMS AS instance serving the UE. If the registered IMS AS instance supports the requested event, the HSS sends to the IMS AS an Nimsas_ImsEE_ Subscribe Request and includes the subscriber IMPU and the DCSF address as notification endpoint.

6) IMS AS returns to HSS Nimsas_ImsEE_Subscribe Response.

7) HSS returns to the DCSF the Nhss_ImsEE_Subscribe Response to the request initiated in step 4

8) UE initiates an action that leads to a detected event in IMS AS.

9) The requested event is fulfilled, hence IMS AS returns to DCSF an Nimsas_ImsEE_Notify Request including the relevant event information.

10) DCSF returns Nimsas_ImsEE_Notify Response.

#### 6.1.2.2 Subscription to IMS events via NEF

6.1.2.2-1 AF Subscription to IMS events via NEF

Figure 6.1.2.2-1 depicts a typical call flow for an AF subscribing to an IMS event for a UE.

- UE performs initial IMS Registration.

- S-CSCF performs third party Registration with the IMS AS instance allocated to the registering UE.

- IMS AS registers in HSS using Nhss_ImsUECM_Registration operation the IMS AS instance allocated to the UE. The IMS AS includes in the registration request the list of events (or event categories) the IMS AS instance supports.

- AF subscribes to NEF initiating the Nnef_ImsEE_Subscribe Request for a specific IMS event.

- NEF subscribes to HSS using the Nhss_ImsEE_Subscribe Request for a specific IMS event.

- HSS locates the IMS AS instance(s) serving the UE.

- If the registered IMS AS instance supports the requested event, HSS subscribes to the IMS AS instance(s) serving the UE using Nimsas_ImsEE_Subscribe Request for the requested IMS event including the IMPU and the NEF address as notification endpoint.

- IMS AS returns to HSS the Nimsas_ImsEE_Subscribe Response

- HSS returns to NEF the Nhss_ImsEE_Subscribe Response.

- NEF returns to AF the Nnef_ImsEE_Subscribe Response

- At some point, the requested event for the UE is detected by the IMS AS.

- The IMS AS sends an Nimsas_ImsEE_Notify Request to NEF

- NEF sends to AF Nnef_ImsEE_Notify Request.

#### 6.1.2.3 Additional Services and Operations in Support of IMS Event subscription

##### 6.1.2.3.1 Enhancements to hss Services

Nhss_ImsUECM service operations are modified to support registering a UE IMS AS instance, list of supported events/event categories.

Table 6.1.2.3.1-1

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nhss_ImsUE Context Management (Nhss_ImsUECM) | Register | Request/Response | S-CSCF, IMS AS instance |
|  | Deregister | Request/Response | S-CSCF, IMS AS instance |

Services provided by HSS are enhanced to enable NEF and DCSF to subscribe to events or event categories of interest related to a given IMS user. To that effect, a new service is introduced for IMS Event Exposure as depicted below.

Table 6.1.2.3.1.1.2-1

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nhss_imsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | NEF, DCSF |
|  | Unsubscribe | Subscribe/Notify | NEF, DCSF |

##### 6.1.2.3.2 Enhancements to Nimsas Services

Service provided by IMS AS are also enhanced to enable HSS to subscribe to an IMS AS instance for IMS events of interest related to a given IMS user. To that effect, a new service is introduced for IMS Event Exposure as depicted below.

Table 6.1.2.3.2-1

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_ImsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | HSS |
|  | Unsubscribe | Subscribe/Notify | HSS |
|  | Notify | Subscribe/Notify | NEF, DCSF |

##### 6.1.2.3.3 Enhancements to Nnef Services

A new service is introduced for IMS Event Exposure as depicted below:

Table 6.1.2.3.3-1

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_ImsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | NEF |

### 6.1.3	Impacts on existing services, entities and interfaces

HSS impacts:

- Stores IMS AS instance serving a UE at initial IMS registration including the list of supported events/event categories.

- Receive requests of subscription to notification from DCSF and NEF for a UE related event.

- Subscribe to an IMS AS instance for a UE for a related event on behalf of the DCSF or NEF.

IMS AS:

- Registering the IMS AS instance serving a UE in HSS.

- Receive requests of subscription to notification from HSS for a UE related event.

- Send notifications of requested events to the NEF or DCSF.

NEF:

- Receive subscription from AFs for a UE for related events.

- Subscribe to HSS for notification of UE related events.

- Receive notifications of requested events from the IMS AS.

- Send notifications of requested events to the AF.

DSCF:

- Subscribe to notification for all events via HSS.

- Receive notification of all events from IMS AS.