3GPP TSG-SA WG2 Meeting #162	S2-2405106

April 15 – April 19, 2024; Changsha, China	(revision of S2-2403958)

Title:	Reply to LS on provisioning ATSSS rules to the UE in EPC

Response to:	LS on provisioning ATSSS rules to the UE in EPC (S2-2403870/ C1-241721)

Release:	Release 18

Work Item:	ATSSS_Ph3

Source:	SA2

To:	CT1

Cc:	CT4

Contact Person:

Name:	DongYeon Kim, Chunhui Zhu

Tel. Number:

E-mail Address:	dy522.kim@samsung.com, zhu_chunhui@nec.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks CT1 to clarify on the provisioning ATSSS rules to the UE in EPC.

CT1 asks a question with clause 4.22.2.3.1 in TS 23.502 text specified in Rel-17 and clause 4.22.2.4.1 in TS 23.502 text specified in Rel-18, as below:

Why can ATSSS rules be provided via untrusted non-3GPP access to EPC but not via 3GPP access?

SA2 wants to provide answer as below:

In R17, the MA PDU Session which has only 3GPP access via EPC is not supported. Therefore, when the 3GPP access via EPC moves from 5GC first then, the non-3GPP access via 5GC should be kept to use MA PDU Session. Consequently, the ATSSS Rule is provided via 5GC non-3GPP access.

In R18, SA2 has enhanced the ATSSS feature with extended scope, the MA PDU Session which has non-3GPP access via EPC access is supported. Therefore, when the non-3GPP access via EPC moves from 5GC first, the ATSSS Rules can be provided via EPC non-3GPP access.

Based on description above, SA2 has not found any Stage 2 functional issues and there is no need to change any SA2 specifications.

2. Actions:

To 3GPP CT1

ACTION: 	SA2 kindly asks CT1 to take into account of the above information.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #163		27-31 May 2024		Jeju, Korea

TSG-SA2 Meeting #164		19-23 August 2024	Maastricht, Netherlands