3GPP TSG-WG2 Meeting #162	S2-2405112

# 15 April-19 April 2024, Changsha, China

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1394 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correlating the application DC with the bootstrap DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause AC.7.1, each UE may have established a distinct bootstrap data channel with a local DSCF and remote DCSF. In clause AC.7.2.1, when UE#2 receives an SDP offer for application data channel, if the UE#2 has established a bootstrap DC with local and remote DCSF, the UE#2 does not know which bootstrap DC is the right one for the application that is associated with the application DC in the SDP offer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause AC.5, clarify that the binding information that is included in the SDP offer and received by the SDP answerer, need to contain information to correlate the DC application indicated in the SDP offer with the bootstrap DC and the associated bootstrap DC application. The encoding of the binding information will be defined by SA WG4. In clause AC.7.2.1, the UE#2 uses the binding information received in the SDP offer to determine the corresponding bootstrap data channel, and download the application, if not downloaded before. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | P2P application data channel cannot be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.5, AC.7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

# AC.5	Binding of DC Application with the related DC

Information for the binding of DC Application with the related DC is required to support multiple, simultaneous DC applications in a UE.

The binding information is maintained by the HPLMN or retrieved from DC application provider when a DC Application is uploaded to DCSF. DCSF may be configured with a DC application profile associated with the binding information, which indicates the DC control policy (e.g. whether the Application DC establishment follows the P2P, P2A/A2P or P2A2P procedure). The UE receives the binding information of a DC application via the Bootstrap DC, e.g. when the DC application is downloaded to the UE.

When the UE is establishing an IMS Application DC as defined in clause AC.7.2, the binding information is provided by the offerer's UE in the SDP offer as described in TS 26.114 [76]. The IMS AS provides the binding information to the DCSF. The DCSF may use the binding information for controlling the Application DC setup.

When the UE is receiving a request to establish an IMS Application DC as defined in clause AC.7.2, the binding information is provided in the SDP offer to the SDP answerer's UE. The UE can use the binding information to determine the corresponding bootstrap DC and download the application, if not downloaded already.

*************************** End of Changes ****************************