SA WG2 Meeting #162	S2-2405353

April 15 - 19, 2024 Changsha, China	(Revision of S2-2405055, 4761)

Source: 	China Mobile

Title: 	KI #6, Evaluation and interim conclusion for KI#6

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution provides evaluation and interim conclusion for key issue #6.

1	Discussion

This contribution provides evaluation and interim conclusions for KI#6.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-77.

FIRST CHANGE

8	Conclusions

8.X	Interim conclusion of KI#6

The following interim conclusions for KI#6 "Support of Standalone IMS Data Channel Sessions" are agreed:

-	Scenarios including establishing IMS session with standalone bootstrap DC, standalone application DC, or a combination of standalone bootstrap DC and application DC shall be supported in this release;

Editor’s note:	Whether to support session changes to/from standalone DC session is FFS.

-	SIP preconditions may not be used for standalone DC IMS session.

-	The binding information is included in the SDP offer as specified in Rel-18.

Editor’s note:	Whether to support version compatibility is FFS. Coordination with SA4 is needed.

-	When the UE initiates a IMS session with standalone bootstrap DC or standalone application DC:

-	The originating UE generally follows existing procedures to establish bootstrap DC or application DC as specified in 23.228 for standalone DC session establishment with the addition that the UE may only include DC media components when generating SDP offer in initial INVITE request;

-	If the terminating UE has not yet downloaded the application, or if the terminating UE does not have a compatible version of application with the originating UE, the terminating UE rejects the session;

-	When the UE initiates an IMS session with combination of standalone bootstrap DC and standalone application DC

-	The originating UE only include DC media components for both bootstrap DC and application DC when generating SDP offer in initial INVITE request;

-	If the terminating UE has downloaded the application, the terminating UE responds with the SDP answers for both bootstrap DC and application DC;

Editor’s note:	How to handle the scenario that the terminating UE has not yet downloaded the application, e.g. how the SDP is negotiated, when to alert terminating UE, is FFS.

-	The UE uses re-INVITE request to manage all session changes;

-	The UE and IMS network negotiate the capability of supporting standalone DC during registration procedure, where the UE indicates its capability with a media feature tag in initial and subsequent REGISTER request and the S-CSCF indicates the capability with a Feature-Caps header field in the 200OK to initial and subsequent REGISTER request;

Editor Note’s:	Whether the capability negotiation of standalone data channel is really needed is FFS.

-	Additional subscription for standalone data channel is optional. When additional subscription for standalone data channel is required, the subscription information is stored in service data of IMS data channel, based on which the IMS AS determines whether to accept the standalone data channel IMS session.

Editor Note’s:	Whether and how to handle session changes with IMS AS only supporting standalone DC is FFS.

End of CHANGES