SA WG2 Meeting #162	S2-2405356

Changsha, China, April 15 – 19, 2024	(revision of S2-2405057)

Source:	ZTE

Title:	KI #1, Sol #3, Resolve ENs

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: Several editor’s notes exist in Solution 3 regarding discovery of IMS AS serving IMS subscribers and the group of IMS subscribers, this paper is proposed to address them.

1. Discussion

The paper is proposed to resolve Editor’s notes in Solution 3:

For the following group of IMS subscribers related ENs, since the KI#1 has been updated to remove the events for a group of IMS subscribers, it is proposed to update the solution for subscription/notification of a list of IMS subscribers; and the related ENs can be deleted.

Editor's note:  The nature of member information of the group of IMS subscribers in HSS is FFS.

Editor's note:  Whether HSS can provide group information of IMS Subscribers and the interaction of IMS AS and HSS for providing group information and the related service operations are FFS.

2. Proposal

It is proposed to capture the following changes into TR 23.700-77.

* * * * Start of Changes * * * *

6.3	Solution #3: Subscription and Notification for DC Events

6.3.1	Description

The subscription and notification for data channel events are introduced in TS 23.228 [5]. The services and their operations provided by IMS AS are defined as following:

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
|  | Unsubscribe | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |

NOTE:	The Unsubscribe service operation is new added in the table by this solution.

In Release 18, for Nimsas_SessionEventControl service only Notify service operation is specified. While explicit subscription to session events is not addressed in Release 18; and notifying IMS session events of a specific IMS subscriber to NFs, e.g. DCSF is only based on local configuration in the IMS AS. In this solution, the Subscribe and Unsubscribe service operations are introduced and related procedures are described.

NOTE:  This solution specifies Subscribe/Unsubscribe service operations and related procedures; the subscription and notification mechanism for discovery of NF(s) serving the specific IMS subscriber(s) will be addressed by other solutions; the service operations can be updated accordingly if needed.

6.3.1.1	Nimsas_SessionEventControl_Subscribe service operation

Service operation name: Nimsas_SessionEventControl_Subscribe

Description: This service operation enables NF consumers (e.g. DCSF) subscribe session events related to a specific served IMS subscriber or a list of IMS subscribers requesting use of data channel media.

Inputs, Required: NF type, Event type

In this release, NT type is DCSF.

Event type is DC Event.

Inputs, Optional: IMS Subscriber, Identity list of IMS subscribers

IMS Subscriber is the subscriber requesting use of data channel media and its session event is subscribed.

Identity list of IMS subscribers is the identity list of IMS subscribers and the IMS Subscriber in the list is requesting use of data channel media.

These two inputs are optional and only one of them will be given in a service operation.

Outputs, Required: Result indication, when the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

6.3.1.2	Nimsas_SessionEventControl_Notify service operation

Nimsas_SessionEventControl_Notify service operation is specified in clause AA.2.4.2.2 of TS 23.228 [5].

6.3.1.3	Nimsas_SessionEventControl_Unsubscribe service operation

Service operation name: Nimsas_SessionEventControl_Unsubscribe

Description: This service operation enables NF consumers (e.g. DCSF) unsubscribe session events related to a specific served IMS subscriber or a list of IMS subscribers requesting use of data channel media.

Inputs, Required: Subscription Correlation ID

Inputs, Optional: None

Outputs, Required: Result indication

Outputs, Optional: None.

6.3.2	Procedures

6.3.2.1	Subscription procedure for a specific IMS Subscriber

Figure 6.3.2.1-1 depicts a flow diagram for subscription procedure for a specific IMS Subscriber.

Figure 6.3.2.1-1 Subscription procedure for a specific IMS Subscriber

The steps in the subscription flow are as follows:

DCSF subscribes session events related to a specific served IMS subscriber requesting use of data channel media using service operation Nimsas_SessionEventControl_Subscribe as specified in 6.3.1.1.

IMS AS will notify DCSF the session event of the subscribed IMS subscriber requesting use of data channel media using service operation Nimsas_SessionEventControl_Notify as specified in clause AA.2.4.2.2 of TS 23.228 [5].

6.3.2.2	Subscription procedure for a list of IMS Subscribers

Figure 6.3.2.2-1 depicts a flow diagram for subscription procedure for a list of IMS Subscribers.

Figure 6.3.2.2-1 Subscription procedure for a list of IMS Subscriber

The steps in the subscription flow are as follows:

DCSF subscribes session events related to a list of IMS subscribers whose member in the list requests use of data channel media using service operation Nimsas_SessionEventControl_Subscribe as specified in 6.3.1.1.

IMS AS will notify DCSF the session event of one of members in the subscribed list of IMS subscribers requesting use of data channel media using service operation Nimsas_SessionEventControl_Notify as specified in clause AA.2.4.2.2 of TS 23.228 [5].

Editor's note:  How to handle unregistered IMS Subscribers is FFS.

6.3.3	Impacts on Existing Nodes and Functionality

IMS AS:

-	Supporting service operation of subscribing session events related to a specific served IMS subscriber or a list of IMS subscribers requesting use of data channel media.

DCSF:

-	Supporting the use of subscription service operation for session events related to a specific served IMS subscriber or a list of IMS subscribers requesting use of data channel media.

* * * * End of Changes * * * *