SA WG2 Meeting #162	S2-2405695

# Changsha China, April 15th–April 19th, 2024        	(revision of S2-2405433)

Source: 	Nokia, NEC

Title: 	Solution for KI#2: Handling and notifying UE for Terminating SMS.

Document for: 	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: solution addressing KI#2

# 1	Discussion

This document describes a solution addressing KI#2. If the User Identity (profile) is using the UE, the terminating SMS attempting to deliver a message to the UE Subscription will fail because UE is not available at that moment for the UE Subscription but is used by the User Identity. While the UE is used by a User Identifier that is linked to the UE Subscription, there could be terminating messages which are attempted by the network and not delivered as the UE is being used by a User Identifier, hence it is required to at least alert with some indication, for the UE to take an action. This is an optional capability that is proposed.

Below is the Scenario:

- UE subscription is owned by the father. (UE)

- User Profile is daughter. (User identifier)

For example, between time T0 to T100, the UE of the father is used by daughter, which means the UDM is aware that UE Subscription of father + User Profile of daughter is active, not just UE Subscription of father.

Now at T20, if (for e.g.) Bank sends an SMS to the father (not the daughter), the UDM knows the UE is used by the UE Subscription of father + User Profile of daughter, an indication alert (without exposing what it is about) to the father, that some terminating SMS has been attempted by network.

User (daughter) is using the UE (of father), but the prime person or the subscription owner is the father, is expected to be alerted. This solution currently considers the need to alert only the UE Subscription scenario only.

Based on the indication received, the UE could take appropriate action.

# 2 Proposal

It is proposed to update TR 23.700-32 as follows.

# * * * * First change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> |
| #1 | X | X | X |  |
| #2 | X |  |  |  |
| #3 | X | X |  |  |
| #4 | X |  |  |  |
| #5 | X | X |  |  |
| #6 | X |  |  |  |
| #7 | X |  |  |  |
| #8 |  | X |  |  |
| #9 |  | X |  |  |
| #10 |  | X | X |  |
| #11 |  | X | X |  |
| #12 |  | X | X |  |
| #13 |  | X |  |  |
| #14 |  | X |  |  |
| #15 |  |  | X |  |
| #16 |  |  | X |  |
| #X: |  | x |  |  |

# * * * * Next change (All new text) * * * *

## 6.X	Solution #X: Handling and notifying the UE when the UE is associated with a User Identifier.

### 6.X.1	Introduction

This solution addresses KI#2 related to WT. Below details more on notification building on top of Solution 6 and Solution 10 which mentions of UECM de-registration of the UE (father) for the SMS service when the User identity (child) is registered to the network.

Note: If the UECM de-registration should be done for the UE (father) when the User Identity (child) is registered to the network can be based on operator policy or subscription data, which is FFS.

### 6.X.2	Functional Description

If the User Identity is using the UE, the terminating services like SMS attempting to deliver a message to the UE Subscription will fail because UE is not available at that moment for the SMS, as UECM is deregistered for the SMS. However, UE is still used by user identity. So, it is required that.

- The terminating SMS as in above are not delivered to the UE, as the UE is being used by the User identity (For e.g., a daughter (User Identity) is using the UE (of the father)).

- It is also required to indicate to the UE Subscription (of the father as in the above example) with an indication that SMS service were attempted to be delivered.

Brief of the solution:

A prior, the UE could indicate support of receiving indication from the network when the UE is being used by User Identifier and there is an attempt from the network for terminating SMS to the UE.

A NAS based indication can be added in the existing NAS message (configuration update procedure) or a new NAS message. The NAS indication is delivered to the UE over NAS layer.

When the UE that is being used by the User Identifier receives this indication, based on the value, it delivers the indication to the right application layer (SMS app).

Example: Application layer can display an indication to the user or add blink on the screen there is some delivery attempted by the network. The user may take some action (e.g., initiate log out so that UE can get logged in again to receive the message). However, this aspect is out of the scope of the 3GPP. Please note, this indication does not reveal any real message.

### 6.X.3	Procedures

#### 6.X.3.1	Illustration for SMS

Figure 6.X.3-1: Procedure for send indication to UE when the terminating service is SMS.

Step1: User Identifier 1 is registered with the UE and the UE may have indicated that is supports receiving an indication.

Step 2: UE gets UECM deregistered for the SMS Service.

Step 3: When SendRoutingInfoForSM reaches to the UDM for terminating SMSF address determination for the UE, and UECM is deactivated state for the SMS, the UDM returns error to the GMSC.

Step 4: UDM understands that there is a terminating SMS initiated for the UE. As UDM knows that UE is de-registered now and User Identity Profile 1 is registered/logged in, UDM sends an optional indication towards UE via AMF (steps 5, 6).

Step 7: Application layer may display an indication to user or add blink on the screen indicating that an action is attempted by the network.

NOTE 1: Any privacy considerations if any will be studied at SA3.

NOTE 2: This indication is for the terminating SMS service over NAS.

### 6.X.4	Impacts on services, entities, and interfaces

UE:

- UE supports indicating UE capability of supporting receiving indications from the network for the UE.

- UE implements receiving of the indication.

UDM:

- Implements identification of terminating SMS to a UE and the state of the UE being in de-registered state.

- Carries out indication to the UE based on the local policies and the UE Subscription to receive such indication.

AMF:

- Receipt of indication from the UDM.

- Transferring of the indication to the UE over NAS.

# * * * * End of Changes * * * *