3GPP TSG-SA WG2#162	S2-2405782

Changsha, China, 15- 19 April 2024	(was S2-2405266)

Source:	Qualcomm Incorporated

Title:	KI 1.3: New Solution for DualSteer PDU Session Establishment

Document for:	Discussion/Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: Discusses a possible solution for session establishment for DualSteer (KI#1.3).

# 1.	Discussion

This pseudo-CR proposes a solution for DualSteer KI 1.3 (session management).

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-54:

# >>>>BEGINNING OF CHANGES<<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #1.1> | <Key Issue #1.2> | <Key Issue #1.3> | <Key Issue #1.4> |
| Z |  |  | X |  |

# >>>>NEXT CHANGE<<<<

## 6.1	Solutions for DualSteer

### 6.1.Z	Solution #Z: Session Establishment

#### 6.1.Z.1	Description

This solution addresses the Key Issue on Session Management for Dual Steer (Key Issue #1.3). In particular, it describes the DualSteer PDU session establishment triggered by a DualSteer Device/UE(s). The principles of the solution are:

-	First the DualSteer device needs to register to the network and the DualSteer feature is activated.

-	The DualSteer device then establishes a PDU Session over one access network for SUPI 1 and then it establishes a PDU Session over another access network for SUPI 2. During the session establishment, the network provides the DualSteer UE Rules and the DualSteer UPF Rules to the UE and the UPF, respectively, to handle the related traffic.

The solution assumes that on the network side the association of UE using SUPI 1 and UE using SUPI 2 (corresponding to the DualSteer Device) via a the DualSteer Correlation ID is done in the UDM and known at the H-PCF (e.g. by configuration) for the creation of UE policies for DualSteer feature, including URSP rules. This solution does not have impact on AMF and V-SMF, therefore, in case of roaming, there are no impacts on the V-PLMN. The UE context of SUPI1 and SUPI2 in UDM use the DualSteer Correlation ID in the SMF selection data in the UDM to ensure the selection of the correct SMF for the second PDU session.

Editor’s Note: whether the DualSteer Correlation ID can be the PDU Session ID is FFS.

Editor’s Note: further details for configuration of DualSteer Correlation ID in H-PCF are FFS.

The solution assumes the use of specific DNNs/S-NSSAIs for DualSteer, without assuming that such DNNs/S-NSSAIs are restricted to DualSteer or that the DualSteer Device can have only one session to a DNN/S-NSSAI for DualSteer.

This solution also assumes that 	the DualSteer device contains a DualSteer Control Function (DSCF) that controls and coordinates DualSteer feature (including registration, session management, and UE policies) across the two 3GPP accesses. The DualSteer Control Function (DSCF) takes into account the registration status in both UEs and the UE policies for DualSteer, including URSP rules, for session management procedures subject to DualSteer. For session management, the DSCF receives indication that DS PDU Session is allowed for SUPI 1 and accepted for SUPI 2, receives UE policies (including URSP rules, via SUPI 1 and, optionally, via SUPI 2), and triggers the PDU Session establishment for SUPI 2.

A DualSteer Device can establish a DualSteer Session with the PSA UPF in order to exchange data with an application server (AS). The DS Session is a logical construct with two access legs, both of 3GPP access type, which are PDU sessions. The 3GPP access type can be the same, e.g., NR and NR, or different, e.g., NR terrestrial and NR non-terrestrial or NR and LTE. Figure 6.1.Z.1-1 gives a high-level view of a DualSteer Session and of the relevant network functions (the UPFs of the two PDU sessions are optional and may be a VPLMN UPF).

Figure 6.1.Z.1-1: DualSteer (DS) Session between DualSteer Device and (PSA) UPF

Editor’s Note: Description of the steering of new services and the switching of existing services is FFS

#### 6.1.Z.1.1	Proposed URSP enhancements for DualSteer

Since the DualSteer policies apply to both 3GPP access networks, it is proposed that these UE policies for DualSteer, including URSP rules, are handled by the DualSteer Control Function in the DualSteer Device.

The URSP rules are proposed to be enhanced as follows to support URSP rules for DualSteer:

-	The Route Selection Descriptors (RSDs) for each SUPI are extended to include a new preferred access type value: DualSteer.

-	It is proposed to include an optional RAT validity. The RAT validity, if included, is used to select the possible RAT combinations for the DualSteer communication. RAT validity values include:

-	NR_NR: this RSD applies to NR + NR combination, i.e. both SUPI 1 and SUPI 2 are in NR.

-	NR_LTE: this RSD applies to NR + LTE combination, e.g. SUPI 1 is in NR and SUPI 2 is in LTE

-	NR: this RSD applies when the UE has only one access, and such access is over NR.

-	LTE: this RSD applies when the UE has only one access, and such access is over LTE.

-	(no RAT validity): the RSD applies to any RAT.

##### 6.1.Z.1.2	DualSteer steering and switching rules (DualSteer UE/UPF Rules)

It is proposed to define DualSteer UE rules, in similar fashion as the ATSSS rules (but without the inclusion of a Load balancing steering mode). When DualSteer applies (i.e., when the preferred access type value is DualSteer, and DualSteer has been enabled), the Dual Steer UE rules apply for traffic steering/switching.

An example of of the structure for DualSteer UE rule is in Table 6.1.Z.1.2-1:

Table 6.1.Z.1.2-1: Structure of DualSteer UE Rule

| Information name | Description | Category | SMF permitted to modify in a PDU context | Scope |
| --- | --- | --- | --- | --- |
| Rule identifier | Unique identifier to identify the DualSteer Rule | Mandatory | No | PDU context |
| Rule Precedence | Determines the order in which the DualSteer rule is evaluated in the UE. | Mandatory (NOTE 1) | Yes | PDU context |
| Traffic Descriptor | This part defines the Traffic descriptor components for the DualSteer rule. | Mandatory (NOTE 2) |  |  |
| Application descriptors | One or more application identities that identify the application(s) generating the traffic (NOTE 3). | Optional | Yes | PDU context |
| IP descriptors (NOTE 4) | One or more 5-tuples that identify the destination of IP traffic. | Optional | Yes | PDU context |
| Non-IP descriptors (NOTE 4) | One or more descriptors that identify the destination of non-IP traffic, i.e. of Ethernet traffic. | Optional | Yes | PDU context |
| Access Selection Descriptor | This part defines the Access Selection Descriptor components for the DualSteer rule. | Mandatory |  |  |
| Steering Mode | Identifies the steering mode that should be applied for the matching traffic and associated parameters. | Mandatory | Yes | PDU context |
| Threshold Values | A Maximum RTT and/or a Maximum Packet Loss Rate. | Optional (NOTE 6) | Yes | PDU context |
| Steering Functionality | Identifies which DualSteer functionality should be applied for the matching traffic (NOTE 8) | Optional (NOTE 5) | Yes | PDU context |
| Transport Mode | Identifies the transport mode (see clause 5.32.6.2.2.1) that should be used for the matching traffic, when the Steering Functionality is the MPQUIC functionality. | Optional (NOTE 7) | Yes | PDU context |
| NOTE 1: Each DualSteer UE rule has a different precedence value from the other DualSteer UE rules. NOTE 2: At least one of the Traffic Descriptor components is present. NOTE 3: An application identity consists of an OSId and an OSAppId. NOTE 4: A DualSteer UE rule cannot contain both IP descriptors and Non-IP descriptors. NOTE 5: If the UE supports only one Steering Functionality, this component is omitted. NOTE 6: The Steering Mode Indicator and the Threshold Values shall not be provided together. NOTE 7: The Transport Mode shall be included when the Steering Functionality is the MPQUIC functionality. In all other cases, the Transport Mode shall not be included. This value will be needed if MPQUIC is defined for DualSteer. NOTE 8: Potential Steering functionality options are the MPTCP functionality, the MPQUIC functionality, or the DualSteer-LL functionality. Which are defined for DualSteer is to be determined. |  |  |  |  |

Each DualSteer UE rule contains an Access Selection Descriptor that contains the following components:

-	A Steering Mode, which determines how the traffic of the matching SDF should be distributed across 3GPP access for SUPI 1 and 3GPP access for SUPI 2. The following Steering Modes are supported:

-	Active-Standby: It is used to steer a SDF on one 3GPP access (the Active access), when this access is available, and to switch the SDF to the available other 3GPP access (the Standby access), when Active access becomes unavailable. When the Active access becomes available again, the SDF is switched back to this access. If the Standby access is not defined, then the SDF is only allowed on the Active access and cannot be transferred on another access.

-	Smallest Delay: It is used to steer a SDF to the access that is determined to have the smallest Round-Trip Time (RTT). Measurements may be obtained by the UE and UPF to determine the RTT over 3GPP access and over the other 3GPP access. In addition, if one access becomes unavailable, all SDF traffic is switched to the other available access. It can only be used for the Non-GBR SDF.

Editor’s Note: how the Smallest Delay mode can be used in conjunction with non-simultaneous transmission is FFS

-	Threshold Values: One or more threshold values may be provided when the Steering Mode is Priority-based. A threshold value may be either a value for RTT or a value for Packet Loss Rate. The threshold values are applicable to both 3GPP accesses and are applied by the UE and UPF as follows:

Editor’s Note: how to measure RTT or packet loss rate in conjunction with non-simultaneous transmission is FFS

-	A Steering Functionality: this is used when the UE supports multiple functionalities for DualSteer (see NOTE 8).

Editor’s Note: Whether Steering functionality is necessary, and which values it can contain is FFS

-	A Transport Mode, which identifies the transport mode that should be applied by the MPQUIC functionality for the matching traffic. (see NOTE 7)

Editor’s Note: whether MPQUIC is required for DualSteer is FFS

#### 6.1.Z.2	Procedures

The solution has the following preconditions:

Precondition 1: Before establishing the DS Session, a DualSteer Device needs to register for both SUPIs to the 5G CN. If the DualSteer device is registered for only one SUPI, the device behaves based on Rel. 18 procedures.

NOTE 1: when the DS device registers with SUPI1, the network may decide to provide URSP rules that also cover SUPI2 (based on the correlation between SUPI1 and SUPI2 known in UDM). If the DS device does not register with SUPI2, the the DS device acts as a normal UE based on Rel. 18 procedure.

NOTE 2; Regular PDU sessions not involved in DS when the registration and connectivity for SUPI2 is established are not impacted.

Precondition 2: There is an association of SUPI 1 and SUPI 2 in the UDM, and the H-PCF is aware of it.

Precondition 3: One or more DNN/S-NSSAI combinations are dedicated to DualSteer.

It is also assumed, to prevent AMF impacts, that the DS Correlation ID exchanged between SFM, UDM and AMF is stored in the form of an SMF ID.

The DS PDU Session Establishment procedures are based on the UE triggered PDU Session establishment procedure defined in TS 23.502 [4] clause 4.3.2.2.1 with the differences described below Figure 6.1.z.2-1 and Figure 6.1.z.2-2, which depict the session establishment for SUPI 1 and SUPI 2, respectively.

Figure 6.1.Z.2-1: DS PDU Session establishment for SUPI 1

PDU Session establishment for DualSteer device for SUPI 1:

0.	The DualSteer Control Function (DSCF) triggers the request of establishment of the PDU Session for SUPI 1.

1.	the UE sends a legacy (Rel-18) PDU Session Establishment Request. The UE may indicate its DS capabilities supported by the DualSteer Device (i.e., whether it supports only steering or both steering and switching, which UP options it supports, etc).

2-3.	The AMF selects the SMF as per Rel-18 (no AMF impacts).

NOTE 3: The selected SMF is DualSteer capable because it is assumed that the DualSteer Device connects to a DNN/S-NSSAI combination that supports DualSteer.

4-7a.	As Steps 4-7a of TS 23.502 [x] clause 4.3.2.2.1. When a serving SMF instance is selected for SUPI 1 and is stored in UDM, the DualSteer Correlation ID of that SMF instance (which coincides with the SMF ID of that SMF instance) is also stored in the UE context of SUPI 2 in the UDM. Consequently, the DualSteer Correlation ID is not provided to the DualSteer Device.

7b.		The PCF, based on the DS PDU Session Request, may provide specific rules to the SMF.

8.		The SMF selects a DS capable UPF.

9.		As Step 9 of TS 23.502 [x] clause 4.3.2.2.1.

10a.	The SMF provides the UPF with the DualSteer UPF Rules, used to handle traffic steering and switching once the DS PDU Session is established.

10b.	As Step 10b of TS 23.502 [x] clause 4.3.2.2.1.

11-13.	Together with the PDU Session Establishment Accept message, the SMF indicates to the DualSteer Device:

-	the DS UE Rules to instruct the DualSteer Device on how to handle traffic steering and switching once the DS PDU Session is established. The DS UE Rules include details on which steering/switching modes and UP options are supported

13b.	The SUPI 1 provides to the DualSteer Control Function (DSCF) the indication that the PDU Session is allowed and the DS UE Rules.

14-21.	As Step 14-21 of TS 23.502 [4] clause 4.3.2.2.1.

Figure 6.1.Z.2-1: DS PDU Session establishment for SUPI 2

PDU Session establishment for DualSteer device for SUPI 2:

0.	The DualSteer Control Function (DSCF) triggers SUPI 2 to request the establishment of the PDU Session.

1.	The UE/DualSteer Device sends a legacy PDU Session Establishment request for SUPI.

2-3.	the AMF selects the SMF as per Rel-18 (no AMF impacts).

NOTE 4:	The AMF retrieves, as per Rel-18, the SMF ID (i.e.the DS Correlation ID) of the SMF instance previously selected for SUPI 1.

4-7b.	As Steps 4-7a of TS 23.502 [x] clause 4.3.2.2.1.

8.	The SMF selects the same DS capable UPF instance selected for the SUPI 1 based on the SUPI 2 subscription information.

NOTE 5:	Based on precondition 2 above, the SMF uses the SUPI 1 to find the correct UE context in the UDM and, in turn, to find the UPF instance used for SUPI 1.

9-10b.	As Step 9 of TS 23.502 [x] clause 4.3.2.2.1, without the need to provision the DS UPF rules to the UPF.

11-13.	Together with the PDU Session Establishment Accept message, the SMF indicates to the UE that the PDU Session is accepted. The SMF does not need to provide the DS UE rules.

13b.	The indication that the PDU Session Establishment is accepted is provided to the DSCF.

14-21.	As Step 14-21 of TS 23.502 [x] clause 4.3.2.2.1.

NOTE 6: The uplink data can only take place for a given service if that service is switched to the connectivity corresponding to SUPI 2.

#### 6.1.Z.3	Impacts on services, entities and interfaces

DualSteer Device (DSCF):

-	Receives indication that PDU Session is allowed for SUPI 1 and accepted for SUPI 2.

-	Receives DS UE Rules.

-	Triggers the PDU Session establishment for SUPI 2.

AMF: no impacts

SMF

-	indicates to UE whether PDU Session for DS is allowed or not.

-	selects DS capable UPF for SUPI 1.

-	selects for SUPI 2 the same UPF instance previously selected for SUPI 1.

-	stores the DualSteer Correlation ID in UE context of SUPI 2 in the UDM during PDU Session establishment for SUPI 1;

UDM

-	SUPI 1 UE context contains the SUPI 2 value

-	SUPI 2 UE context includes SUPI 1 value.

-	DualSteer correlation ID is stored in SUP1 and SUPI 2 UE contexts

PCF

- is aware of SUPI 1 and SUPI 2 association.

# >>>>END OF CHANGES<<<<