SA WG2 Meeting #S2-162	S2-2405796

15 - 19 April, 2024, Changsha, China	(revision of S2-2405525/S2-2404379)

Source:	ZTE

Title:	KI#2, evaluation and conclusion

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This paper proposes the evaluation and conclusion to Key issue #2

1. Proposal

It is proposed to accept the change to 23.700-45.

>>>> Start of Change <<<<

# 7	Overall Evaluation

Editor's note:	This clause provides evaluations of different solutions.

## 7.X	Key Issue #2: Enabling provisioning of subscribers allowed to access CAG cell and managing access control by the CAG owner or an authorized administrator

Solution #1, #2, #3 and #4 are proposed to address Key Issue #2: Enabling provisioning of subscribers allowed to access CAG cell and managing access control by the CAG owner or an authorized administrator.

A comparison of solutions on above aspect is shown in Table 7.X-1.

Table 7.X-1: Comparison of solutions

|  | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Which network the CAG information is provisioned | Serving network | Serving network | Serving network | Home network |
| Which network Function the CAG information is stored | CAS, stores the Serving PLMN-specific CAG subscription data | V-UDR, stores visited CAG Information and visited allowed CAG list indication per subscriber. | CMF, store the CAG information based on the provisioned data. | UDM |
| UE impact | N/A | UE provide capability indication to AMF whether it supports visited CAG information | UE indicates it is a visitor UE in the Registration message. | N/A |
| AMF impact (Note 1) | According to the flag in subscription data, AMF retrieves the CAG list from 5G CAS. | According to the allowed indication in subscription data and UE indication, the AMF retrieves the CAG info from V-UDR. | According to UE indication, the AMF retrieves the CAG info from CMF. |  |
| UDM subscription data | a flag in subscription data, indication Serving PLMN-specific subscription data needs to be retrieved | visited CAG allow indication and MSISDN in UE subscription data |  |  |
| NEF impact | V-NEF, enhance the PP procedure to provision the CAG to CAS | V-NEF receives the AF request, and store CAG information to V-UDR | Same with sol#1. | H-NEF, enhance the PP procedure to provision the CAG to UDM |
| Note 1: After obtaining the CAG list, the AMF perform the access control according to 23.502 [3] |  |  |  |  |

>>>> Next Change <<<<

## 8.X	Key Issue #2: Enabling provisioning of subscribers allowed to access CAG cell and managing access control by the CAG owner or an authorized administrator

For interim conclusions, the following aspects are taken as way forward:

-	The NEF API, i.e. Parameter Provisioning specified in the 23.502 [3] clause 4.15.6 is enhanced to support the 5G Femto CAG provisioning.

Editor's note:	Whether this enhanced procedure is applied to Serving network or Home network is FFS.

>>>> End of Change <<<<