3GPP SA WG2 Meeting #167	S2-2501572

Athens, Greece, 17-21 February 2025

# Title:	[Draft] Reply LS on emergency service and eDRX

# Response to:	LS emergency service and eDRX (R2-2411041/S2-250abcd)

# Release:	Release 19

# Work Item:	NR_newRAT-Core, NR_redcap-Core

Source:	SA2

To:	RAN2, CT1, RAN5

Cc:	RAN3

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR 6083 (TS 23.501), TS 23.401 CR 3884 (TS 23.401), TS 23.503 CR 1508(TS 23.503)

1. Overall Description:

SA2 thanks RAN2 for the LS (R2-2411041) on emergency services and eDRX.

SA2 discussed the scenarios related to the incoming LS and came to the conclusion that a) the UE is not supposed to be in CM-IDLE state when an emergency PDU session is active and b) PSAP callback is delivered as “normal call” i.e. within the IMS PDU session and not the emergency PDU session. To clarify these scenarios, SA2 approved CRs against TS 23.501 and TS 23.502 (for 5GS) as well as TS 23.401 (for EPS).

Specifically when the UE is in CM-CONNECTED/RRC_INACTIVE state SA2 discussed the scenarios RAN2 named in the incoming LS and would like to provide the following information:

1) TS 23.501 has specified the following that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to 5G-AN:

#### 5.16.4.3        Mobility Restrictions and Access Restrictions for Emergency Services

When the (R)AN resources for Emergency Services are established, the ARP value for Emergency Services indicates the usage for Emergency Services to the 5G-AN.

#### 5.16.4.7        PCC for Emergency Services

Dynamic PCC is used for UEs establishing emergency service and shall be used to manage IMS emergency sessions when an operator allows IMS emergency sessions. When establishing Emergency Services with a SMF, the PCF provides the SMF with the QoS parameters, including an ARP value reserved for the Emergency Services to prioritize the QoS Flows when performing admission control, as defined in TS 23.503 [45].

#### 5.16.4.9        Handling of PDU Sessions for Emergency Services

The ARP reserved for emergency service shall only be assigned to QoS Flows associated with an emergency PDU Session.

2) TS 23.502 has also specified the following:

#### 6.1.3.10        IMS emergency session support

The PCC Rule Authorization function selects QoS parameters that allow prioritization of IMS Emergency sessions. If an IMS Emergency session is prioritized the QoS parameters in the PCC Rule shall contain an ARP value that is reserved for intra-operator use of IMS Emergency services. The PCF does not perform subscription check; instead it utilizes the locally configured operator policies to make authorization and policy decisions.

SA2 therefore considers that RAN is enabled to take into consideration that CN provides RAN with both ARP and proper eDRX values for the UE context.

2. Actions:

To RAN2:

ACTION: 	Take the above information into account.

To CT1, RAN5:

ACTION: 	Take into account the attached SA2 approved CRs and update your specifications accordingly.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #168		7-11 April 2025	Goteborg, Sweden

TSG-SA2 Meeting #169		19-23 May 2025	Fukuoka, Japan