SA WG2 Meeting #167	S2-2501630

# 17 - 21 February, 2025, Athens, Greece

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1385 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sample ID intersection when untrusted AF acting as VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung [Nokia] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following exception has been agreed upon in SP-241513/S2-2413037: Vertical Federated Learning (VFL) For the case where AF is untrusted whether additional functionality in NEF is needed during the VFL preparation When untrusted AF is acting as the VFL server, as it has been documented in clause 6.2H.2.2.2 of TS 23.288, the NEF will forward the VFL preparation response from every VFL client individually to the server by mapping a list of internal sample IDs to external IDs. Exposing the supported sample IDs of every single NWDAF VFL clients can potentially increase the risk of 5GC privacy leakage. CR S2-2500432 was submitted to SA2#166AHE with proposing to enhance NEF to perform sample ID intersection. But the CR was noted due to concerns on NEF complexity. In this CR, we propose to use NWDAF VFL client to perform sample ID intersection, which will no add additional complexity of NEF but can still protect 5GC privacy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove EN on NEF work during VFL preparation in clause 6.2H.2.2.2. Introduce new steps into clause 6.2H.2.2.2 to support NWDAF VFL client to determine the sample ID intersection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC privacy cannot be secured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

##### 6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL server

This clause specifies the preparation (including sample alignment) procedure for untrusted AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL Server

0.	If UEs are to be used as samples, an untrusted AF may use the Nnef_MemberSelection service to ask the NEF to reduce the list of candidate UEs based on filter criteria it provides.

1-	The untrusted AF as VFL server sends VFL preparation requests to the candidate VFL client(s), using Nnef_VFLTrainingRequest_Request to check if the VFL client(s) can meet the ML Model training requirement (which includes the Analytics ID, list of sample IDs, and optionally Feature ID, etc.). The external NWDAF IDs obtained in the discovery procedure (see clause 6.2H.2.1.1) is included to indicate the target NWDAFs. The suggested VFL Interoperability Information to negotiate the intermediate results that will be used in training.

1a-1c.	The untrusted AF as VFL server sends VFL preparation requests to each candidate VFL client(s) via NEF separately.

1d.	Alternatively, the untrusted AF as VFL server sends one VFL preparation request to NEF by including the candidate VFL client NWDAF IDs. The candidate VFL client NWDAF IDs include the Ids of the VFL client aggregator and other VFL clients.

NOTE : subject to operators’ policies, the untrusted AF maybe required to contact the other candidate NWDAF VFL client(s) via one NWDAF VFL client to protect 5GC privacy. The VFL client aggregator might be pre-configured or selected by VFL server based on its stored information.

2-	The NEF maps the external NWDAF and GPSI(s) to the internal NWDAF and SUPI(s).

2a-2c.	The NEF sends VFL preparation request to the corresponding candidate internal NWDAF ID using Nnwdaf_VFLTraining_Request service with the same information as provided in step 1 in clause 6.2H.2.2.1.

2d-2e.	 Alternatively, the NEF sends a VFL preparation request to one VFL client. Then this VFL client sends multiple VFL preparation requests to the other corresponding candidate NWDAF VFL client(s).

3-	Same as step 2 in clause 6.2H.2.2-1.

4-	The candidate VFL clients invokes Nnwdaf_VFLTraining_Response to indicate whether it accepts the ML Model training requirements, the VFL Client can also indicate that it cannot join the FL process.

4a-4c. Same as step 3 in clause 6.2H.2.2-1, if the VFL server sends preparation request to each candidate VFL client separately in step 1.

4d-4f.	Alternatively, the candidate VFL clients in step 2e sends response to the VFL client aggregator. The VFL client aggregator aggregates the responses from multiple candidate NWDAF VFL clients and determines the sample ID intersection among the other candidate NWDAF VFL clients and itself. Then the VFL client aggregator sends the intersected sample IDs and the other information in step 3 of clause 6.2H.2.2-1 to the VFL server via NEF.

5-	The NEF maps the internal NWDAF and SUPI(s) to the external NWDAF and GPSI(s). The NEF sends the Nnef_VFLTraining_Request Response to the VFL Server with the same information as provided in step 4.

6-	Same as step 4 in clause 6.2H.2.2-1.

* * *End of Changes * * *