3GPP TSG-WG SA2 Meeting #167	S2-2501841

# Athens, GR, 17th Feb – 21st Feb, 2025		(revision of S2-2501201)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0292 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF triggered EAS rediscovery via HPLMN for HR-SBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For AF triggered EAS rediscovery, the SMF converts APP ID to the impact field, which is based on old EAS information (e.g. EDI, IP address of the offloaded traffic). In HR-SBO case, UE accesses to VPLMN EAS, so the APP ID indicated by AF needs old EAS information in VPLMN to determine the corresponding impact field. However, the H-SMF may not have the old EAS information in VPLMN (e.g. VPLMN EDI). Hence, the logic for generating impact field cannot be reused and need more clarifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description about how H-SMF converts APP ID to impact field when AF indicates EAS rediscovery via HPLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS rediscovery via HPLMN cannot work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5270 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | NOTE 3: Only one of "afAppId", "trafficFilters", "ethTrafficFilters" or “trafficDataSets” shall be present. As defined in clause 5.4.3.3.2 of TS 29.522, if AF provides APP ID via the TrafficInfluence API for EAS rediscovery (which is different mechanism than EAS IP replacement), it shall not provide traffic filters, and the H-SMF cannot derive the impact field. Hence, the interaction between H-SMF and V-SMF should be involved to enable EAS rediscovery when AF provides APP ID. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.7.3.2	Network triggered EAS change in HR-SBO context

Figure 6.7.3.2-1 shows the procedure of EAS re-discovery and edge relocation when HR-SBO is supported and allowed in the target serving PLMN (i.e. V-SMF change or insertion).

NOTE 1:	For the V-SMF removal case, the AF triggers EAS relocation refer to clause 6.2.3.3.

Figure 6.7.3.2-1: Network triggered EAS rediscovery and edge relocation in HR-SBO context procedure

0.	The procedures described in clauses 6.7.2.6, 6.7.2.7 and 6.7.2.9 are performed: inter V-SMF inter-PLMN N2 handover or mobility registration (0a) in the HR-SBO case; or inter V-SMF intra-PLMN N2 handover or Xn handover or mobility registration update (0b) in the HR-SBO case; or V-SMF insertion in the HR-SBO case.

1.	In the case of the procedures happened in step 0a, if the AF had subscribed to the corresponding event and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF, indicating the new serving PLMN ID as well as HPLMN DNN and S-NSSAI for HR-SBO session and H-SMF may include capability of supporting EAS IP replacement in 5GC with PLMN ID of VPLMN. This may take place as soon as the H-SMF has received an indication of Handover Complete (see step 13 of Figure 6.7.2.6-1).

NOTE 2:	Via this mechanism, the AF is aware of the PLMN to contact to issue traffic influence requests for HR-SBO sessions, if available, with HPLMN DNN and S-NSSAI information. The AF is assumed to check whether it has an SLA with the new serving PLMN. If the AF has no SLA with the new serving VPLMN, the AF interacts with H-NEF to issue traffic influence requests.

This may trigger the AF triggered edge relocation / EAS rediscovery as defined in step 1b of Figure 6.2.3.3-1 and in step 4a of Figure 6.3.3.1.1-1.

2a.	For AF triggered EAS re-discovery and edge relocation via interacting with HPLMN, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the H-PCF via the H-NEF for the H-AF untrusted, or directly to the H-PCF for the AF trusted, then transfer to the H-SMF and optional H-NEF, H-UDR using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3]. This may trigger step 2 of Figure 6.2.3.3-1, where the H-SMF can derive the impacted field. If the H-SMF is not able to derive the impact field, the H-SMF sends the received Application Identifier(s) to V-SMF. Based on old EAS information in VPLMN (e.g. the EAS Deployment Information of VPLMN, traffic offloaded in VPLMN) and Application Identifier(s), the V-SMF derives the impacted field and sends the related impact field information to H-SMF for generating PDU Session Modification Command.

The AF may also provide EAS IP replacement information and target DNAI together with an indication of the PLMN associated with this target DNAI, i.e. the serving PLMN ID in the case that the VPLMN (V-SMF) supports EAS IP replacement capability and has already notified this capability information to AF (directly or via H-SMF/H-NEF).

NOTE 3:	V-SMF providing the capability of supporting EAS IP replacement to AF can happen either:

-	during V-SMF insertion/change phase where the capability information is provided to H-SMF and then to AF via H-NEF as noted in step 1; or

-	as part of PDU session update respond procedure that is initiated by H-SMF in step 3a.

2b.	For AF triggered EAS re-discovery and edge relocation via interacting with serving VPLMN, the AF may indicate the EAS rediscovery for the impacted applications via the V-NEF using the procedure described in clause 4.3.6 of TS 23.502 [3].

This may trigger step 2 of Figure 6.2.3.3-1 where the SMF that initiates PDU Session modification is the V-SMF that initiates Nsmf_PDUSession_Update request with the requested PCO.

The AF may also provide EAS IP replacement information and target DNAI to the VPLMN (i.e. V-SMF). In this case, steps 3-4 are skipped. If the V-SMF cannot serve the target DNAI, it triggers V-SMF change/removal, and the EAS IP replacement information and target DNAI may be provided in SM context as described in clause 6.7.2.6.

3a.	(For AF triggered EAS re-discovery and edge relocation via interacting with HPLMN case): The AF traffic influence request information is sent to H-SMF via PCC rule. This may trigger step 2 of Figure 6.2.3.3-1 where the SMF that initiates the PDU Session modification is the H-SMF. The H-SMF issues a Nsmf_PDUSession_Update request. The Nsmf_PDUSession_Update request includes policies due to AF provided traffic influence information, which may also contain EAS IP replacement information and target DNAI provided by AF in step 2. If the V-SMF cannot serve the target DNAI, it invokes a Nsmf_PDUSession_SMContextStatusNotify service operation to send the target DNAI to AMF, and the AMF selects a target V-SMF based on the target DNAI as described in clause 4.23.5.4 of TS 23.502 [3] by replacing I-SMF with V-SMF. The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response, containing Authorization Result for HR-SBO, EAS IP replacement information and target DNAI in the Request. The target V-SMF may select a new V-EASDF as described from steps 2 to 12 in Figure 6.7.2.6-1.

3b.	(For AF triggered EAS re-discovery and edge relocation via interacting with VPLMN case): The V-SMF initiates Nsmf_PDUSession_Update request with the EAS rediscovery indication and the impact field to the H-SMF, and the H-SMF initiates Nsmf_PDUSession_Update Response towards the (target) V-SMF including the PCO information to be sent to the UE as described in step 2 of Figure 6.2.3.3-1. In intra-PLMN V-SMF change, the target V-SMF may use the source and target DNAI to determine the Impact field to be sent to the UE. In inter-PLMN mobility, the target V-SMF provides EAS rediscovery information without an Impact field.

4.	The V-SMF may initiate PDU Session Modification command including the PCO to the UE.

The PCO may include EAS rediscovery indication (optional) and the impact field (optional).

5	The V-SMF may configure the V-UPF (UL CL and L-PSA) with EAS IP replacement information.

# * * * * End of changes * * * *