SA WG2 Meeting #167	S2-2501953

17 - 21 February, 2025, Athens, Greece                  	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1595 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Third Party ID Signing and Verification Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to clarify steps in third party ID information signing and verification procedure. Descriptions of several steps in third party ID information signing and verification procedure are not clear; and some of them do not align with the steps in the figure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify several steps in third party ID information signing and verification procedure and align the step description with the figure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The step description for third party ID information signing and verification procedure is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AF.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AF.3	Procedure for signing and verification of third party user identity information in IMS

Figure AF.3-1 depicts the procedure for signing and verification of third party user identity information in IMS.

Figure AF.3-1: Procedure for signing and verification of third party user identity information in IMS

1-2.	If the third party network is providing third party user identity information in the outgoing initial SIP INVITE request and is authorized to use third party user identity information, and based on operator policies, S-CSCF, IMS AS or IBCF invoke signing of the provided third party user identity information with a signing AS.

3.	If using third party user identity information is successfully authorized, the originating IMS AS retrieves RCD server address, RCD URL, or RCD information from the HSS, based on the originating IMPU or wildcarded IMPU.

4.	The originating IMS AS receives RCD server address, RCD URL, or RCD information from the HSS.

5.	If the IMS AS has retrieved an RCD URL in step 4, the IMS AS, based on configuration, may retrieve the RCD information from the RCD server using the RCD URL. The IMS AS includes either the RCD URL or, if retrieved from the RCD server, the RCD information in the outgoing SIP INVITE request.

If the IMS AS has retrieved an RCD server address in step 4, the IMS AS provides the originating IMPU or wildcarded IMPU to the RCD server, to retrieve the RCD information. The IMS AS includes the retrieved RCD information in the outgoing SIP INVITE request.

If the IMS AS has retrieved an RCD information in step 4, this step is skipped.

NOTE 1:	How RCD URL or RCD information are carried in SIP signalling is defined in TS 24.229 [10a].

6-7.	The originating IMS network invokes signing of the RCD information or RCD URL included in the outgoing SIP INVITE request with a signing AS.

NOTE 2:	Based on operator policies, either the S-CSCF, IMS AS or IBCF in the originating network invoke signing of the RCD information or RCD URL.

8.	The originating IMS network sends the SIP INVITE request which includes the signed RCD information or RCD URL.

9-10.	Based on operator policies, the S-CSCF, IMS AS or IBCF in the terminating IMS network invoke signature verification of the RCD information or RCD URL included in the incoming SIP INVITE request with a verification AS. The S-CSCF, IMS AS or IBCF in the terminating IMS network may reject the call when the signature verification is unsuccessful.

11.	The terminating IMS network sends the SIP INVITE request which includes the signed RCD information or RCD URL to the terminating UE.

12.	The terminating UE may present third party user identity information of the calling-party to the called party retrieved from RCD information, if present, in the SIP INVITE request. The called party may use the RCD URL, if present, to retrieve RCD information from the RCD server to download third party user identity information.

END OF CHANGES