3GPP TSG-SA2 Meeting #167	S2-2502067

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025                               (Revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1518 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Traffic detection and QoS handling of multiplexed data flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Procedure for AF to provide the QoS requirements for multplexed media flows need to be added to the specification (KI#4). KI#4 describes differentiated QoS handling for multiplexed media flows. KI#2 describes E2E encrypted XRM traffic. Therefore also differentiated QoS handling for multiplexed, encrypted (other than (S)RTP) media flows need to be covered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe differentiated QoS handling for multiplexed, e2e encrypted media flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Differentiated QoS handling for multiplexed, e2e encrypted media flows is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note to editor: please add the text in this CR for clause 6.1.3.27.x under the same clause and after the text in CR1388. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

##### 6.1.3.27.x	Traffic identification and differentiated QoS for multiplexed media flows

When the AF request is for differentiated QoS handling for multiplexed encrypted media flows (see clause 5.37.11.y of TS 23.501 [2]):

- 	The AF includes the Multiplexed Media Identification Information for encrypted traffic in the Flow Description  Information (see TS 23.501 [2] clause 5.7.6.2) and specific QoS requirements per media flow as described in clause 6.1.3.22.

-	The PCF determines the PCC Rules and QoS requirements for the media flows with Multiplexed Media Identification Information for encrypted traffic as follows:

-	When the AF request includes downlink direction, the PCF generates different PCC rules for the downlink direction for the corresponding media flows. Each PCC Rule includes the QoS requirements that apply to the corresponding media flow identified by Multiplexed Media Identification Information for encrypted traffic in the IP Packet Filter set.

-	When the AF request includes uplink direction, the PCF may generate PCC rules for the uplink direction based on operator configuration. The PCF may generate PCC rules for the uplink direction for the end-to-end transport layer connection and may notify the AF that the differentiated QoS handling for multiplexed media flows cannot be fulfilled. As an alternative, PCF may reject the AF request indicating lack of support for Multiplexed Media Identification Information for encrypted traffic.

NOTE Z:	It is assumed the PSA UPF supports the IP Packet Filter with Multiplexed Media Identification Information for encrypted traffic.

# * * * * End of changes* * * *