SA WG2 Meeting #168	S2-2502995

07 - 11 April, 2025, Goteborg

Source:	SK Telecom, China Mobile, China Telecom, Deutsche Telekom, KPN N.V., LG Uplus, Verizon

Title:	WT#2 Key Issue: Analytics of detection and control of UPF traffic bursts to support efficient performance of User Plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes a new key case to cover Work Task#2 of the FS_AIML_CN_Ph2 SID.

1	Discussion

This paper discusses a Key Issue for FS_AIML_CN_Ph2 (SP-250413) Work Task #2 (WT#2), as described below.

Whether and how to enhance the 5GC analytics to support efficient performance user plane. Note 4: This may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework. Note 5: The study will not address how UPF detects wrong or malformed or duplicated packets. This WT shall have neither RAN or UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF.

A related Key Issue for the WT#2 is provided in accordance with the SA2 guidance (SP-250418).

2	Proposal

It is proposed to include the following changes in TR 23.700-04 V.0.0.

* * * * Start of Changes – All New Text * * * *

5.Y	Key Issue #x: Analytics of detection and control of UPF traffic bursts to support efficient performance of User Plane

5.Y.1	Description

Depending on the time of day, configurations, UE applications, and UPF deployments, certain data burst packets from UEs/Sessions/Applications may cause unexpected UPF overloads and irregular behaviour in network functions, which can result in overall network performance degradation. NWDAF for example can provide a monitoring and analysis to solve UPF related performance issues, thereby e.g., reconfiguring UPF parameters related to queues, buffering, jitter, and latency, to ensure optimal performance. In this context, it is proposed to study:

- Whether and how to identify UE/Sessions/Applications that are causing (un)expected traffic bursts and how to mitigate it efficiently.

- Whether and how to report UPF events or data efficiently to NWDAF.

- How to reconfigure UPF, e.g., using Operator configurable UPF capability, PFCP association effectively.

* * * * End of Changes * * * *