3GPP TSG-WG SA2 Meeting #168	S2-2503154

Goteborg, Sweden, 07 – 11April 2025	  (was S2-250xxxx)

Source:	vivo

Title:	Update to inventory procedure

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: It proposes the update to AIoT Inventory procedure to fix ENs.

- Discussion

It is proposed the following update to the AIoT Inventory procedure:

1. AIOTF selection will be reflected in clause 5.3.X2, then “Editor's note:	How the NEF selects the AIOTF needs further definition.” can be removed and add a reference to 5.3.X2.

2. Add the inventory processing description among AIoT RAN, AMF and AIOTF for indirect path.

3. Term alignment for parameters with NF services.

4. Since AIOTF should report the progress of AF service operation request to AF, AF should know whether the AF service operation request is finished or not. Hence, it’s proposed that the AIoT RAN indicates whether it is the last report from the AIoT RAN for the AIoT Service Operation and AIOTF indicates whether it is the last report from AIOTF for the AIoT Service Operation.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change (all new texts)***

6.2	AIoT Service Procedures

6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command.

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires additional details, alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [Target area information], [information of the target AIoT Device ], [Approximate number of AIoT Devices]) service operation request to the NEF.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF authorizes the AF Inventory request as in clause 5.6 and then selects the AIOTF as in clause 5.3 to handle the request. The AIOTF Selection is as defined in clause 5.3.X2.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target area information], [AIoT Device information], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request as in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by AIoT RAN.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message (Correlation ID, [Target area information], [information of the target AIoT Device ], [Approximate number of AIoT Devices])) to the AIOT RAN. The correlation identifier is the identifier of the AIoT service operation (e.g. inventory or command) and is used by the RAN to determine the destination of AIoT service operation responses.

In case of the direct path:

-	7a-1. the AIOTF sends the Inventory Request message to AIOT RAN directly.

-	7a-2. The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID.

In case of the indirect path:

-	7b-1. the AIOTF invokes the AMF Namf_AIoT_MessageDelivery (Correlation ID, Inventory Request message, AIOT RAN ID) service operation.

-	7b-2. The AMF sends the Inventory Request message to the AIOT RAN according to the indicated AIOT RAN ID.

-	7b-3. the AIoT RAN sends an Inventory response to AMF including whether accepts the Inventory Request with the Correlation ID.

-	7b-4. The AMF sends the Namf_AIoT_Notify (Inventory response, Correlation ID) to the AIOTF.

Editor's note:	AIoT Device Identification information needs further definition.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

9.	AIoT RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s). The AIOTF indicates whether the report is the last report for the AIoT inventory.

In case of the indirect path, the AIoT RAN sends the Inventory Report messages to AMF, then the AMF sends a Namf_AIoT_Notify (Inventory Report, correlation ID) to the AIOTF.

10.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

11.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s). The AIOTF indicates whether the report is the last report for the AIoT inventory request.

12.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s). The AIOTF indicates whether the report is the last report for the AIoT inventory request.

# * * * * End of changes * * * *