SA WG2 Meeting #168	S2-2503217

# Goteborg, 7th – 11th April, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1423 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE LCS privacy check for ground truth data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#167 meeting, S2-2502497 and S2-2502257 were postponed due to lack of consensus on UE LCS privacy check for ground truth data collection (S2-2502257) and ground truth data storage at ADRF(S2-2502497). Based on the discussion paper S2-250abcd, we propose to extend the NF that query to check the LCS privacy profile in UDM to include LMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the NF that query to check the LCS privacy profile in UDM to include LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for Procedure for MTLF-based AI/ML model performance monitoring for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

### 6.2E.4	Procedure for MTLF-based AI/ML model performance monitoring for LMF-based AI/ML Positioning

NWDAF containing MTLF may support to perform the performance monitoring for ML model used by LMF for LMF-based AI/ML positioning.

The LMF may request the NWDAF to monitor the ML model performance by including the ML Model identifier of the ML model, for which the performance needs to be monitored, and an ML Model Accuracy Threshold, which is used as an indication to execute the accuracy monitoring operations, in the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request service operation as described in clause 6.2A.

When the NWDAF containing MTLF monitors the ML Model performance, he NWDAF requests the LMF to provide the following as input data using the procedure of input data collection as specified in clause 6.22.4 of TS 232.273 [39]:

-	Optionally, Location measurement data from the PRU(s)/UE(s) and/or the NG-RAN.

-	Ground truth data from PRU(s)/UE(s).

-	Optionally, the corresponding inference output, i.e. location estimation of the PRU(s)/UE(s) using LMF-based AI/ML positioning.

Before collecting the input data from UE(s), LMF needs to check the user consent in UDM during the data collection procedure. The LMF also performs UE LCS privacy check before sending the input data which is the ground truth data collected to the NWDAF.

The NWDAF containing MTLF evaluates the ML Model performance by comparing the ground truth data against the corresponding location estimation of the PRU(s)/UE(s), which is derived by the NWDAF containing MTLF using its local ML model and the location measurement data from the PRU(s)/UE(s) and/or the NG-RAN, or derived by the LMF using the ML model and location measurement data from the PRU(s)/UE(s) and/or the NG-RAN.

If the ML Model Accuracy threshold requested by the LMF is not satisfied, the NWDAF containing MTLF retrains the ML model, or the NWDAF containing MTLF sends to LMF an ML Model degradation indicator as specified in clause 6.2A.2, which may trigger the LMF to change the positioning method, e.g. from LMF-based AI/ML Positioning to legacy positioning.

Figure 6.2E.4-1 illustrates the procedure by which the NWDAF containing MTLF monitors the ML model performance for LMF-based AI/ML positioning.

Figure 6.2E.4-1: Monitoring performance of ML Model used for LMF-based AI/ML positioning

1.	The LMF may request an ML Model for LMF-based AI/ML positioning by invoking the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request service operation. The LMF may include an ML Model identifier and an ML Model Accuracy Threshold for ML model performance monitoring.

2.	The NWDAF containing MTLF trains the ML Model and, based on the ML Model Monitoring information (i.e. ML Model identifier, ML Model Accuracy Threshold) or its local policy, decides to start monitoring the accuracy of the ML model.

3.	The NWDAF containing MTLF requests input data from the LMF for ML model monitoring as per clause 6.22.4 of TS 23.273 [39].

4.	The NWDAF containing MTLF determines, by comparing the ground truth data against the corresponding location estimation of the PRU(s)/UE(s), whether the ML model is degraded or not.

5.	If the ML model performance cannot meet the requirement (e.g. lower than the ML Model Accuracy threshold), the NWDAF containing MTLF may retrain the ML model.

6.	The NWDAF containing MTLF provides a new or retrained ML model to the LMF, or notifies the LMF that the ML model is degraded by invoking the Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelInfo_Request response service operation.

* * * End of Changes* * *