3GPP TSG-WG SA2 Meeting #168	S2-2503290

07 - 11 April, 2025, Goteborg, Sweden	(revision of)

Source:	vivo(rapporteur), Samsung(rapporteur)

Title:	New Key Issue for transfer of data over UP for UE data collection

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposed to include a new key issue for transfer of data over UP for UE data collection.

1. Introduction/Discussion

Agreed R20 FS_AIML_CN_Ph2 SID (SP-250413) contains the following.

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training. Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389. Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase. Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

The following table are the Analysis of different data collection options for UE-side model training from RP-242389, which introduced the requirements for option 2 including that the first termination entity should inside the CN, the AI/ML-specific Data Transfer Path should be UE-> CN -> Server, etc.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

In this study, it is proposed to include a new key issue for transfer of data over UP for UE data collection.

2. Text Proposal

It is proposed to agree the following key issue(s) into TR 23.700-04.

# * * * * First change * * * *(All new text)

X.Y	Key Issue #X – Transfer of data over UP for UE data collection

X.Y.1	Description

This key issue aims to provide solutions for UE data collection to meet the requirements for AI/ML for NR air interface operation with UE-side model training, which will investigate the following aspects:

- Whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

NOTE 1:	This study is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

NOTE 2:	The requirements elaborated in LSs RP-242389 and R2-2411152 will be considered for investigating and evaluating potential solutions documented for this key issue.

NOTE 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

# * * * * End of changes * * * *