3GPP SA WG2 Meeting #168	S2-2503296

Gothenburg, Sweden, 7-11 April 2025

# Title:	[Draft] Reply LS on multiplexing capability negotiation

# Response to:	LS on multiplexing capability negotiation (C1-250885/ S2-2502807)

# Release:	Release 19

# Work Item:	NG_RTC_Ph2

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Kefeng Zhang

Tel. Number:

E-mail Address:	kefezhan AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-250XXXX

1. Overall Description:

SA2 thanks CT1 for their liaison on data channel multiplexing capability negotiation. The attached CR has been agreed in SA2 to streamline the capability negotiation within the home IMS network.

In general, the S-CSCF determines whether data channel multiplexing capability can be supported for the UE in the home IMS network based on the aggregated capabilities of related NFs (e.g., P-CSCF, IMS-AGW, MF). The method by which the S-CSCF becomes aware of the data channel multiplexing capabilities of other NFs (e.g., P-CSCF, IMS-AGW, MF) is dependent on network deployment.

On the requirement of the P-CSCF:

Q1: During the capability negotiation between UE and IMS network, what is the consideration to require the P-CSCF to indicate its capability to the home IMS network of the UE?

Answer 1: The P-CSCF and the IMS-AGW need to understand the multiplexed SDP to avoid gating the multiplexed media streams

If the P-CSCF and IMS-AGW do not support data channel multiplexing and the MF in the home IMS network of the UE supports data channel multiplexing:

Q2A: what the S-CSCF will indicate in the Feature-Caps header field included in 200 OK response to REGISTER request?

Answer 2A: In this case, the S-CSCF does not indicate supporting data channel multiplexing in the Feature-Caps header field included in 200 OK.

Q2B: what the P-CSCF will do if the Feature-Caps header field included in 200 OK response to REGISTER request indicates support of data channel multiplexing and the P-CSCF and IMS-AGW do not support data channel multiplexing?

Answer 2B: N/A: this situation should not occur because the S-CSCF take P-CSCF capability into account when determines supporting data channel multiplexing in the Feature-Caps header field.

On the requirement of the S-CSCF:

Q3: What is the consideration to require the S-CSCF to indicate its capability to the remote IMS network?

Answer 3: N/A: this requirement has been removed.

Q4: If the P-CSCF and IMS-AGW do not support data channel multiplexing and the MF in the home IMS network of the UE supports data channel multiplexing what the S-CSCF will indicate in the Feature-Caps header field of the INVITE request to the remote network?

Answer 4: In this case, the S-CSCF shall not indicate supporting data channel multiplexing in the Feature-Caps header field included in 200 OK.

Q5: Can the multiplexed data channel media descriptions within an SDP offer included in the SIP request messages implicitly point out this capability of the IMS network?

Answer 5: Yes, if multiplexed data channel media descriptions included in the SIP request messages, the remote IMS can implicitly know this capability of the originating IMS network. But if multiplexed data channel media descriptions are not available (e.g., single DC) in the SDP Offer, the remote IMS network can’t judge whether DC multiplexing is supported or not in the originating IMS network.

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above provided answers into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #169		19-23 May 2025	Fukuoka, Japan

TSG-SA2 Meeting #170		25-29 August 2025	Goteborg, Sweden