3GPP TSG-WG SA2 Meeting #168	S2-2503471

Goteborg, SE, 7 – 11 April, 2025	(revision of S2-250xxxx)

Source:	Sony

Title:	Update Inventory Procedure

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Updates the Inventory procedure with clarifications and corrections

# 1. Introduction/Discussion

This pCR propose a few updates as following:

- In step 1 and step 3: The AIoT Device ID identification information is optional IE. We believe that this must be a mandatory IE, as either this IE indicate a specific device or multiple devices. Even if this is a “address all” inventory, the IE would have a defined value.

Proposal: Remove the [] brackets in both steps.

- Step 4: Remove the cause code, as this would be a stage 3 thing to decide how the reject is formatted.

- Step 6: Clarify that this is the last step performed if the service operation response is a Reject.

- In step 7: As the text is written now, it could be interpretated that the Correlation ID is included in the paging message, this is not correct.

Proposal: Reformat the text into a message format and delete the text of what is included in the paging message.

- The EN in step 7 is copied and placed under step 1 but the new EN is reworded to apply to the ID IE in Step 1. The existing EN in step 7 is reworded.

Editor's note:	AIoT Device Identification information needs further definition.

4) 	The “AIoT Device Identification information” has a too similar name compared to “AIoT Device ID identification information” used in step 1. As these IE might not be the same it is proposed that “Device” is removed in step 7, and the new IE name would be “AIoT Identification information”.

5) 	The EN in step 9 shall state Device ID protection and not “concealed or encrypted” as these are two kind of solutions that are captured in the TR, and actually no longer considered solutions as the initial conclusion states that SA3 aim to protect the device ID using Temporary ID. We have two options:

a)	Remove the EN and change the AIoT device ID to Temporary ID, or

b)	Change the EN to “protected”

We believe option (b) is the best option right now as SA3 has more details to be concluded before SA2 can align fully.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, AIoT Device ID identification information, [Target area information], [Approximate number of AIoT Devices]) service operation request to the NEF.

Editor's note:	The parameters for the inventory service operation need further definition.

Editor's note:	AIoT Device ID identification information needs further definition especially when addressing multiple AIoT Devices.

2.	The NEF selects the AIOTF to handle the request.

Editor's note:	How the NEF selects the AIOTF needs further definition.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, AIoT Device ID identification information, [Target area information],  [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF. If no AIOTF can be selected, the NEF rejects the AIoT Command request and step 6 is performed before ending the procedure.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request, and steps 5 and 6 are performed before ending the procedure.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by AIoT RAN.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3. If the response was a reject the procedure stops here.

7.	The AIOTF sends the Inventory Request ( correlation ID, AIoT Identification information, [assistance information] to the selected AIoT RAN. The AIoT Identification information can be an individual AIoT Device ID or identification information addressing a multiple of AIoT devices.

Editor's note:	Whether and how the AIoT Device ID is protected will be determined and aligned with SA WG3.

Editor's note:	The format of the AIoT Identification information when addressing a multiple of AIoT devices is FFS.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the AIoT Device ID is protected will be determined and aligned with SA WG3.

10.	AIoT RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Inventory_Notify message including a list of AIoT Device Permanent ID(s).

Editor's note:	Whether only one, or more than one report is generated by the AIOTF to send to the AF needs clarification.

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Inventory_Notify message including the AIoT Device Permanent ID(s).

# * * * * End of changes * * * *