SA WG2 Meeting #168	S2-2503688

07 - 11 April, 2025, Goteborg, Sweden	(revision of S2-25xxxxx)

Source:	ZTE

Title:	pCR to TS 23.369: Address EN for AIoT Reader Control Transportation

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper clarifies that the NGAP protocol is used to transport the AIoT Reader Control messages and removes related ENs.

1. Discussion

In TS 23.368, there is an EN regarding AIoT Reader Control message transportation as following:

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

RAN3 has reached the following working assumption regarding how to transport AIOTF information.

Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

Based on RAN3 WA, it is proposed to remove the EN and add the description that the AIoT Reader Control messages will be included in the NGAP protocol.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

4.6.2.2	Protocol Stack between AF and AIoT Device for AIoT RAN Direct Path

Legend:

-	AIoT Reader Control: It is between the AIOTF and AIoT RAN.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Path option

The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the AIoT RAN transparently.

The AIoT Reader Control messages between the NG RAN and the AIOTF are included in the NGAP protocol.

* * * * Next Change * * * *

4.6.2.3	Protocol Stack between AF and AIoT Device for AIoT RAN Indirect Path

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Path via AMF option

In this Protocol Stack, the AMF is additionally involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the AIoT RAN and AMF transparently.

The AIoT Reader Control messages between the NG RAN and the AMF are included in the NGAP protocol.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

* * * * End of Changes * * * *